DeletSA3#95-LI	 S3i240643

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages – Part XI (single MMS Relay/Server case with redirection - redirected MMS User Agent retrieves)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs when only one MMS Relay Server is used and redirection happens. In the illustrations the redirected-to MMS User Agent retrieves the MM message – three illustrations, A is the target, B is the target, or C is the target.

Since only one MMS Relay/Server is used, all users are presumed to be MMS User Agents identified with a phone number.

For the scenario where the MMS User Agent even though served by the same MMS Relay/Server but addressed using an MSISDN-based Email Address is not included here since in the process of MM messages going over the MM3 interface, it is treated as if in a different MMS Relay/Server.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

In the illustrations shown in this pCRs, originating, redirecting, and redirected MMS User Agents are served by the same MMS Relay/Server. This pCR covers the case when the redirected-to MMS User Agent retrieves MM message.

The originating MMS User Agent or redirected-to MMS User Addressed using an MSISDN based Email Address are not illustrated here because from an MMS LI perspective, messages are transferred over MM3 interface.

The cases are the same as in multiple MMS Relay/Server scenario illustrated before.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

#### 4.4.3.3	Originating MMS domain

#### 4.4.3.4	Terminating MMS domain

#### 4.4.3.5	Redirecting MMS domain

#### 4.4.3.6	Single MMS Relay/Server - non-redirection case

***** New text ****

#### 4.4.3.7	Single MMS Relay/Server: redirection case

##### 4.4.3.7.1	General

###### 4.4.3.7.1.1	Scope

In the illustrations, originating, redirecting and redirected-to MMS User Agents are served by the same MMS Relay/Server. Seven cases are illustrated in the subsequent clauses:

- Retrieval case, originating MMS User Agent is the target (immediate retrieval).

- Retrieval case, redirecting MMS User Agent is the target (immediate retrieval).

- Retrieval case, redirected-to MMS User Agent is the target (immediate retrieval).

- Retrieval case, redirected-to MMS User Agent is the target (deferred retrieval).

- Deletion case, originating MMS User Agent is the target.

- Deletion retrieval case, redirecting MMS User Agent is the target.

- Deletion retrieval case, redirected-to MMS User Agent is the target.

The retrieval, deletion refers to the actions taken at the final destination, i.e. the redirected-to MMS User Agent. There is no impact due to retrieval method for MMS LI with originating MMS User Agent as a target or with the redirecting MMS User Agent as a target.

The case of Email Client or MSISDN based Email Address as the target Ids or target non-local ID do not come into play in these illustrations, since all users are local to the MMS Relay Server.

###### 4.4.3.7.1.2	MMBox specific steps

In the illustrations, MMBox interactions are shown as and when the MM message pass through. Each MMS User Agent has own inboxes in the MMBox. In the illustrations, separate MMBoxes are shown for each MMS User Agent to infer that individuality.

If the MMBox is not supported or not setup by any of the MMS User Agents, then MMBox related steps shown for that MMS User Agent in the subsequent clauses are not applicable. In that case, the MMS Relay/Server stores the MM message locally and a reference then would be to the internally stored MM message.

Specific descriptions that apply to a particular illustration noted in the respective clause.

When the MMBox is used, each MMS User Agent may also perform a few MMBox specific operations as illustrated in clause 4.4.3.2.2.

Originating MMS User Agent

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox with the status showing "sent", if an MMBox is supported for the originating MMS User Agent. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message, even if the MMBox is supported for the originating MMS User Agent.

Redirecting MMS User Agent

When the MMBox is supported, the incoming MM message is automatically stored in the MMBox, if setup and configured so, by the redirecting MMS User Agent. The status of thus stored MM message would be "New". In that case, redirecting MMS User Agent would point to the MM message stored in the MMBox in the forwarding request. Once forwarded, the status of the MM message in the MMBox is changed to "forwarded".

If the MMBox is supported, but not configured to store the incoming MM message automatically, then the MMS User Agent may request the MMS Relay/Server to store the MM message in the MMBox, when a forwarding request is sent. The status of the thus stored MM message would be "forwarded".

Redirected-to MMS User Agent

When the MMBox is supported, the incoming MM message is automatically stored in the MMBox , if setup and configured so, by the redirected-to MMS User Agent. The status of thus stored MM message would be "New". In that case, the redirected-to MMS User Agent would point to the MM message stored in the MMBox for any subsequent actions (e.g. retrieval, deletion, forwarding). The status of the MM message changes according to the action taken (e.g. "Retrieved", "Forwarded").

If the MMBox is supported, but not configured to store the incoming MM message automatically, then the redirected-to MMS User Agent may request the MMS Relay/Server to store the MM message in the MMBox using the m-mbox-store-req message as illustrated in clause 4.4.3.2.2.

###### 4.4.3.7.1.3	Delivery reports

Delivery report to the originating MMS User Agent

The sending of delivery report to the originating MMS User Agent is conditional. The delivery report is sent only if the originating MMS User Agent has requested such a report and the redirecting MMS User Agent permits the sending of such a report.

Delivery report from the redirected-to MMS domain

The sending of delivery report to the redirecting MMS User Agent is conditional. The delivery report is sent only if the redirecting MMS User Agent has requested such a report and the redirected-to MMS User Agent permits the sending of such a report.

###### 4.4.3.7.1.4	Read reports

A read report is not sent to the originating MMS User Agent.

A read report may be sent by the redirected-to MMS User Agent and delivered to the redirecting MMS User Agent, if the redirecting MMS User Agent requests for such a report, and the redirected-to MMS User Agent sends such a report.

##### 4.4.3.7.2	Overview

Figure 4.4.3.7.2-1 gives an overview where originating, redirecting and redirected-to MMS User Agents are served by the same MMS Relay/Server. In the illustration, all three MMS User Agents are the targets.

Figure 4.4.3.7.2-1: Single MMS Relay/Server serving originating and terminating targets

The MM1 messages that flow between the MMS Relay/Server and the target MMS User Agent are used to generate the corresponding xIRIs.

Use of the MMBox is optional on a per MMS User Agent level. In other words, either one, or two, or three, or none of the three MMS User Agents may have the MMBox. When used, the MMBox specific operations (which either of the three MMS User Agents may perform in a mutually exclusive manner) along with the associated LI aspects are illustrated in 4.4.3.2.2.

##### 4.4.3.7.3	MM message retrieval by the redirected-to MMS User Agent

###### 4.4.3.7.3.1	General

In the illustrations single MMS Relay/Server serves all the three MMS User Agents. Four scenarios are illustrated.

-	MMS User Agent (A) is the target. MMS User Agent (C) retrieves the MM message automatically.

-	MMS User Agent (B) is the target. MMS User Agent (C) retrieves the MM message automatically.

-	MMS User Agent (C) is the target. MMS User Agent (C) retrieves the MM message automatically.

- 	MMS User Agent (C) is the target. MMS User Agent (C) retrieves the MM message after a time-gap (deferred retrieval).

The retrieval, automatic or deferred has no impact on the MMS LI when either originating MMS User Agent (A) or the redirecting MMS User Agent (B) is a target.

###### 4.4.3.7.3.2	Originating MMS User Agent (A) is the target

In the figure 4.4.3.7.3.2-1, the originating MMS User Agent (A) is the target and sends an MM message to MMS User Agent (B) who in turn forwards the same to the MM User Agent (C). The MMS User Agent (C) retrieves the MM message automatically.

Figure 4.4.3.7.3.2-1: Single MMS Relay/Server – MMS User Agent (A) is the target

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

Even though the trigger for the generation of xIRI MMSSend is the m-send-conf message that the MMS-Relay/Server sends to the MMS User Agent (A), the xIRI includes information from the m-send-req as well.

Since the sending of the m-delivery-ind (forwarded) is conditional, the generation of xIRIs MMSDeliveryReport is also conditional.

###### 4.4.3.7.3.3	Redirecting MMS User Agent (B) is the target

In the figure 4.4.3.7.3.3-1, the originating MMS User Agent (A) sends an MM message to MMS User Agent (B) who in turn forwards the same to the MM User Agent (C). In this illustration, the MMS User Agent (B) is the target. The MMS User Agent (C) retrieves the MM message automatically.

Figure 4.4.3.7.3.3-1: Single MMS Relay/Server – MMS User Agent (B is the target

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

Even though the trigger for the generation of xIRI MMSForward is the m-forward-conf message that the MMS-Relay/Server sends to the MMS User Agent (B), the xIRI includes information from retrieved MM message as well

Since the sending of the m-delivery-ind and m-read-orig-ind to the MMS User Agent (B) are conditional, the generation of xIRIs MMSDeliveryReport and MMSReadReport are also conditional.

###### 4.4.3.7.3.4	Redirected-to MMS User Agent (C) is the target (immediate retrieval)

In the figure 4.4.3.7.3.4-1 the originating MMS User Agent (A) sends an MM message to MMS User Agent (B) who in turn forwards the same to the MM User Agent (C). In this illustration, the MMS User Agent (C) is the target. The MMS User Agent (C) retrieves the MM message automatically.

Figure 4.4.3.7.3.4-1: Single MMS Relay/Server – MMS User Agent (C) is the target (immediate)

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

The MMSDeliveryAck is sent when the whe MMS Relay/Server sends the notifyresp-ind to the target MMS User Agent (C).

The trigger for the xIRIMMSReadReport is MMS Relay Server receiving the m-read-rec-ind from the target MMS User Agent (C). However, MMS User Agent (C) sending of the m-read-rec-ind is conditional, and therefore, the generation of xIRI MMSReadReport is conditional.

###### 4.4.3.7.3.5	Redirected-to MMS User Agent (C) is the target (deferred retrieval)

In the figure 4.4.3.7.3.5-1 the originating MMS User Agent (A) sends an MM message to MMS User Agent (B) who in turn forwards the same to the MM User Agent (C). In this illustration, the MMS User Agent (C) is the target. The MMS User Agent (C) retrieves the MM message after a time-gap (deferred retrieval).

Figure 4.4.3.7.3.5-1: Single MMS Relay/Server – MMS User Agent (C) is the target (deferred)

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

The MMSDeliveryAck is sent when the whe MMS Relay/Server sends the m-acknowledge-ind to the target MMS User Agent (C).

The trigger for the xIRIMMSReadReport is MMS Relay Server receiving the m-read-rec-ind from the target MMS User Agent (C). However, MMS User Agent (C) sending of the m-read-rec-ind is conditional, and therefore, the generation of xIRI MMSReadReport is conditional.

***** End of the new text ****