3GPP SA3#95-LI	s3i240752

# 29 October – 01 November 2024, Las Vegas (US)

Title:	Reply LS on UE-Satellite-UE Communication Architectures

Response to:	LS (S2-2409442) on "Reply LS on KI#3 UE-Satellite-UE Communication Architectures"

Release:	Release 19

Work Item:	5GSAT_Ph3_ARCH

Source:	SA3-LI

To:	SA2

Cc:	SA3, CT1, CT3, CT4

Contact Person:

#### Name:	Nagaraja Rao

E-mail Address:	nagaraja.rao@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3-LI thanks SA2 for their LS and the clarification in pursuing the UE-Satellite-UE communication architectures.

Point number 1

In reference to point number 1 (where the IMS-AGW would be on-board the satellite), SA3-LI would update LI specifications to accommodate the possibility of a change in the IMS-AGW during an active IMS Session.

SA3 LI believes that LI can be performed as long as P-CSCF is in control of the IMS-AGW change.

Point number 2

In reference to point number 2 (where an intermediate UPF (I-UPF) is inserted to the media path with no IMS-AGW for a UE-Satellite-UE communication) the LI specifications for IMS LI will have to be enhanced.  SA3-LI believes to support such an architecture, P-CSCF needs sufficient information to isolate the media flow packets for all IMS sessions, at any time during the IMS session.

In SA3-LI understanding, the following are required for all UE-Satellite-UE IMS sessions, independent of LI:

- P-CSCF is able to identify the I-UPF. For this, I-UPF address/identity will be made available to the P-CSCF.

- P-CSCF is able to identify the tunnel used for the IMS media flow. For this, the tunnel ID used at the I-UPF will be made available to the P-CSCF. If a point-to-point tunnel is used (e.g. on N6), then the UDP port number along with the IP address of the I-UPF shall be made available to the P-CSCF.

- P-CSCF is able to identify the UDP port number (RTP) used for the media packets. Port numbers from the access side (N3) and the core-side (N6) will be available.

- P-CSCF is able to identify the QoS flow. For this, the QFI associated with QoS flow will be made available to the P-CSCF.

SA3-LI kindly asks SA2 to confirm the above understanding.

2. Actions:

To SA2:

ACTION: 	SA3-LI kindly asks SA2 to:

-	In reference to point 1, to confirm SA3-LI that P-CSCF will be able to determine as and when the IMS-AGW is changed during a mid-session irrespective of LI.

-	In reference to point 2, to inform SA3-LI whether the requirements listed in Point Number 2 are correct.

3. Date of Next TSG-SA WG3-LI Meetings:

3GPP SA3#96-LI			28 - 31 January 2025			Sophia Antipolis, FR

3GPP SA3#97-LI			29 April – 02 May 2025			Washington DC, US