3GPP TSG-SA3 Meeting #95-LI	s3i240647

# Las Vegas, United States, 29th Oct 2024 - 1st Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0678 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobile IAB Authorized Indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Authorization status information of the mobile IAB-node is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Authorization status of mobile IAB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorization status information of the mobile IAB-node would still be missing (regulatory issue) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/211 Commit Hash: https://forge.3gpp.org/rep/sa3/li/-/commit/ae7eb8577724aab3d6bd78fa228727234bba90bc |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 6.2.2.2A	Definitions for AMF message Types

##### 6.2.2.2A.1	Type: InitialRANUEContextSetup

The purpose of the InitialRANUEContextSetup type is to provide information the AMF sends to the NG-RAN to request the setup of the UE context. Encoded per TS 38.413 [23] clause 9.2.2.1.

Table 6.2.2.2A.1-1 contains the details for the InitialRANUEContextSetup type.

Table 6.2.2.2A.1-1: Structure of the InitialRANUEContextSetup type

| Field name | Type | Cardi nality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| aMFUENGAPID | AMFUENGAPID | 1 | Identity that the AMF uses to uniquely identify the target UE over the NG Interface, See TS 38.413 [23] clause 9.3.3.1. | M |
| rANUENGAPID | RANUENGAPID | 1 | Identity that the AMF receives from the NG-RAN node uniquely identifying the target UE within the NG-RAN Node. See TS 38.413 [23] clause 9.3.3.2. | M |
| oldAMF | GUAMI | 0..1 | Previous serving AMF’s GUAMI, include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. Format is defined in TS 29.571 [17] clause 5.3.4.1. | C |
| pDUSessionSetupRequest | SEQUENCE (SIZE (1..MAX)) OF PDUSessionSetupRequestItem | 1..MAX | Identifies the PDU Sessions for a UE. Derived from the information in the PDU Session Resource Setup Request Item IE defined in TS 38.413 [23] clause 9.2.2.1. | M |
| allowedNSSAI | AllowedNSSAI | 1 | Indicates the S-NSSAIs permitted by the network. | M |
| mobilityRestrictionList | MobilityRestrictionList | 0..1 | Provides roaming or access restrictions related to mobility from AMF to the RAN Node. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. See TS 38.413 [23] clause 9.3.1.85. | C |
| uERadioCapability | UERadioCapability | 0..1 | Contains the UE Radio Capability information. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. Defined in TS 38.413 [23] clauses 9.3.1.74, and 9.3.1.74a. | C |
| rATFrequencySelectionPriority | RATFrequencySelectionPriority | 0..1 | Used to define local configuration for RRM strategies. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. See TS 38.413 [23] 9.3.1.61. | C |
| uERadioCapabilityForPaging | UERadioCapabilityForPaging | 0..1 | Contains paging specific UE Radio Capability information. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. Defined in TS 38.413 [23] clause 9.3.1.68. | C |
| iABAuthorizedIndicator | IABAuthorizedIndicator | 0..1 | Provides information about the authorization status of the UE to operate as an IAB node. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. See TS 38.413 [23] clause 9.3.1.129. | C |
| nRV2XServicesAuthorization | NRV2XServicesAuthorization | 0..1 | Provides information on the authorization status of the UE to use the NR sidelink for V2X services. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. See TS 38.413 [23] clause 9.3.1.146. | C |
| lTEV2XServiceAuthorization | LTEV2XServiceAuthorization | 0..1 | Provides information on the authorization status of the UE to use the LTE sidelink for V2X services. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. See TS 38.413 [23] clause 9.3.1.147. | C |
| rGLevelWirelineAccessCharacteristics | OCTET STRING | 0..1 | Indicates the wireline access technology specific QoS information corresponding to a specific wireline access subscription. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. Specified in TS 23.316 [122] clause 4.5.1.2. | C |
| uERadioCapabilityID | OCTET STRING | 0..1 | Identifier used to represent a set of UE radio capabilities. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. Defined in TS 23.003 [19] clause 29.2. | C |
| targetNSSAIInfo | TargetNSSAIInfo | 0..1 | Contains the Target NSSAI and Index to RAT/Frequency Selection Priority. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. Defined in TS 38.413 [23] clause 9.3.1.229. | C |
| fiveGProSeAuthorizationIndication | FiveGProSeAuthorizationIndication | 0..1 | Provides information on the authorization status of the UE to use ProSe services. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. Defined in TS 38.413 [23] clause 9.3.1.233. | C |
| mobileIABAuthorizedIndicator | MobileIABAuthorizedIndicator | 0..1 | Provides information about the authorization status of the UE to operate as a mobile IAB node. Include when sent in the INITIAL CONTEXT SETUP REQUEST or when known at the NF. See TS 38.413 [23] clause 9.3.1.259. | C |

##### 6.2.2.2A.12		Enumeration: MobileIABAuthorizedIndicator

The MobileIABAuthorizedIndicator provides information about the authorization status of the mobile IAB node. Defined in TS 38.413 [23] clause 9.3.1.259.

Table 6.2.2.2A.12 contains the details of the MobileIABAuthorizedIndicator type.

Table 6.2.2.2A.9-1: Enumeration for MobileIABAuthorizedIndicator

| Enumeration value | Description |
| --- | --- |
| authorized(1) | Indicates the UE is authorized to operate as a mobile IAB node. |
| notAuthorized(2) | Indicates the UE is not authorized to operate as a mobile IAB node. |

END OF FIRST CHANGE

*** Start of attachment change ***

START OF CHANGE 1

---a/33128/r19/TS33128Payloads.asn
+++b/33128/r19/TS33128Payloads.asn

@@ -1833,7 +1833,8 @@ InitialRANUEContextSetup ::= SEQUENCE

1833	1833		    rGLevelWirelineAccessCharacteristics [13] OCTET STRING OPTIONAL,

1834	1834		    uERadioCapabilityID                  [14] OCTET STRING OPTIONAL,

1835	1835		    targetNSSAIInfo                      [15] TargetNSSAIInfo OPTIONAL,

1836		-	    fiveGProSeAuthorizationIndication    [16] FiveGProSeAuthorizationIndication OPTIONAL

1836	+	    fiveGProSeAuthorizationIndication    [16] FiveGProSeAuthorizationIndication OPTIONAL,

1837	+	    mobileIABAuthorizedIndicator         [17] MobileIABAuthorizedIndicator

1837	1838		}

1838	1839

1839	1840		PDUSessionSetupRequestItem ::= SEQUENCE

@@ -1917,6 +1918,11 @@ FiveGSUpdateType ::= OCTET STRING (SIZE(1))

1917	1918

1918	1919		UnavailabilityPeriodDuration ::= OCTET STRING (SIZE(1))

1919	1920

1921	+	MobileIABAuthorizedIndicator ::= ENUMERATED

1922	+	{

1923	+	    authorized(1),

1924	+	    notAuthorized(2)

1925	+	}

1920	1926		-- ==================

1921	1927		-- 5G SMF definitions

1922	1928		-- ==================

END OF CHANGE 1

*** End of attachment change ***