3GPP TSG-SA3 Meeting #95-LI	s3i240695

# Las Vegas, United States, 29th Oct 2024 - 1st Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0266 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI for IMS HSS Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently there is no method for LI reporting from an HSS deployed to support IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add one new IRI for subscriber record change at the EPC supporting HSS, add capability to report various IRI from IMS supporting HSS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution will remain incomplete. CSPs may not be able to meet LI obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.1, 7.2.3.3, 7.2.4 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*START OF CHANGES**

**START OF FIRST CHANGE**

### 7.2.3	LI at HSS

#### 7.2.3.1	Architecture

The HSS contains subscription-related information for users served by the CSP. The HSS provides the support functions for mobility management, session setup, user authentication and access authorization.

The HSS shall have LI capabilities to generate the xIRIs as described in clause 7.2.3.3. The present document specifies two options for HSS LI capabilities:

1.	Use TS 33.107 [11] and TS 33.108 [21] natively as defined in those documents.

2.	Use the capabilities specified below in the present document for stage 2 and in TS 33.128 [15] for stage 3.

When the deployed architecture supports service based interfaces between the HSS and IMS (e.g. Nhss_IMS), TS 33.128 [15] shall be used for stage 3 LI reporting. For details of stage 2 requirements, see clause 7.2.X of the present document.

Extending the generic LI architecture presented in clause 5, figure 7.2-2 below gives a reference point representation the LI architecture with HSS as a CP NF providing the IRI-POI functions.

Figure 7.2-2: LI architecture for LI at HSS

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides it to the LIPF.

The LIPF present in the ADMF provisions IRI-POI (over LI_X1) present in the HSS and MDF2.

The IRI-POI present in the HSS detects the target UE's service area registration and subscription related functions, generates and delivers the xIRI to the MDF2 over LI_X2. The MDF2 generates and delivers the IRI messages based on received xIRI to the LEMF over LI_HI2.

The HSS shall provide the IRI-POI functions independent of the services on which the interception is active.

When multiple intercepts are active, IRI-POI functions in the HSS may send one xIRI which can then be distributed to the LEMFs associated with those multiple intercepts from the MDF2.

**END OF FIRST CHANGE**

**START OF SECOND CHANGE**

#### 7.2.3.3	IRI events

The IRI-POI present in the HSS shall generate xIRI, when it detects the applicable events specified in TS 33.107 [11].

The IRI-POI present in the HSS shall also generate a start of intercept with already registered target xIRI when the IRI-POI present in the HSS detects that intercept has been activated for a UE that has existing context in the HSS. Format of this xIRI is described in TS 33.128 [5] clause 7.2.3.3.3.

The IRI-POI present in the EPC supporting HSS shall generate a subscriber record change record when the IRI-POI in the EPC-HSS observes an update to the IMEI context for the target at the HSS.

If HSS-UDM interworking is supported, the IRI-POI present in the HSS shall generate a serving system xIRI as defined in TS 33.128 [5] clause 7.2.3.3.2.

A serving system xIRI is generated when the IRI-POI present in the HSS detects that the HSS has received a roaming status update from the UDM as part of a UE context update.

NOTE:	The serving system xIRI may carry the information of one or more serving systems based on the target UE's network connectivity.

**END OF SECOND CHANGE**

**START OF THIRD CHANGE**

### 7.2.4	LI at the IMS supporting HSS

#### 7.2.4.1	Architecture

The IMS-HSS contains subscription-related information IMS users served by the CSP. The IMS-HSS provides the support functions for mobility management, session setup, user authentication and access authorization to a 5G network in support of enhanced IMS services (e.g. VoNR).

The IMS-HSS shall have LI capabilities to generate the xIRIs as described in clause 7.2.X.3.

When the deployed architecture supports service based interfaces between the HSS and IMS (e.g. Nhss_IMS), TS 33.128 [15] shall be used for stage 3 LI reporting.

The presented in figure 7.2-2 in clause 7.2.3.1 gives a reference point representation the LI architecture with HSS (or IMS-HSS) presented as an NF providing the IRI-POI functions.

#### 7.2.4.2	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the IMS-HSS:

-	IMSI.

-	IMEI.

-	MSISDN.

-	IMPU/IMPI.

The interception performed on the above identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

#### 7.2.4.3	IRI events

The IRI-POI present in the HSS shall generate a start of intercept with registered target xIRI when the IRI-POI present in the IMS-HSS detects that intercept has been activated for a UE that has existing registration context in the IMS-HSS. Format of this xIRI is described in TS 33.128 [5] clause 7.2.4.3.2.

A serving system xIRI is generated when the IRI-POI present in the IMS-HSS detects that the IMS-HSS has received a roaming status update from the UDM as part of a UE context update or has received an authorization request including a visited network identifier.

NOTE:	The serving system xIRI may carry the information of one or more serving systems based on the target UE's network connectivity.

The IRI-POI present in the IMS-HSS shall generate a subscriber record change record when the IRI-POI in the IMS-HSS observes an update subscriber information for the target at the IMS-HSS.

#### 7.2.4.4	Common IRI parameters

The list of xIRI parameters is specified in TS 33.128 [15]. All xIRIs shall include the following information:

-	Target identity.

-	Time stamp.

#### 7.2.4.5	Specific IRI parameters

The parameters in each xIRI are defined in TS 33.128 [15].

#### 7.2.4.6	Network topologies

The HSS shall provide the IRI-POI functions in the following network topology cases:

-	Non-roaming case.

-	Roaming case, in HPLMN.

**END OF THIRD CHANGE**

**END OF ALL CHANGES**