Source:	VoiceAge Corporation

Title:	DCR FOA/HOA3 test experiment using recorded content

Document for:	Information

Agenda Item:	1.3

# Introduction

The goal of this contribution is to report on a P.800 DCR test [1] experiment run at the immersive listening laboratory at the University of Sherbrooke with the goal to contribute to the collective experience with immersive listening testing using naïve listeners. It is meant as a complement to results of previous DCR experiments with naïve listeners [2] contributed by the same source. The test used 7.0+4 loudspeaker listening setup.

# Audio database

  - Real-live recorded samples using Zylia ambisionic microphone (HOA3). The recording was done in a large and rather reverberant room.

  - Phonetically balanced single sentences concatenated into pairs with related meaning.

  - 4 male and 4 female talkers, always a male and a female talker in a sentence pair.

  - Sentence pairs simulating a conversation with natural transition from one talker to another. Half of the samples partially overlapped. The targeted overlap was about 30%.

  - Length of the samples – 6 or 7 s.

  - 48 kHz sampling rate.

  - HOA3 and FOA input codec format.

  - Talkers Scenarios

  - 2 talkers sitting at a table, at 90° with respect to the microphone. The utterances were non-overlapping.

  - 2 talkers sitting at a table, face-to-face, with the microphone in between. The utterances were overlapping.

  - 1 talker sitting at a table, second talker walking around the table. The utterances were non-overlapping.

  - 2 talkers sitting at a table at 90°, one talker stands up and sits down while talking. The utterances were overlapping.

  - 2 talkers walking side-by-side around the table. The utterances were non-overlapping.

  - 2 talkers walking around the table in opposite directions. The utterances were overlapping.

  - Background

  - Reproduced stereo music at 15 dB SNR. Constant position of the speakers in a usual stereo setup.

  - Different music was used for each speech sentence pair.

# Test Setup

- P.800 DCR test, instructions mentioning spatial aspect. The instructions are provided below in the section Instructions to listeners.

- 4 categories corresponding to the different pairs of talkers.

- 24 naïve listeners.

- 6 panels, each using different audio samples. Further, to distribute the spatial effects evenly among and within panels, different Talker Scenarios are used for each talker pair with a panel.

- 4 listeners per panel, one listener at a time in the 7.0+4 immersive room.

- Each condition was evaluated 24 x 4 = 96 times.

- Anchors - P.50 MNRUs (Modulated Noise Reference Units) [4] – applied coherently (using the same seed) to all ambisonic channels.

- No SDRUs (Spatial Distortion Reference Unit) or ESDRUs spatial anchors were used as they are not defined for loudspeaker listening.

- CuT – mutli-mono EVS applied on FAO and HOA3 channels.

- Rendering

  - All conditions rendered to 7.0+4 loudspeaker system using All-Round Ambisonic Decoding (AllRAD) [5].

  - Rendering was done on concatenated files.

- Level adjustment

  - The level was adjusted to -26 LKFS.

  - The direct signal level was first measured on the 7.0+4 signal using B.1770 [3] and level difference was computed with -26 LKFS (Loudness, K-weighted, relative to Full Scale). The corresponding gain was then applied to the original HOA3 input channels. No level readjustment was done on the coded signals.

- Loudspeaker listening setup - 7.0+4 Genelec SAM 3031 speaker setup in the following configuration:

| Speakers | Azimuth | Elevation |
| --- | --- | --- |
| Left front | 30 | 0 |
| Right front | -30 | 0 |
| Centre front | 0 | 0 |
| LFE | - | - |
| Left rear surround | 135 | 0 |
| Right rear surround | -135 | 0 |
| Left side surround | 90 | 0 |
| Right side surround | -90 | 0 |
| Left front height | 30 | 35 |
| Right front height | -30 | 35 |
| Left rear surround height | 135 | 35 |
| Right rear surround height | -135 | 35 |

Instructions to Listeners

We used the following instructions to listeners emphasizing the immersive aspect of the test. In particular, we used the word Alteration instead of the word Degradation to avoid confusion of some listeners whether reducing spatial image should be considered as a degradation. When drafting the instructions, we took into account the recommendations of section 7.2 of [6].

Instructions in French

INSTRUCTIONS AUX AUDITERUS NAÏFS POUR UN TEST DCR IMMERSIF Durant cette expérience, vous allez évaluer des systèmes qui pourraient être utilisés dans des services de télécommunications futurs utilisant l’audio immersif. L’audio immersif signifie que vous pouvez localiser différentes sources de son autour de vous. Par exemple, un premier locuteur peut parler à votre gauche et un deuxième locuteur à votre droite, un locuteur peut se déplacer, etc. Dans chaque essai, vous entendrez un échantillon d’un signal audio de référence, suivi par un échantillon de test. L’échantillon de test a le même contenu que l’échantillon de référence, mais il a été possiblement altéré après son passage par le système de télécommunications. Votre tâche est d’évaluer le niveau de l’altération du deuxième échantillon par rapport au premier échantillon. L’altération peut être causée aussi bien par un changement de la qualité du son (par exemple à cause d’un bruit additionnel, une rugosité, des clics ou autres distorsions), que par une modification de la représentation spatiale (par exemple changement dans la localisation de la source du son, de sa distance, de son étendue, de son déplacement, etc.) Écoutez attentivement les deux échantillons dans un essai. Veuillez ensuite enregistrer votre opinion globale sur l’altération que vous percevez dans le deuxième échantillon par rapport au premier échantillon, en utilisant l’échelle suivante : 5 - Aucune altération audible 4 - Faible altération parfois audible 3 - Altération audible 2 - Altération clairement audible 1 - Forte altération clairement audible Notez que le niveau d’altération présent dans différents échantillons de test est supposé couvrir toute l’échelle de vote durant l’expérience. Veuillez ne pas discuter vos opinions avec les autres auditeurs participants à l’expérience. Si vous avez des questions, veuillez-vous adresser à l’administrateur du test.

Instructions in English

INSTRUCTIONS TO NAÏVE LISTENERS FOR IMMERSIVE DCR TEST In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio means that you can locate various sound sources around yourself. For example, a first talker may appear to talk from the left-hand side and a second talker from the right-hand side, a talker can be moving, etc. In each trial, you will hear a reference audio sample followed by a test sample. The test sample has the same content as the reference sample, but it was possibly altered after it has passed through a telecommunication system. Your task is to evaluate the overall alteration of the second sample compared to the first sample, comprising both alteration of the sound quality (e.g., due to additional noise, roughness, clicks or other distortions), and alteration of the spatial representation (e.g., sound source location, distance, spatial width, movement, etc.). You should listen carefully to both samples within a trial. When they have finished, please record your overall opinion about the amount of any alteration you can perceive in the second sample relative to the first sample using the following rating scale: 5 - Alteration is not audible 4 - Slight alteration is sometimes audible 3 - Alteration is audible 2 - Alteration is clearly audible 1 - Strong alteration is clearly audible Note that the level of alteration present in different test samples is expected to span the complete range of the rating scale during the experiment. Please do not discuss your opinions with other listeners participating in the experiment. If you have any questions, please ask the test administrator.

# Loudspeaker Listening Test Results for live-recorded database

# Screening of listeners

In order to be considered, a listener had:

- To use the whole voting scale during the session. In other words, he must have voted at least once “1” and at least once “5”.

- To vote, in average, the direct condition better than or equal to the MNRU 29 dB condition. To reflect the fact that the perceptual quality of MNRU 29 dB is close to Direct, the listener was still kept if the median of his votes was below 4.

- To vote, in average, the MNRU 29 dB condition better than the MNRU 24 dB condition.

- To vote, in average, the MNRU 24 dB condition better than the MNRU 19 dB condition.

- To vote, in average, the MNRU 19 dB condition better than the MNRU 14 dB condition.

# Comments

- Overall, naïve listeners could reliably detect coding deficiencies (MOS increase with bitrate).

- Addition of ambisonic channels (i.e., HOA3 vs FOA) consistently increases the perceptual experience.

- Naïve listeners do not seem to be too sensitive to the spatial aspect, e.g., differentiating between FOA and HOA3. Nevertheless, they were still able to discriminate the direct HOA3 from FOA with statistical significance.

# Main differences with respect to the Loudspeaker setup used in [2]

- The samples were live-recorded, the reverberance was stronger than in the artificially created database used in [2].

- The audio scenes were more difficult as some talkers were moving.

- On the other hand, the sound distribution was more limited spatially, covering only a part of the whole range of azimuth and elevation.

- More explicit Listener Instructions were used, and a written copy was provided to the listeners.

# Issues, potential areas of improvement

- The distribution of sound sources was concentrated within an azimuth of about 180°. Better spatial distribution covering the whole range of 360° would be beneficial.

- In Talker Scenario number 4, the change in vertical position was very difficult to detect.

# References

Recommendation ITU-T P.800 (08/1996): Methods for subjective determination of transmission quality.

S4aA220007 - DCR test experiments for FOA and HOA3 input in 7.0+4 and binaural listening setup.

Recommendation ITU-R BS.1770-04 (10/2015): Algorithms to measure audio programme loudness and true-peak audio level.

Recommendation ITU-T P.811 (01/2019): Subjective test methodology for evaluating Speech oriented stereo communication systems over headphones.

F. Zotter and M. Frank, “All-Round Ambisonic Panning and Decoding,” in J. Audio Eng. Soc., Vol. 60, No. 10, 2012.

SG12-TD138: Draft new ITU-T P.800-series – Supplement P.SUPPL800: ITU-T Rec. P.800 use case examples.