Source:	HEAD acoustics GmbH

Title:	Test Signals and Performance Evaluation for FOA mode

Document for:	Discussion

# Introduction

For the performance evaluation of UEs supporting first-order Ambisonics (FOA), an initial proposal was made in [1] within the work item ATIAS. Based on a defined multi-talker scenario and an AM-FM-modulated signal, the approach basically calculates the level of the corresponding spectral components of the four Ambisonics channels (W, X, Y, Z). However, as already indicated in [2], several details of this test method are still undefined.

In order to progress on a well-defined test method for FOA-transmitted signals, the source investigated test signal, setup and analysis more in detail. A preliminary summary of the current findings is provided in the present document.

# Test Signal

## AM-FM-Modulated Signal

Clause 7.2.4 of Recommendation ITU-T P.501 [3] specifies the generation of amplitude-/frequency-modulated test signals, which consists of orthogonal frequency components from two talkers. This signal structure allows to analyze also the captured, processed and transmitted superposition of both talkers even in mono signals. The two signals are generated as follows:

sfm(1,2)t= nA(1,2)(n)⋅cos[2πf0(1,2)n⋅t+μfm(1,2)n⋅sin2πffm(1,2)⋅t]  n = 1,2,...

where μfm(1,2)n=∆f(1,2)nffm(1,2)

sam(1,2)t=[1+μam(1,2)⋅cos2πfam(1,2)⋅t]

In ITU-T P.501, the following parameters are defined in a frequency-independent manner:

ffm(1,2)=1 Hz

μam(1,2)=23

fam(1,2)=3 Hz

The amplitudes A(1,2)(n) that determine the signal levels may be chosen according to the application (see also section 0). The remaining parameters f0(1,2) and ±∆f(1,2) describe center frequency and modulation bandwidth are provided in ITU-T P.501 in tabular form.

NOTE 1:	These lists of frequencies and corresponding modulation bandwidths were arbitrarily determined to represent typical voiced parts of real speech. For this reason, the center frequencies are not evenly spaced (like e.g., as 1/3rd octaves).

In [1], it was only suggested to use center frequencies of 1/3rd octave bands for f0(1,2) (range: 250 Hz – 19 kHz), otherwise no parameters were provided.

NOTE 2:	The two test signals in ITU-T P.501 are specified in a range from 125 Hz up to 7 kHz (see Table 7-6) for wideband, super-wideband and fullband. These minimum/maximum frequencies are considered to be sufficient to model voiced parts of human speech.

Another important parameter of the AM-FM-signal generation is the modulation bandwidth ∆f(1,2), which approximately defines the bandwidth of each voiced component. In ITU-T P.501, these values are provided in along with the center frequencies in Table 7-6. To determine arbitrary modulation bandwidths for different frequencies that are "compatible" (up to a certain degree) with ITU-T P.501, the tabular values are provided as a scatter plot in Figure 1. It can be observed that the parameter ∆f(1,2) linearly increases up to 2 kHz and the remain constant.

Figure 1: Modulation bandwidth vs frequency of ITU-T P.501

A linear regression with clipping according to equation 1 is applied to determine a frequency-dependent modulation bandwidth:

∆ff0=min(40.0,  0.0195033⋅f0-0.230789)	(1)

## Shaping Filter

To generate speech-like frequency characteristics of speech, ITU-T P.501 specifies a lowpass at 250 Hz that provides a slope of 5 dB per octave. The description in [1] suggests a slope of 3 dB/octave. Another reasonable shaping filter would be the average speech spectrum from ITU-T P.50 [4]. An upcoming update of ITU-T G.191 [5] will provide an implementation for frequencies up to 20 kHz. The three shaping filters are illustrated in Figure 2.

Figure 2: Possible shaping filters of test signal

## Signal used in Evaluation

For the current evaluation, the following parameters were chosen:

- ffm(1,2)=1 Hz, μam(1,2)=23  , fam(1,2)=3 Hz (same as in ITU-T P.501)

- Interleaved 1/3rd octave bands as center frequencies, from 100 Hz to 14 kHz

- Duration: 8 s

- Speech-like shaping filter

- Calibrated playback level (at MRP) was set to -1.7 dBPa according to ITU-T P.581 [6].

  - Overall active speech level according to ITU-T P.56 [7]

The spectrum of the resulting signal is shown in Figure 3.

Figure 3: Generated AM-FM-Signal for the current evaluation

## Level measurement

In Tdoc [2], the question was raised how the separate levels for speakers A and B can be calculated from captured signals that contain the superposition of both spectrally orthogonal sources. The general approach is to consider only the frequency bins/bands of the corresponding talker/source and use these specific frequencies in the subsequent analysis (like e.g., frequency response, level calculation).

Beside several basis analysis parameters to be defined (e.g., FFT size, window, overlap and/or fraction of octave band analysis), there are two ways to determine these frequency bins/bands from the source spectra:

1.	Since the values of Δf and f0 should be known for talker A/B separately, the bins closest to (f0-Δf, f0+Δf) are chosen. However, this requires providing the correct side-/metadata to the calculation method.

2.	The spectrum is analyzed with a peak-detection algorithm.

The identified frequency bins/bands can then be applied on the captured signals.

# Test Setup

Due to lack of real terminals supporting FOA encoding, a commercially available Ambisonics microphone was used as a device under test (DUT), which was connected via ASIO interface to the measurement equipment. Even though this microphone is not really a communication device; however, it is expected that the measurement and subsequent analysis should show clear results in terms of the level metrics. Figure 4 illustrates the difference between a test setup with a real terminal and with the dummy device.

Figure 4: Test setup and signal processing with real and dummy DUT

The test setup and the considered talker positions are illustrated in Figure 5 in 3D view and in spherical coordinate system.

Figure 5: Measurement setup

Talkers A and B are represented by two head and torso simulators (HATS) according to ITU-T P.58 [8], which are positioned at distances 30 cm and 50 cm (MRP to acoustic center of DUT). For each of the six source positions (talker A), five counter-positions (talker B) including the swapped talker case (source position: talker B, counter-position: talker A) were measured, leading to 30 recordings per distance. In addition, concurrent as well as separated single talk were measured. To minimize the number of HATS repositions, the DUT was rotated accordingly (especially for positions in Z-direction).

The two HATS and the DUT/microphone were setup in a semi-anechoic test room that is typically used for terminal testing according to e.g., 3GPP TS 26.131/132 [9] [10], i.e., free-field conditions cannot be expected for frequencies lower than 275 Hz.

# Measurement Results

[TBD: Results will be presented at SA4#122 in Athens]

# Conclusion

The initial proposal [1] for a performance evaluation of UEs providing encoded FOA output was reviewed, and several missing parameters were investigated more in detail. For an upcoming measurement series, one set of parameters was chosen to generate a test signal with properties that are close to ITU-T P.501 as well as to the initial proposal in [1]. The source offers to share the source signals and the configurable scripts for the generation with interested parties upon request.

An initial measurement series was conducted with a commercially available Ambisonics microphone as a device under test. The level-based analyses of the recordings will be conducted according to the proposals made in [1]. In addition, also other analyses like e.g., overall/per talker frequency response can be carried out with the additionally available recordings.

Analysis results are planned to be presented at SA4#122.

# References

| [1] | 3GPP S4-220729, „On ATIAS acoustic performance testing for FOA audio,“ Dolby Laboratories Inc.. |
| --- | --- |
| [2] | 3GPP S4-221412, „Clarifications for ATIAS FOA audio tests,“ Nokia Corporation. |
| [3] | ITU-T Recommendation P.501, „Test signals for use in telephony and other speech-based applications,“ 06/2022. |
| [4] | ITU-T Recommendation P.50, „Artificial voices,“ 09/1999. |
| [5] | Recommendation ITU-T G.191, „Software tools for speech and audio coding standardization,“ 07/2022. |
| [6] | ITU-T Recommendation P.581, „Use of head and torso simulator (HATS) for hands-free and handset terminal,“ 02/2014. |
| [7] | ITU-T Recommendation P.56, „Objective measurement of active speech level,“ 12/2011. |
| [8] | ITU-T Recommendation P.58, „Head and torso simulator for telephonometry,“ 06/2021. |
| [9] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release-17.1. |
| [10] | ETSI TS 126 132 v17.1.0, „Universal Mobile Telecommunications System (UMTS); LTE; 5G; Speech and video telephony terminal acoustic test specification,“ 2022-04. |