SA4-e (AH) Audio SWG post 123-e						    S4aA230059

28th April 2023

Source:	Fraunhofer IIS

Title:	Proposed Updates to IVAS Processing Plan

Document for:	Discussion and Agreement

Agenda Item:	1.4

- Introduction

This contribution aims to provide missing text for IVAS-7a [1], with focus on especially item/pattern naming and auxiliary things such as randomization. The text is inspired by EVS-7c [2] and was adapted to the IVAS selection testing. The source would like to point out the following main points:

- It is proposed to use uncorrelated noise [-4;+4] for the preamble across audio tracks. The preamble is intended to simulate weak recording noise, which can be considered uncorrelated.

- It is proposed to adapt the audio sample denotation for both, speech and non-speech content types to the denotation used for mixed/music in EVS-7c. Given the multitude of different combinations of male and female talkers within one sentence pair, it is proposed to leave up the exact definition to the test plan IVAS-8a [3], but not reflect it in the audio sample names itself.

- Simplification of the random seed mechanism for error pattern and noise-offset generation: It is proposed to make the random seed dependent on a MASTER_SEED and a per-run number P which is dependent on experiment and listening lab. Compared to EVS, this removes extra seeds dependent on background noise and frame-error-rate, which are considered as too much effort with comparable little gain in overall randomization.

- In the edited text blocks, reference to codecs which are not used in selection test were removed.

- Many mechanical adaptations.

- Proposed Changes

The following changes are proposed:

Start change 1

## 3.3 Experiments

The selection test plan‎ (IVAS-8a) [1] describes the listening experiments. For the processing of each experiment, a two-letter experiment designator shall be used for the file naming convention. An overview of all experiments is presented in Table 1.

Table 1: Overview of experiments

| Experiment Designator | Experiment Name (IVAS-8a) | Input format | Source Material |
| --- | --- | --- | --- |
| p01 | P800-1 | Stereo | Clean speech |
| p02 | P800-2 | Stereo | Speech+Background |
| p03 | P800-3 | Stereo | Mixed & Music |
| p04 | P800-4 | FOA | Clean speech |
| p05 | P800-5 | FOA | Speech+Background |
| p06 | P800-6 | 1 Object | Clean speech |
| p07 | P800-7 | 2 Objects | Clean speech |
| p08 | P800-8 | MASA | Clean speech |
| p09 | P800-9 | MASA | Speech+Background |
| m1a | BS1534-1a | Stereo | Generic Audio |
| m1b | BS1534-1b | Stereo | Generic Audio |
| m2a | BS1534-2a | 5.1 | Generic Audio |
| m2b | BS1534-2b | 5.1 | Generic Audio |
| m3a | BS1534-3a | 7.1.4 | Generic Audio |
| m3b | BS1534-3b | 7.1.4 | Generic Audio |
| m4a | BS1534-4a | FOA | Generic Audio |
| m4b | BS1534-4b | FOA | Generic Audio |
| m5a | BS1534-5a | HOA3 | Generic Audio |
| m5b | BS1534-5b | HOA3 | Generic Audio |
| m6a | BS1534-6a | Objects | Generic Audio |
| m6b | BS1534-6b | Objects | Generic Audio |
| m7a | BS1534-7a | MASA | Generic Audio |
| m7b | BS1534-7b | MASA | Generic Audio |

## 3.4 Listening lab designators

The following table lists the responsible listening labs (LL) with their dedicated LL designator to be used for the file naming convention.

Table 2: List of listening lab designators

| LL designator l | Listening Lab Company |
| --- | --- |
| a | Force Technology |
| b | Head Acoustics/ IKS |
| c | MQ University |
| d | Mesaqin.com |

End change 1

Start change 2

## 3.9 Preamble definition

A preamble shall be low-level noise obtained as a digital silence of 16-bit values with amplitudes between +4 and -4. Uncorrelated noise shall be used across audio tracks.

End change 2

Start change 3

### 3.10.2 Naming examples of the input files and processed output files

The following sections describe the file naming conventions where all letters in blue indicate variables in the filename while black letters indicate constants.

#### 3.10.2.1 P.SUPPL800 Input content files

The filenames of the input content samples are represented by:

leeeayszz.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.met for metadata-assisted spatial audio

leeeayszz.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- a stands audio, and y is the per experiment category according to IVAS-8a

- s stands for sample and zz is the sample number; 01, 02, 03, 04, 05, 06, 07,  where 07 is the preliminary

#### 3.10.2.2 BS.1534 Input content files

The filenames of the input content samples are represented by:

leeeayszz.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.met for metadata-assisted spatial audio

leeeayszz.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. m1a (see Table 1)

- a stands audio, and y is set to 1

- s stands for sample and zz is the sample number; 01 … 12

[

#### 3.10.2.3 Noise files

The filenames of the input noise samples are represented by:

nx.wav

where:

- n stands for noise[, and x is c for car noise, s for street noise and o for office noise].

]

#### 3.10.2.4 Processed P.SUPPL800 content files

The filenames of the processed content samples are represented by:

leeeayszz.cnn.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.cnn.met for metadata-assisted spatial audio

leeeayszz.cnn.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- a stands audio, and y is the per experiment category according to IVAS-8a

- s stands for sample and zz is the sample number; 01, 02, 03, 04, 05, 06, 07  where 07 is the preliminary

- c stands for the condition with the number nn = 01, 02, .. as specified in IVAS-8a

#### 3.10.2.5 Processed BS.1534-4 content files

The filenames of the processed content samples are represented by:

leeeayszz.cnn.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.cnn.met for metadata-assisted spatial audio

leeeayszz.cnn.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. m1a (see Table 1)

- a stands audio, and y is set to 1

- s stands for sample and zz is the sample number; 01 … 12

- c stands for the condition with the number nn = 01, 02, .. as specified in IVAS-8a

End change 3

Start change 4

### 4.1.1 Source material requirements

The input source material shall follow the format specification in section 6.

### 4.1.2 Concatenated sequences processing

In all Experiments, the pre-processed material will be processed in concatenated files comprising a preamble followed by the samples being evaluated. The preamble concatenation will be 10 seconds long.

Audio files are concatenated after level adjustment. The concatenated files need to be 20 ms block aligned per audio track for further processing. Therefore, the concatenated sequence needs to be padded at the end with the first samples of the preamble to reach an integer number of 20 ms blocks for each audio track.

### 4.1.3 Frame error application

In frame erasure conditions, the erasures shall affect the same segments of speech signal for the CuT and reference codecs. This shall be done by compensating for all encoder-side delays (or for all encoder-side delays of the core layer in case of embedded codecs). The CuT encoder and decoder executables shall compensate the delay of the output files. The delay of reference codecs shall be compensated prior to the processing by the reference encoder, as specified in 5.2.6.

Note: No delay compensation is applied for jitter related delay.

Note: Exact delay compensation might not be possible.

### 4.1.4 File naming error patterns

Frame erasure files will have the name “eeefrr.g192”, where:

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- f stands for frame loss pattern file

- rr stands for the frame loss rate in per cent[, i.e. 03, 05, 06, or 10].

### 4.1.5 File naming jitter profiles

The MTSI jitter profiles have the file names “dly_error_profile_x.dat” where x is the profile number, i.e. [1,2,3,4,5,6,7,8,9,10].

### 4.1.6 File naming jitter derived error patterns

The error patterns which are derived from the MTSI jitter profiles have the file names “f_profile_x.g192”, where:

- f stands for frame loss pattern file

- profile_x stands for the number of the MTSI jitter profile, see 4.1.5

[

### 4.1.7 File naming rate switching profiles

Rate switching profiles  will have the name “eeersx”, where:

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- rs stands for rate switching file

- x stands for the sequence in the experiment, i.e. 1, 2, etc.

]

### 4.1.8 Concatenation setup

Speech and music files are concatenated after level adjustment. If the format includes metadata (metadata-assisted spatial audio, object-based audio), same concatenation is applied to the corresponding metadata-files.

Concatenation for P.SUPPL800 is carried out according to Table X below where all samples are concatenated from left to right and line by line. The concatenation order is therefore a1s1, a3s1, a2s1, a4s1, a5s1, a6s1, a2s2, a4s2, etc. [The final concatenated speech file results in one 336 seconds long sequence (10 sec preamble + 6x7x8 sec speech/music file).] For mixed content and music, the total length may vary. For conditions that include background noise, the speech files must be concatenated prior to the addition of the background noise which will be provided in 300s long files. (Editor’s note: If individual background noises per category are defined, the exact concatenation procedure is FFS). The starting sample (offset) of the noise file is chosen randomly as described in Annex C.4.2.

Table X: Concatenation order of P.SUPPL800 files

| Sample | Item sequence in concatenated files |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | a1 | a3 | a2 | a4 | a5 | a6 |
| 2 | a2 | a4 | a1 | a5 | a3 | a6 |
| 3 | a1 | a6 | a3 | a5 | a2 | a4 |
| 4 | a2 | a5 | a6 | a1 | a3 | a4 |
| 5 | a3 | a2 | a1 | a6 | a5 | a4 |
| 6 | a5 | a1 | a2 | a6 | a4 | a3 |
| 7 | a6 | a2 | a1 | a5 | a3 | a4 |

For all BS.1534 Tests, the files are concatenated in ascending order (by sample number).

For all Experiments, the processed concatenated files should be divided into separate speech/music samples. This must be performed before final up-sampling stage, and a cosine (Hanning) window of duration 100ms must be applied to the start and end of the separated files. The procedure for splitting and windowing concatenated files is described in 5.1.10.

For the experiments where IVAS is tested in the context of JBM testing, 0.2s of pure digital silence will be added before and after each audio sample during concatenation.

End change 4

Start change 5

- Annex C  Randomization scripts

The randomization scripts require one master seed, which will be provided by the HL. The master seed (MASTER_SEED) determines all parameters for noise files, EPFs, jitter and rate switching.

In addition to the master seed, a pre-run number P is chosen per experiment and lab, which is used in combination with the master seed to determine error patterns and noise file offsets.

Table Y: Definition of Pre-run numbers P

| Experiment | LL a Pre-run number P | LL b Pre-run number P | LL c Pre-run number P | LL d Pre-run number P |
| --- | --- | --- | --- | --- |
| P800-1 | 101 | 102 | 103 | 104 |
| P800-2 | 105 | 106 | 107 | 108 |
| P800-3 | 109 | 110 | 111 | 112 |
| P800-4 | 113 | 114 | 115 | 116 |
| P800-5 | 117 | 118 | 119 | 120 |
| P800-6 | 121 | 122 | 123 | 124 |
| P800-7 | 125 | 126 | 127 | 128 |
| P800-8 | 129 | 130 | 131 | 132 |
| P800-9 | 133 | 134 | 135 | 136 |
| BS1534-1a | 137 | 138 | 139 | 140 |
| BS1534-1b | 141 | 142 | 143 | 144 |
| BS1534-2a | 145 | 146 | 147 | 148 |
| BS1534-2b | 149 | 150 | 151 | 152 |
| BS1534-3a | 153 | 154 | 155 | 156 |
| BS1534-3b | 157 | 158 | 159 | 160 |
| BS1534-4a | 161 | 162 | 163 | 164 |
| BS1534-4b | 165 | 166 | 167 | 168 |
| BS1534-5a | 169 | 170 | 171 | 172 |
| BS1534-5b | 173 | 174 | 175 | 176 |
| BS1534-6a | 177 | 178 | 179 | 180 |
| BS1534-6b | 181 | 182 | 183 | 184 |
| BS1534-7a | 185 | 186 | 187 | 188 |
| BS1534-7b | 189 | 190 | 191 | 192 |

End change 5

Start change 6

- C.3 Initialization of gen-patt tool

In order to initialize the gen-patt tool, an sta file needs to written. Therefore, two templates are provided:

Template for 6% frame error rate:

EID

BER           = 0.060000

GAMMA         = 0.000000

RAN-seed      = 0x176d71ac

Current State = G

GOOD->GOOD    = 0.880000

GOOD->BAD     = 1.000000

BAD ->GOOD    = 0.880000

BAD ->BAD     = 1.000000

Template for 3% frame error rate:

EID

BER           = 0.030000

GAMMA         = 0.000000

RAN-seed      = 0xab5d4825

Current State = G

GOOD->GOOD    = 0.940000

GOOD->BAD     = 1.000000

BAD ->GOOD    = 0.940000

BAD ->BAD     = 1.000000

where the RAN-seed needs to be replaced in HEX format. To generate these numbers, use

random -n 1 -s MASTER_SEED –d P –r 0 99999999

where P is the pre-run number according to Table Y.

- C.4 Random noise file selection

- [

- C.4.1 Overview

Editor’s note: Exact procedure to be decided.

- C.4.2 Background noise file offset

To randomly select an offset for the background noise, use:

random.exe -n 1 -s MASTER_SEED –d P –r 1 2400000

where the resulting value determines  the offset to be used for the dedicated background noise file. The value Q depends on the lab and the experiment as outlined in [TBD].

]

End change 6

- Conclusion

It is proposed to agree on the proposals outlined in Section 2 of this document and include the changes into IVAS-7a.

References:

[1] S4-230690 - IVAS-7a: Processing plan for selection phase, v0.9.0

[2] S4-141392 – EVS-7c: Processing functions for characterization phase v1.1.0

[3] S4-230703 – IVAS-8a: Test Plan for selection phase, v0.8.0