SA4-e (AH) Audio SWG post 126						    S4aA230124

15th December 2023

Source:	Fraunhofer IIS

Title:	Proposal for IVAS Characterization Tests

Document for:	Discussion

Agenda Item:	1.4

Introduction

At SA4#125-post and SA4#126, first proposals on the IVAS characterization testing have been made. The source would like to discuss general concepts for the IVAS characterization testing and come up with a proposal for agreement.

Purpose of the codec characterization testing is to evaluate the performance of a newly introduced codec ideally against state-of-the-art solutions, to provide guidance for service providers and device manufacturers: A new codec might either provide new operation points or significant changes on the QoE of existing operation points, which might affect new services or the quality of existing services.

For IVAS, the impact on existing operation points is probably negligible since IVAS comes with a bit-exact EVS implementation for the coding of monoaural speech. However, IVAS introduces many new operation points by means of immersive audio.

Discussion of Concepts and Test Capacity

The source sees the following general concepts for the IVAS characterization testing. It needs to be understood that in practice more than a concept might be applied in different tests or concepts might be combined even within a single test.

In the IVAS selection tests, the IVAS candidate was for many operation points tested against the corresponding performance requirements. A repetition of testing of the exact same operation points brings little additional value for potential customers, since this data is already available in the IVAS selection test results.

Testing against current service quality

In 2014, the EVS characterization tests were carried out. The probably two most important features of the EVS codec over deployed 3GPP codecs were firstly the support for SWB audio bandwidth and secondly improved error robustness. Consequently, the in hindsight most helpful tests were the ones that demonstrated these features over the existing codecs and services (multi-bandwidth experiments, experiments testing PLC).

As stated above, IVAS is the first 3GPP communication codec which introduces immersive audio. Similar to the multi-bandwidth testing for EVS, it is seen as uttermost importance to test these new features against existing codecs and services. As far as the source is aware, currently only monaural communication audio services are deployed commercially. It is thus proposed to test the immersive operation modes against existing monoaural operation points. Rate distortion curves for immersive and monaural audio operation points help to also determine potential crossing points in service quality. In addition, TS 26.114 (IMS) [2] also specifies dual-mono operation for EVS. In addition to the comparison against EVS mono, a comparison of immersive versus stereo operation might be considered.

Testing of previously untested operation points

A good test coverage of operating is important for service providers in order to determine which operation should be used in service.

Due to the many supported audio formats and rendering features, only a subset of the IVAS operation points has been tested so far. The focus of the selection tests was on pass-through operation. The combined formats (OSBA, OMASA), EVS compatible stereo downmix and JBM have not been tested. Also, the renderer itself and rendering features such as head-tracking, format conversion and configurable room reverb have not been tested.

The following tables provide on a per-bitrate level an indication what has been tested so far (green: tested; red: untested; P: P.Suppl. 800; M: MUSHRA):

Stereo:

|  | 13.2 kbps | 16.4 kbps | 24.4 kbps | 32 kbps | 48 kbps | 64 kbps | 80 kbps | 96 kbps | 128 kbps | 160 kbps | 192 kbps | 256 kbps |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DTX off | P | P | P | P | P, M | P, M |  | M | M |  |  |  |
| DTX on | P | P | P | P | P |  |  |  |  |  |  |  |

SBA:

|  | 13.2 kbps | 16.4 kbps | 24.4 kbps | 32 kbps | 48 kbps | 64 kbps | 80 kbps | 96 kbps | 128 kbps | 160 kbps | 192 kbps | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOA, DTX off |  | P | P | P | P | P | P | P, M | M | M |  |  |  |  |
| FOA, DTX on |  | P | P | P | P | P | P |  |  |  |  |  |  |  |
| HOA2, DTX off |  |  |  |  |  |  |  |  |  | M | M |  |  |  |
| HOA2 DTX on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HOA3, DTX off |  |  |  |  |  |  |  |  |  |  | M | M | M | M |
| HOA3 DTX on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In addition to the operating points above, planar SBA has also not been tested in selection tests.

Multi-channel:

|  | 13.2 kbps | 16.4 kbps | 24.4 kbps | 32 kbps | 48 kbps | 64 kbps | 80 kbps | 96 kbps | 128 kbps | 160 kbps | 192 kbps | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1 |  |  |  |  |  | M |  | M | M | M |  |  |  |  |
| 7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1+2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1+4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1+4 |  |  |  |  |  |  |  |  | M | M |  |  | M | M |

ISM:

|  | 13.2 kbps | 16.4 kbps | 24.4 kbps | 32 kbps | 48 kbps | 64 kbps | 80 kbps | 96 kbps | 128 kbps | 160 kbps | 192 kbps | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISM1, DTX off | P | P | P | P | P | P |  |  |  |  |  |  |  |  |
| ISM1, DTX on | P | P | P |  |  |  |  |  |  |  |  |  |  |  |
| ISM2, DTX off |  | P | P | P | P | P |  |  |  |  |  |  |  |  |
| ISM2 DTX on |  | P | P | P | P |  |  |  |  |  |  |  |  |  |
| ISM3, DTX off |  |  |  |  | M | M |  | M |  |  |  |  |  |  |
| ISM3 DTX on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ISM4, DTX off |  |  |  |  |  |  |  | M | M |  |  | M |  |  |
| ISM4 DTX on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

MASA:

|  | 13.2 kbps | 16.4 kbps | 24.4 kbps | 32 kbps | 48 kbps | 64 kbps | 80 kbps | 96 kbps | 128 kbps | 160 kbps | 192 kbps | 256 kbps | 384 kbps | 512 kbps |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1TC, DTX off |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1TC, DTX on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2TC, DTX off | P | P | P | P | P | P | P | M | M |  | M | M |  |  |
| 2TC, DTX on | P | P | P | P | P | P |  |  |  |  |  |  |  |  |

It is currently unclear to the source whether all operation which were not tested during selection tests could now be covered during characterization testing. Especially in combination with e.g. rendering features this seems unlikely. It is thus proposed to focus on operation points which can’t easily be extrapolated from existing test results.

Testing of BASOP code basis

IVAS Selection tests were carried out on the floating-point code basis. The BASOP port is currently work in progress and expected to be completed by end of March 2024. Apart from objective verification methods, subjective testing of this BASOP code basis is seen as an important part of the IVAS characterization testing.

Subjective Testing of the BASOP code basis can be done in different ways:

- Direct comparison against floating-point code, e.g. by putting identical operating points for floating-point and BASOP code side by side in one test. This allows for per-operation-point comparison of floating-point and BASOP code, but at least doubles the number of conditions per IVAS operation point.

- Indirect comparison against floating-point code, e.g. by comparing against individual floating-point operation modes or reference conditions. This keeps the number of conditions per IVAS operation point small, but has obvious deficiencies for the comparison of floating-point and BASOP code.

Test Capacity

The Test Plan for IVAS Characterization Testing [1] currently assumes the following minimum of available test capacity (external + internal labs):

- 18 x P.SUPPL800

- 30 x BS.1534, up to 12 with LS rendering

- 1 x ACR

- 1 x room acoustics testing

Proposal

It is proposed to put the emphasis of the characterization testing on testing against current service quality and at the same time try to include as many untested operation points by means of IVAS format and bitrate combinations as possible.

In the following, two alternatives for consideration are proposed. They differ in the comparison against stereo operation modes and include the comparison of floating-point and BASOP code in the same test. Whereas in alternative 1 the comparison of BASOP and floating-point code is included directly in the P.800 tests, alternative 2 focuses on an additional comparison of the IVAS immersive formats against IVAS stereo.

Alternative Testing against current service quality and untested operation points, comparison against floating-point in same test

This alternative focuses on the following two points:

- Comparison of IVAS against EVS mono at the same bitrate, to evaluate the difference in QoE between the deployed monaural service and the immersive formats that are provided by IVAS

- Comparison of IVAS floating-point and BASOP code bases at the same bitrate (where applicable)

For this proposal, a combination of testing with P.SUPPL800 and MUSHRA is proposed.

The general assumptions for these tests are:

- The internal IVAS renderer is used, rendering to the playout configuration; no head rotation is to be used for these tests.

- P.SUPPL800 tests are carried out via headphones; for these, the decoder output format is set to binaural

- Given that the BASOP codebase is available and verified,  a comparison of BASOP and floating-point codebases is performed in the tests

- The mono conditions are generated using the IVAS renderer as a preprocessing stage.

- It is proposed to use mixed material for both, P.SUPPL800 and MUSHRA, i.e. a combination of clean-speech, noisy-speech and mixed/music.

Testing of format/bitrate combinations under clean-channel conditions

The proposed P.SUPPL800 listening test layout for IVAS formats under clean-channel conditions is as follows:

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | Stereo | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | MNRU Q=xx dB | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx | - | - |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Table 1: P.SUPPL800 testing of format/bitrate combinations under clean-channel conditions incl. a comparison of BASOP and Floating-Point code

It is proposed to conduct in total 14 P.SUPPL800 clean-channel test of this type for the following formats

- Stereo

- FOA

- Planar FOA

- HOA3

- 5.1

- 7.1

- 5.1+2, 5.1+4 (both formats in same test)

- 7.1+4

- ISM (1-2 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering)

- ISM (3-4 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering)

- MASA 1TC

- MASA 2TC

- OSBA (1-4 objects in same test)

- OMASA (1-4 objects in same test)

To also cover the higher bitrate/quality range, it is proposed to complement these P.SUPPL800 tests by 14 MUSHRA tests at higher bitrates (clean-channel, same formats as above):

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | LP 3.5 kHz | - | - |
| c04 | EVS | 1x128.0 | Off |
| c05 | IVAS FL | 128.0 | Off |
| c06 | IVAS FL | 256.0 | Off |
| c07 | IVAS FX | 128.0 | Off |
| c08 | IVAS FX | 256.0 | Off |

Table 2: Additional MUSHRA tests for a per format/bitrate comparison of BASOP and floating-point code

Testing of format/bitrate combinations under impaired-channel conditions

In addition, it is proposed to also include tests for impaired channel conditions:

| Label | Condition | Bitrate [kbps] | DTX | FER/Profile |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | Mono | - | - |  |
| c03 | Stereo | - | - |  |
| c04 | MNRU Q=xx dB | - | - |  |
| c05 | MNRU Q=xx dB | - | - |  |
| c06 | MNRU Q=xx dB | - | - |  |
| c07 | ESDRU α=xx | - | - |  |
| c08 | ESDRU α=xx | - | - |  |
| c09 | ESDRU α=xx | - | - |  |
| c10 | EVS | 1x13.2 |  | 0% FER |
| c11 | EVS | 1x24.4 |  | 0% FER |
| c12 | EVS | 1x48.0 |  | 0% FER |
| c13 | EVS | 1x64.0 |  | 0% FER |
| c14 | EVS | 1x128.0 |  | 0% FER |
| c15 | EVS | 1x13.2 |  | 5% FER |
| c16 | EVS | 1x24.4 |  | 5% FER |
| c17 | EVS | 1x48.0 |  | 5% FER |
| c18 | EVS | 1x64.0 |  | 5% FER |
| c19 | EVS | 1x128.0 |  | 5% FER |
| c20 | EVS | 1x24.4 |  | DlyErr Profile XX |
| c21 | EVS | 1x64.0 |  | DlyErr Profile XX |
| c22 | IVAS FL | 24.4 |  | 0% FER |
| c23 | IVAS FL | 24.4 |  | 5% FER |
| c24 | IVAS FL | 24.4 |  | DlyErr Profile XX |
| c25 | IVAS FX | 1x13.2 |  | 0% FER |
| c26 | IVAS FX | 1x24.4 |  | 0% FER |
| c27 | IVAS FX | 1x48.0 |  | 0% FER |
| c28 | IVAS FX | 1x64.0 |  | 0% FER |
| c29 | IVAS FX | 1x128.0 |  | 0% FER |
| c30 | IVAS FX | 1x13.2 |  | 5% FER |
| c31 | IVAS FX | 1x24.4 |  | 5% FER |
| c32 | IVAS FX | 1x48.0 |  | 5% FER |
| c33 | IVAS FX | 1x64.0 |  | 5% FER |
| c34 | IVAS FX | 1x128.0 |  | 5% FER |
| c35 | IVAS FX | 1x24.4 |  | DlyErr Profile XX |
| c36 | IVAS FX | 1x64.0 |  | DlyErr Profile XX |

Table 3: P.SUPPL800 testing of format/bitrate combinations under impaired channel conditions incl. a limited BASOP against floating-point comparison

It is proposed to conduct in total 4 P.SUPPL800 impaired-channel test of this type for the following formats

- Stereo

- Multi-channel 5.1

- OMASA

- OSBA

The proposed formats can be seen as a super-set of all supported IVAS formats.

Summary

The testing scheme as proposed above would consume 18/18 P.SUPPL800 tests and 14/(estimated) 30 MUSHRA tests. The remaining MUSHRA tests could be used for testing other untested features such as EVS compatible mono downmix, rendering aspects, etc.

Alternative 2: Testing against current service quality and untested operation points, comparison against floating-point in separate test

This alternative focuses on the following two points:

- As for alternative 1, comparison of IVAS against EVS mono at the same bitrate, to evaluate the difference in QoE between the deployed monaural service and the immersive formats that are provided by IVAS

- Comparison of IVAS immersive against IVAS stereo operating points at the same bitrate. This evaluates the difference in QoE between stereo and immersive services, although important features such as head-tracked scene manipulation (which is really only possible with the immersive formats) are left out in this comparison, since its effects on the test methodology (especially for P.SUPPL800) are unknown or the reproduction system has limitations (such as non-headtracked stereo headphones).

- A limited comparison of IVAS floating-point and BASOP code bases at the same bitrate (where applicable)

For this proposal, a combination of testing with P.SUPPL800 and MUSHRA is proposed.

The general assumptions for this tests are:

- The internal IVAS renderer is used rendering to the playout configuration; no head rotation is to be used for these tests.

- P.SUPPL800 tests are carried out via headphones; for these, the decoder output format is set to binaural

- Given that the BASOP codebase is available and verified, the BASOP codebase is used for the tests; otherwise, floating-point code basis is used

- Mono and Stereo conditions are generated using the IVAS renderer as a preprocessing stage (i.e. before coding at either mono (1x EVS) or stereo (IVAS Stereo). Preprocessing to stereo doesn’t include binauralization.

- It is proposed to use mixed material for both, P.SUPPL800 and MUSHRA, i.e. a combination of clean-speech, noisy-speech and mixed/music.

Testing of format/bitrate combinations under clean-channel conditions

The proposed P.SUPPL800 listening test layout for IVAS immersive formats under clean-channel conditions is as follows:

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | Stereo | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | MNRU Q=xx dB | - | - |
| c07 | MNRU Q=xx dB | - | - |
| c08 | ESDRU α=xx | - | - |
| c09 | ESDRU α=xx | - | - |
| c10 | ESDRU α=xx | - | - |
| c11 | EVS | 1x13.2 |  |
| c12 | EVS | 1x16.4 |  |
| c13 | EVS | 1x24.4 |  |
| c14 | EVS | 1x32.0 |  |
| c15 | EVS | 1x48.0 |  |
| c16 | EVS | 1x64.0 |  |
| c17 | EVS | 1x96.0 |  |
| c18 | EVS | 1x128.0 |  |
| c19 | IVAS Stereo | 13.2 |  |
| c20 | IVAS Stereo | 16.4 |  |
| c21 | IVAS Stereo | 24.4 |  |
| c22 | IVAS Stereo | 32.0 |  |
| c23 | IVAS Stereo | 48.0 |  |
| c24 | IVAS Stereo | 64.0 |  |
| c25 | IVAS Stereo | 80.0 |  |
| c26 | IVAS Stereo | 96.0 |  |
| c27 | IVAS Stereo | 128.0 |  |
| c28 | IVAS Immersive | 13.2 |  |
| c29 | IVAS Immersive | 16.4 |  |
| c30 | IVAS Immersive | 24.4 |  |
| c31 | IVAS Immersive | 32.0 |  |
| c32 | IVAS Immersive | 48.0 |  |
| c33 | IVAS Immersive | 64.0 |  |
| c34 | IVAS Immersive | 80.0 |  |
| c35 | IVAS Immersive | 96.0 |  |
| c36 | IVAS Immersive | 128.0 |  |

Table 4: P.SUPPL800 testing of format/bitrate combinations under clean-channel conditions incl. a comparison to IVAS stereo operation points

It is proposed to conduct in total 13 P.SUPPL800 clean-channel test of this type for the following formats

- FOA

- Planar FOA

- HOA3

- 5.1

- 7.1

- 5.1+2, 5.1+4 (both formats in same test)

- 7.1+4

- ISM (1-2 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering)

- ISM (3-4 objects in same test; ISM incl. metadata/rendering, EVS w/o metadata/rendering)

- MASA 1TC

- MASA 2TC

- OSBA (1-4 objects in same test)

- OMASA (1-4 objects in same test)

To also cover the higher bitrate/quality range, it is proposed to complement these P.SUPPL800 tests by 13 MUSHRA tests at higher bitrates (clean-channel, same formats as above):

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | Mono | - | - |
| c03 | LP 3.5 kHz | - | - |
| c04 | EVS | 1x128.0 | Off |
| c05 | IVAS Stereo | 128.0 | Off |
| c06 | IVAS Stereo | 256.0 | Off |
| c07 | IVAS Immersive | 128.0 | Off |
| c08 | IVAS Immersive | 256.0 | Off |

Table 5: Additional MUSHRA tests for a per format/bitrate comparison IVAS immersive and stereo formats

Testing of format/bitrate combinations under impaired-channel conditions

In addition, it is proposed to also include tests for impaired channel conditions:

| Label | Condition | Bitrate [kbps] | DTX | FER/Profile |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | Mono | - | - |  |
| c03 | Stereo | - | - |  |
| c04 | MNRU Q=xx dB | - | - |  |
| c05 | MNRU Q=xx dB | - | - |  |
| c06 | MNRU Q=xx dB | - | - |  |
| c07 | ESDRU α=xx | - | - |  |
| c08 | ESDRU α=xx | - | - |  |
| c09 | ESDRU α=xx | - | - |  |
| c10 | EVS | 1x13.2 |  | 0% FER |
| c11 | EVS | 1x24.4 |  | 0% FER |
| c12 | EVS | 1x48.0 |  | 0% FER |
| c13 | EVS | 1x64.0 |  | 0% FER |
| c14 | EVS | 1x128.0 |  | 0% FER |
| c15 | EVS | 1x13.2 |  | 5% FER |
| c16 | EVS | 1x24.4 |  | 5% FER |
| c17 | EVS | 1x48.0 |  | 5% FER |
| c18 | EVS | 1x64.0 |  | 5% FER |
| c19 | EVS | 1x128.0 |  | 5% FER |
| c20 | EVS | 1x24.4 |  | DlyErr Profile XX |
| c21 | EVS | 1x64.0 |  | DlyErr Profile XX |
| c22 | IVAS Stereo | 24.4 |  | 0% FER |
| c23 | IVAS Stereo | 24.4 |  | 5% FER |
| c24 | IVAS Stereo | 24.4 |  | DlyErr Profile XX |
| c25 | IVAS Immersive | 1x13.2 |  | 0% FER |
| c26 | IVAS Immersive | 1x24.4 |  | 0% FER |
| c27 | IVAS Immersive | 1x48.0 |  | 0% FER |
| c28 | IVAS Immersive | 1x64.0 |  | 0% FER |
| c29 | IVAS Immersive | 1x128.0 |  | 0% FER |
| c30 | IVAS Immersive | 1x13.2 |  | 5% FER |
| c31 | IVAS Immersive | 1x24.4 |  | 5% FER |
| c32 | IVAS Immersive | 1x48.0 |  | 5% FER |
| c33 | IVAS Immersive | 1x64.0 |  | 5% FER |
| c34 | IVAS Immersive | 1x128.0 |  | 5% FER |
| c35 | IVAS Immersive | 1x24.4 |  | DlyErr Profile XX |
| c36 | IVAS Immersive | 1x64.0 |  | DlyErr Profile XX |

Table 6: P.SUPPL800 testing of format/bitrate combinations under impaired channel conditions incl. a limited comparison to IVAS stereo operating points

It is proposed to conduct in total 3 P.SUPPL800 impaired-channel test of this type for the following formats

- Multi-channel 5.1

- OMASA

- OSBA

The proposed formats can be seen as a super-set of all supported IVAS formats.

Testing of BASOP against floating-point code

For the testing of BASOP against floating point code, the testing scheme of table 1 could be applied. Since only two P.SUPPL800 would be left, it is proposed to conduct these tests as multi-format tests, e.g.

- Test 1:

  - Stereo

  - FOA

  - HOA3

  - 5.1

  - 7.1

  - 7.1+4

- Test 2:

  - ISM (1-2 objects)

  - ISM (3-4 objects)

  - MASA 1TC

  - MASA 2TC

  - OSBA (1-4 objects)

  - OMASA (1-4 objects)

Summary

The testing scheme as proposed above would consume 18/18 P.SUPPL800 tests and 13/(estimated) 30 MUSHRA tests. The remaining MUSHRA tests could be used for testing other untested features such as EVS compatible mono downmix, rendering aspects, etc. Additional testing might be carried out by means of MUSHRA tests.

Discussion

This document proposes two alternatives for the IVAS characterization testing:

- In alternative 1, a pairwise testing of IVAS floating-point and BASOP code against EVS mono at the same bitrate is proposed. This allows for an evaluation of the difference in service quality between EVS mono and IVAS via a rate-distortion curve and at the same time an evaluation of the floating-point and BASOP code. Additional testing is included for PLC and JBM conditions.

- In alternative 2, testing of IVAS immersive and stereo operating points against EVS mono at the same bitrate is proposed. This allows for an evaluation of the difference in service quality between EVS mono and IVAS in two ways: First against the complete IVAS immersive format, second against IVAS stereo. However, it has to be seen that this comparison has its limitations, since important additional advantages of the immersive formats, such as scene manipulation via head-tracking,  are not considered in the tests. A limited testing of floating-point and BASOP code as well as PLC and JBM conditions is included.

Alternative 1 seems to be a pragmatic approach to achieve first a rate distortion comparison of EVS and IVAS and second the comparison of floating-point and BASOP code in the same test. Alternative 2 shifts the focus of the evaluation of EVS versus IVAS stereo and immersive formats, and limits the amount of tests for the comparison of floating-point and BASOP code.

References:

[1] S4-232029 - IVAS-8b: Test Plan for Characterization Phase, v0.1.0

[2] 3GPP TS 26.114 version 18.4.0: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction"