3GPP TSG-SA WG4 Meeting #SA4-e (AH) Audio SWG post 128	SA4aA240036

# Online, 07th June 2024

Source:	HEAD acoustics GmbH

Title:	Directional information requirements for stereo

Agenda item:	1.4

Document for:	Agreement

# Introduction

The clause regarding stereo capture requirements in the proposed changes to TS 26.261 v0.2.0 in SA4aA240033 is not mature and contains mistakes and imprecise wording. The present document proposes an updated version of the respective clauses.

# Proposal for modification

Proposed update to TR 26.261, clauses 3.1, 3.2, and 5.5.1:

## 3.1 Terms

[…]

Stereo panorama: the spatial image of a stereo signal, in which the sound source directions lie in a range from -100% (left) to 100% (right).

## 3.2 Symbols

dB	Decibel

Hz	Unit of frequency (Hertz)

ϕ	Azimuth angle (phi)

θ	Elevation angle (theta)

Pϕi

TR	UE delay in receiving direction

TS	UE delay in sending direction

ζϕ	Estimate for a single source direction in the stereo panorama that is physically positioned in direction ϕ relative to the capturing device

## 5.5.1 Stereo

The estimated source directions in the stereo panorama ζϕiPϕi shall be monotonically increasing between -60° and +60°, i.e., ζϕi-ζϕi-1≥10% for i = 3,…,6, where i is the source direction index according to Table 5 in TS 26.260Pϕi-Pϕi-1≥10%. This requirement verifies that the estimated stereo panorama Pϕiis consistent for the relevant source positions ϕiϕi.

The estimated source directions in the stereo panoramaPϕi shall provide a minimum and symmetric width between ϕ2 and ϕ6, i.e., ζϕ2 and ζ(ϕ6)ϕ2ϕ6 Pϕ2P(ϕ6) shall be larger than or equal to 60%.

The Pϕiestimated source directions in the stereo panorama shall be consistent at the edges, i.e., ζϕ1≤ ζϕ2 and ζϕ7≥ζϕ6.Pϕ1≤ Pϕ2Pϕ7≥ Pϕ6

For frontal incidence position (ϕ4=0°), the estimated sound source direction shall be in the center of the stereo panorama, i.e., Pα6<3%1…5 %.

For all L = 7 source directions of Table 5 in TS 26.260, the absolute value of ICTD shall be less than 1.5 ms.

Compliance shall be checked by the relevant tests described in TS 26.260.

# Conclusion

The source proposes to include the above text in clauses 3.1, 3.2, and 5.5.1 for draft TS 26.261 v0.3.0.