Source:		Qualcomm Inc.

Title:	Real-time metadata transport over RTP

Agenda Item:	4.6

Document for:	Discussion and Agreement

# Introduction

At the 3GPP-SA4#120-e, the Permanent Document for MeCAR v3.0.0 [1] was approved. The Permanent Document specifies AR/MR media type definition and real-time characteristics. The 5G_RTP Permanent Document v0.0.2 [2] also specifies the requirements tentatively, including reliability, traffic and direction of AR media types.

Based on the metadata definition and the description specified in MeCAR PD, the AR/MR metadata may be categorized into 3 class.

- Device class: the metadata specifying the device capabilities and features such as FOV, camera information and projection information. The information is usually available at the beginning of an AR/MR session and may not change within an AR/MR session.

- Content class: the metadata describing the space or media object content such as scene description, spatial description, and 3D visual mode. The data size may be large (>10MB) and the information usually does not change frequently and may be event driven.

- Interaction class: the metadata representing the user interaction such as user pose, viewport, gesture, body action, facial expression and AR anchor point. The interaction may trigger the event at the receiver side and the sender may expect low-latency response (50~1000ms). The interaction data size may be small, but the frequency could be high (>1KHz). The data transport may be continuously or may require burst transport. Depending on the applications, the interactive media types may be synchronized to each other, or synchronized to other media streams, and the reliability may be strict or not strict.

Table 1 is the metadata categories and the characteristics

Table 1 Metadata categories

|  | Device class | Content class | Interaction class |
| --- | --- | --- | --- |
| metadata | FOV, camera information, projection information | Scene description, spatial description, 3D visual mode | AR anchor, user pose, viewport, gesture, body action, facial expression |
| Data size | Small | Large (>10MB) | Small |
| Frequency | Beginning of the session | Low (event based) | High (>1KHz) |
| Real-time | Not timed | Timed/Not timed | Timed |
| Latency |  |  | 50~1000ms |
| Reliability | reliable | reliable | reliable/unreliable |
| Time Sync. | N | Y | Y |

# Metadata transport

The device class metadata is usually static and available in advance. The metadata may be exchanged using SDP [3] framework for immersive real-time communication or IMS-based AR conversation stream respectively.

The content class metadata may be transported over RTP [4] given its size and characteristics. The RTP payload type may indicate the specific media type such as scene description, spatial description, or 3D visual model. The assignment of an RTP payload type may be performed in a dynamic way and indicated in SDP media description lines. The RTP payload format may be refer to the corresponding IETF RFCs.

The real-time interaction metadata may be transported over RTP or WebRTC data channel, this document proposes the candidate solution for the interaction metadata transport over RTP.

# Real-time interaction metadata transport over RTP

For the applications that the media streams are exchanged using RTP, the media types may be carried in RTP [4] or RTP extensions [5] given the data size and low-latency requirements. A generic payload format is preferred to carry the real-time interaction metadata.

## RTP and RTP extension

RTP fixed header is specified in [4] and the format is shown in Figure 1.

Figure 1 RTP fixed header format

When the extension bit (X) is set, a variable-length header extension must be appended to the RTP header, following the CSRC list if present. Figure 2 shows the general extension format. The RTP header extension may carry metadata in addition to the usual RTP header information as an optimization to lower latency.

Figure 2 RTP header extension

Two types of extension designs are specified in [5], one-byte header and two-byte header form of extension.

In the one-byte header form of extension, the 16-bit “defined by profile” must have the fixed bit pattern 0xBEDE. Each extension element MUST start with a byte containing an ID and a length. The 4-bit ID is the local identifier of this element in the range 1-14 inclusive. The 4-bit length is the number, minus one, of data bytes of this header extension element following the one-byte header. Figure 3 is an example of one-byte header extension.

Figure 3 One-byte header extension example

In the two-byte header form of extension, the 16-bit “defined by profile” has 12-bit 0x100 and 4-bit appbits. The appbits field is 4 bits that are application dependent and may be defined to be any value or meaning. Each extension element starts with a byte containing an ID and a byte containing a length. The 8-bit length field is the length of extension data in bytes, not including the ID and length fields. The value zero (0) indicates that there is no subsequent data. Figure 4 is an example of two-byte header extension.

Figure 4 Two-byte header extension example

## OpenXR

OpenXR is an API for XR applications. OpenXR is the interface between an application and an in-process or out-of-process "XR runtime system", or just "runtime" hereafter. The runtime may handle such functionality as frame composition, peripheral management, and raw tracking information. Figure 5 illustrates the OpenXR API for the XR applications and XR devices.

Figure 5 OpenXR API

OpenXR defines API extensions to supports various input actions and haptics, such as hand tracking (Figure 6), palm pose (Figure 7), keyboard tracking, facial tracking (Figure 8), and controllers for various devices.

Figure 6 OpenXR XR_EXT_hand_tracking

Figure 7 OpenXR XR_EXT_Palm_pose

Figure 8 OpenXR XR_HTC_facial_tracking

It would be beneficial to have a generic RTP design for the metadata transport to accommodate the existing metadata structure defined in the specifications like OpenXR.

## RTP header extension for interactive metadata

The RTP header extension may be used to carry metadata while the media content is carried in the RTP payload data. Depending one the data length, either one-byte or two-byte header extension may be used; a single metadata type or multiple metadata types may be carried in the extension in a RTP packet.

Figure 9 illustrates an RTP header extension design concept to carry the real-time interactive metadata.

Payload protocol ID is a fixed length field indicating the sub-protocol or specifications used for the metadata format, such as OpenXR. The field may not be necessary if OpenXR is the only default format to be supported.

Metadata type is a fixed length field indicating the metadata type specified in the sub-protocol.

Metadata attributes is a fixed length field indicating the metadata attributes such as time synchronization and reliability.

Metadata length is a fixed length field indicating the length of metadata payload in bytes.

Figure 9 RTP header extension for the metadata

# Proposal

It is requested to include clause 3 of this document in the permanent document and take them into account in the discussion of 5G_RTP and related topics.

# References

- 3GPP TSG SA WG4 S4-221150, “MeCAR Permanent Document v3.0”, August 2022

- 3GPP TSG SA WG4 S4-221209, “5G_RTP Permanent Document v0.0.2”, August 2022

- IETF RFC4566, “SDP: Session Description Protocol”, July 2008

- IETF RFC3550, "RTP: A Transport Protocol for Real-Time Applications", July 2003

- IETF RFC8285, “A General Mechanism for RTP Header Extensions”, Oct. 2017