Source:		Qualcomm Inc.

Title:	Real-time metadata transport over data channel

Agenda Item:	4.3

Document for:	Discussion and Agreement

# Introduction

At the 3GPP-SA4#120-e, the Permanent Document for MeCAR v3.0.0 [1] was approved. The Permanent Document specifies AR/MR media type definition and real-time characteristics. The 5G_RTP Permanent Document v0.0.2 [2] also specifies the requirements tentatively, including reliability, traffic and direction of AR media types.

Based on the metadata definition and the description specified in MeCAR PD, the AR/MR metadata may be categorized into 3 class.

- Device class: the metadata specifying the device capabilities and features such as FOV, camera information and projection information. The information is usually available at the beginning of an AR/MR session and may not change within an AR/MR session.

- Content class: the metadata describing the space or media object content such as scene description, spatial description, and 3D visual mode to be consumed by the user. The data size may be large (>10MB) and the information usually does not change frequently and may be event driven.

- Interaction class: the metadata representing the user input and haptics such as user pose, viewport, gesture, body action, facial expression and AR anchor point. The interaction may trigger the event at the receiver side and the sender may expect low-latency response (50~1000ms). The interaction data size may be small, but the frequency could be high (>1KHz). The data transport may be continuously or may require burst transport. Depending on the applications, the interactive media types may be synchronized to each other, or synchronized to other media streams, and the reliability may be strict or not strict.

Table 1 is the metadata categories and the characteristics

Table 1 Metadata categories

|  | Device class | Content class | Interaction class |
| --- | --- | --- | --- |
| metadata | FOV, camera information, projection information | Scene description, spatial description, 3D visual mode | AR anchor, user pose, viewport, gesture, body action, facial expression |
| Data size | Small | Large (>10MB) | Small |
| Frequency | Beginning of the session | Low (event based) | High (>1KHz) |
| Real-time | Not timed | Timed/Not timed | Timed |
| Latency |  |  | 50~1000ms |
| Reliability | reliable | reliable | reliable/unreliable |
| Time Sync. | N | Y | Y |

# Metadata transport

The device class metadata is usually static and available in advance. The metadata may be exchanged using SDP [3] framework for immersive real-time communication or IMS-based AR conversation stream respectively.

The content class metadata may be transported over RTP [4] given its size and characteristics. The RTP payload type may indicate the specific media type such as scene description, spatial description, or 3D visual model. The assignment of an RTP payload type may be performed in a dynamic way and indicated in SDP media description lines. The RTP payload format may be refer to the corresponding IETF RFCs.

The real-time interaction metadata may be transported over RTP or WebRTC data channel, this document proposes the candidate solution for the interaction metadata transport over WebRTC data channel.

# Real-time interaction metadata transport over data channel

For the streaming applications like gaming, the users receive the RTP packets from the server but may not send RTP packet to the server. The interaction metadata may be carried in WebRTC data channel [4] given the data size and low-latency requirements. A generic payload format is preferred to carry the real-time interaction metadata.

## WebRTC data channel

In the WebRTC framework, communication between the parties consists of media (for example, audio and video) and non-media data. Media is sent using the SRTP, and non-media data is handled by using the Stream Control Transmission Protocol (SCTP) [5] as shown in Figure 1.

Figure 1 WebRTC protocol stack

The priority associated with a media flow or data flow is classified as “very-low”, “low”, “medium”, or “high” in the API. WebRTC implementations may attempt to set QoS on the packet sent according to the guidelines in [6].

The SCTP provides the following features for transporting non-media data between browsers:

•	Support of multiple unidirectional streams

•	Ordered and unordered delivery of user messages

•	Reliable and partial reliable transport of user message

An SCTP packet is composed of a common header and chunks. A chunk contains either control information or user data. The SCTP packet format is shown below:

Figure 2 SCTP packet format

The common header field includes a source port number, a destination port number, a verification tag and checksum, as shown in Figure 3.

Figure 3 SCTP common header format

Each chunk is formatted with a chunk type field, a chunk-specific Flag field, a Chunk Length field, and a value field as shown below.

Figure 4 SCTP chunk field format

SCTP defines a number of chunk types, and one of the types is Payload Data (DATA) when chunk type is 0. The payload data format is shown in Figure 5. The U bit indicates unordered data chunk; B and E bit indicate the fragmented user message; the payload protocol identifier represents an application specific protocol identifier and is passed to SCTP by its upper layer and sent to its peer.

Figure 5 SCTP chunk payload data type format

## OpenXR

OpenXR is an API for XR applications. OpenXR is the interface between an application and an in-process or out-of-process "XR runtime system", or just "runtime" hereafter. The runtime may handle such functionality as frame composition, peripheral management, and raw tracking information. Figure 5 illustrates the OpenXR API for the XR applications and XR devices.

Figure 5 OpenXR API

OpenXR defines API extensions to supports various input actions and haptics, such as hand tracking (Figure 6), palm pose (Figure 7), keyboard tracking, facial tracking (Figure 8), and controllers for various devices.

Figure 6 OpenXR XR_EXT_hand_tracking

Figure 7 OpenXR XR_EXT_Palm_pose

Figure 8 OpenXR XR_HTC_facial_tracking

It would be beneficial to have a generic WebRTC data channel payload design for the metadata transport to accommodate the existing metadata structure defined in the specifications like OpenXR.

## Data channel for interactive metadata

The WebRTC data channel may be used to convey real-time interactive metadata.

It is proposed to use Payload Data chunk to carry the metadata, the candidate format is show below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    Type = 0   | Reserved|U|B|E|           Length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             TSN                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Stream Identifier S      |    Stream Sequence Number n   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 Payload Protocol Identifier                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        metadata type          |    metadata attributes        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

\                                                               \

/              Metadata Payload Data (seq n of Stream S)        /

\                                                               \

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The payload protocol identifier can be used to indicate the protocol such as OpenXR.

16-bits metadata type field may indicate the metadata type specified in the payload protocol.

16-bit metadata attributes field may indicate the metadata attributes such as time synchronization and reliability.

SCTP does not carry timestamp information. OpenXR defines a 64-bit signed integer representing nanoseconds (XrTime) and associates it to the APIs. For the payload protocol that does not define timestamp, a timestamp field may be added to the chunk user data section as below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    Type = 0   | Reserved|U|B|E|           Length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             TSN                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Stream Identifier S      |    Stream Sequence Number n   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 Payload Protocol Identifier                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           timestamp                           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        metadata type          |    metadata attributes        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

/              Metadata Payload Data (seq n of Stream S)        /

\                                                               \

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

# Proposal

It is requested to include clause 3 of this document in the permanent document and take them into account in the discussion of IRTCW and related topics.

# References

- 3GPP TSG SA WG4 S4-221150, “MeCAR Permanent Document v3.0”, August 2022

- 3GPP TSG SA WG4 S4-221209, “5G_RTP Permanent Document v0.0.2”, August 2022

- IETF RFC4566, “SDP: Session Description Protocol”, July 2008

- IETF RFC8831, "WebRTC Data Channels", January 2021

- IETF RFC4960, “Stream Control Transmission Protocol”, Sept. 2007

- IETF RFC8837, “Differentiated Services Code Point (DSCP) Packet Markings for WebRTC QoS"