Agenda item:

Source: 	Qualcomm Inc.

Title: 	[iRTCW] Signaling Protocol for iRTCW

Document for	Discussion and Agreement

# Introduction

In the previous meeting, we have agreed a set of requirements for the WebRTC signaling protocol. These requirements are listed here:

- It shall support any WebRTC application, i.e. it should not overfit for a specific use case.

- It shall enable communicating parties to match based on

- Flexible matching with a wide range of matching criteria that suit the needs of different WebRTC applications.

- Secure matching to avoid security issues such as DDoS attacks

- It may be extended in the future to support

- Global matching possibilities for applications that work across multiple WebRTC signaling servers, potentially hosted by different MNOs.

- It shall enable communicating parties to verify each other’s identity, if required by the application.

- It shall support the secure exchange of messages supporting integrity-protection and/or encryption.

- It shall protect user privacy and mitigate the linkability and tracking attack caused by unnecessary user information disclosure. 
Note: detailed security requirements and mechanisms need further co-work with SA3.

- It shall support basic session setup messages allowing extensions for application-specific information.

- It should be web-friendly to support easy deployment in web environments

- by using web technologies such as JSON, WebSockets, etc…

- complying with WebRTC standards (e.g., SDP for session description and supporting the exchange of ICE candidates, etc…) defined in IETF and W3C, with an exception for codecs

- It shall be simple to implement and deploy (e.g. simpler in complexity compared to SIP).

In this contribution, we propose the format for the WebRTC signaling protocol and identify a set of messages that the protocol is expected to offer.

# WebRTC Signaling Protocol

The WebRTC signaling protocol shall operate over a full-duplex WebSocket session. We propose to name the protocol Simple/Secure WebRTC Signaling Protocol (SWAP).

The WebRTC signaling protocol shall use the “swap.3gpp” subprotocol identifier. The WebRTC Signaling protocol shall use the ‘text’ format, i.e. text frames. The messages shall be encoded as UTF-8 encoded JSON messages.

The following messages shall be supported:

| Message Type | Parameters | Description |
| --- | --- | --- |
| register | - message id - matching criteria that can be used to match endpoints when establishing a connection. | the endpoint registers with the WebRTC signaling server and provides the matching criteria that may be used to match this end-point for the connection. This message may not be required, if the end-point is providing the WebRTC signaling server functionality. |
| response (ack/error) | - message id - type: (ack/response) - error code - error description | A Response message is generated by the WebRTC signaling server and sent to the end-point. It may be an acknowledgement or an error message. The ack message is sent by the WebRTC signaling server to confirm the reception of a message. An error message may be sent by the WebRTC signaling server to signal an error to the endpoint. The message id refers to the message that caused the error. |
| connect | - message id - matching criteria - offer SDP - ICE candidates | An endpoint sends a connect message to establish a connection to another endpoint. It provides the matching criteria to allow the WebRTC signaling server to locate and match the remote endpoint. The message may also carry ICE candidates, which may be used for trickle ICE negotiation. |
| accept | - message id - connection id - answer SDP - ICE candidates | This message is sent by the receiving endpoint to indicate that it accepts the offer. It includes the remote SDP as well as other information such as the ICE candidates. |
| update | - message id - connection id - updated offer SDP - ICE candidates | This message is used to send an update to the session, e.g. sharing additional ICE candidates or adding/modifying/removing media streams. |
| reject | - message id | A rejection message is sent by the endpoint to indicate that it rejects an initial offer or an update. |
| application | - message id - connection id - application-specific information | Application messages are used to exchange application-specific messages. The content of the message is defined by the application. |
| close | - message id - connection id | The close message is sent by an endpoint or by the WebRTC signaling server to close a WebRTC connection. |

To address the needs of different applications, the following matching criteria shall be supported:

- IPv4/IPv6 address

- FQDN

- service or application identifier

- user identifier such as an SIP address, a GPSI, or an MSISDN

- EAS identifier

- application-specific matching criteria that is compared using binary or string comparison

The previous matching criteria may be combined together or with a set of requirements, such as latency, load, geographic location, etc.

Integrity and confidentiality protection are supported through the protection of the message information as follows:

- a key derivation mechanism is configured by the application provider to the session participants, e.g. using a shared secret algorithm

- For integrity protection, the derived key is used to provide integrity protection, e.g. using a Message Authentication Code (MAC) for message payload.

- For encryption, the derived key is used to encrypt the message payload. The encrypted data may then be encoded using base64 to enable embedding it in JSON.

These mechanisms are possible to implement using the WebCrypto API, which makes them web-friendly. Consulting with SA3 on these security algorithms is recommended.

# Proposal

We propose to agree the protocol proposal as a baseline and continue working on fulfilling all the requirements of the protocol.