3GPP TSG-S4  ad hoc Meeting #Post 121-e	S4aR230017

Online, 30th November 2022 –10th February 2023

# Title:	(draft) Reply LS on PDU Set Handling

# Response to:	(R2-2213351) LS on PDU Set Handling

# Release:	Release 17

# Work Item:	5G_RTP

Source:	SA4

To:	RAN2

Cc:	SA2

Contact Person:

Name:	Thorsten Lohmar

E-mail Address:	Thorsten.lohmar@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA4 would like to thank RAN2 for the LS on PDU Set Handling. SA4 has discussed the Liaison Statement and would like to give some immediate feedback on the following topic:

Feedback: An RTP/UDP receiver contains a chain of functions for packet and frame processing. One of the first functions is a de-jitter buffer function, which collects and keeps all received PDUs (RTP/UDP packets) for a specific duration. When RTP packets (PDUs) are received out of sequence (according to the RTP sequence number), this buffer function can re-arrange the RTP Packets to provide the RTP packets in-sequence (according to the sequence numbers within the RTP headers) to the next function in the chain. The duration of the de-jitter buffer is usecase specific and set by the application. For low latency XR applications, the duration for buffering should be small.

SA4 has discussed, whether the PDU Set Delay Budget (PSDB) can be used to define the maximal duration of out of sequence delivery of PDUs.

When the 5G System starts assigning different priorities or different importance’s to PDU Sets in the stream, SA4 wonders whether this handling leads to out-of-sequence delivery of PDU Sets. .

2. Actions:

To RAN2 group.

ACTION: SA4 kindly asks RAN2 to take above information into account. SA4 kindly asks RAN2 to clarify the usage of the PDU Set Delay Budget (PSDB) in context of out-of-sequence delivery of PDUs. Further, SA4 kindly ask RAN2 and SA2 whether the PSDB is respected, when handling PDU Sets with different priority.

3. Date of Next SA4 Meetings:

3GPP TSG SA4#122		20th to 24th February 2023	Athens, Gr