Agenda item: 	4.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP] RTP Header Extension for PDU SET Marking

Document for	Discussion and Agreement

# Introduction

In a previous contribution, we proposed the definition of an RTP header extension to carry PDU Set identification information.

In this contribution, we just provide information to the group about the agreed CR towards TS23.501 for the introduction of the PDU Set identification information that is provided by the UPF to the gNB through GTP-U.

# Agreed PDU Set Information in TS23.501

The following information is copied from the agreed CR in S2-2301379. The information is provided as a guide for our stage 3 work. Different media applications that want to benefit from the PDU Set concept need to ensure this information is provided by the application. In the case of RTP/SRTP, the proposal is to provide this information through an RTP header extension.

-----------------------------   Begin of Copied Text   -----------------------------

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set

-	PDU Sequence Number within a PDU Set

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the importance of a PDU Set within a QoS Flow.

The NG-RAN may use the PDU Set Importance for PDU Set level packet discarding in presence of congestion.

NOTE 1: 	The PDU Set Size is pending SA WG4 progress on SA WG4 5G_RTP WI. It is up to an application to decide whether to send PDU Set Size in bytes or not.

NOTE 2: 	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

The SMF instructs UPF to perform PDU Set marking and may provide UPF the Protocol Description indicating the header (e.g., RTP/SRTP) and payload type (e.g. H.264) used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.x.1.  Alternatively, PSA UPF implementation specific procedures can be used for identifying PDU Sets.

For each DL PDU received on N6 for which PDU Set handling should be performed based on the instruction from SMF, the PSA UPF applies the rules for PDU Set identification and provides PDU Set Information which is available to the RAN in the GTP-U header.

-----------------------------   End of Copied Text   ----------------------------

# Proposal

We propose to take this information into account when defining the RTP header extension.