Agenda item: 	4.6

Source: 	InterDigital Communications

Title: 	[5G_RTP] End-of-burst signaling in PDU Set RTP HE

Document for:	Discussion and Agreement

# Introduction

In S4aR230057 the RTC working group has agreed to work on signaling the end-of-burst indication in PDU set information in an RTP header extension.

In this contribution, we discuss the syntax and semantics of end-of-burst signaling in RTP HE and the guidelines on how to set this field in the RTP HE at the application level.

# Data burst

S2-2303879 defines a data burst “A set of multiple PDUs generated and sent by the application in a short period of time”. The definition also includes a note that states “A Data Burst can be composed of one or multiple PDU Sets”.

The concept of a Data Burst is relevant when discussing traffic that is associated with extended reality (XR) services. It is important because XR traffic is often transmitted in bursts. For example, an Application Server (AS) may send a burst of data to a UE. The burst of data may represent audio information, video information, etc. After transmitting the burst of the data to the UE, a period of time may elapse before the Application Server transmits another data burst to the UE.

In S2-2303845, SA2 has agreed that:

- “An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g., to configure UE power management schemes like connected mode DRX.” And

- “According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the Protocol Description and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst.”.

The SMF configuration is based on a protocol description within the PCC Rules. S2-2303845 explains that:

- “The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies.”

One such mechanism is to indicate the EOB indication in the PDU set information RTP Header Extension. The AS can set the EOB field in the PDU set information RTP HE and inform the UPF about the EOB. The UPF may send an EOB Indication to NG-RAN. When the NG-RAN receives the EOBI in the GTP-U header it understands that that specific PDU is the last PDU of a given Data Burst and it may put the UE to sleep.

# Proposal

The end-of-burst indication can be signalled in the PDU Set information header extension. Alternatively, or in addition, a burst identifier can be signalled in the PDU Set information header extension.

  - End-of-Burst signaling

The syntax and semantics for the end of data burst (2 bits in length) in PDU Set information header extension is as follows:

    - Syntax

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=6   |E|EOB| PRI   |R|      PSSN     |     PSN       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                            PSSize                             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1. RTP header extension using one-byte header format.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x10    |    0x00       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=7   |E|EOB| PRI   |R|      PSSN     |     PSN       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                            PSSize                             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2. RTP header extension using two-byte header format.

    - Semantics

EOB (2 bits): End-of-burst indication.

- When this flag is set to 0x00 the current PDU is not the last PDU of the data burst.

- This flag shall be set to 0x01 for the last PDU of the data burst.

- When this flag is set to 0x10 the end-of-burst indication is not indicated in the PDU Set information RTP header extension or reserved for future usage.

- When this flag is set to 0x11 the PDU is part of the last PDU set of the data burst. The end-of-burst indication is signalled for all the PDUs in the last PDU set of the data burst.

  - Burst ID signaling

Using a burst identifier enables start of burst and end of burst detection by UPF, with limited detection processing requirements. For example, UPF can use a change in burst ID to determine that a new burst has started.

The syntax and semantics for the end of data burst (2 bits in length) and burst ID (8 bits in length) in PDU Set information header extension is as follows:

    - Syntax

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=6   |E|EOB| PRI   |R|      PSSN     |     PSN       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Burst ID      |      PSSize                                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 3. RTP header extension using one-byte header format.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x10    |    0x00       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=7   |E|EOB| PRI   |R|      PSSN     |     PSN       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Burst ID      |      PSSize                                   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 4. RTP header extension using two-byte header format.

    - Semantics

Burst ID (8 bits): Burst identifier. This field value shall be set to an identifier for a burst. PDUs or PDU sets which are intended to be sent in a single burst, e.g., which correspond to the same display time, should have the same Burst ID.

  - Guidelines for setting EOB field

The end of a burst in a media stream may be identified by setting the RTP HE EOB field to 0x01 for the last PDU of an Intra Random Access Pictures (IRAP) picture. The IRAP frames in a media stream are larger in size and usually require several PDUs and PDU sets to transmit the coded frame data.

The end-of-burst may be indicated for the last PDU of each coded picture or the last PDU of a coded group of pictures (GOP). In video coding, a GOP structure, specifies the order in which intra and inter coded frames are arranged. The GOP is a collection of successive pictures within a coded video stream. Each coded video stream consists of successive GOPs, from which the visible frames are generated.

The end-of-burst may be indicated for all the PDUs in the last PDU set of a coded frame or GOP, using RTP HE EOB field value 0x11. In this case, the UPF/RAN may detect an EOB when it completely receives the PDUs of the last PDU set (e.g., by comparing the number of bytes received for this PDU set with the PDU set size or checking if the E flag in the RTP HE is set.

Being able to distinguish PDUs from a given burst from PDUs from another burst can help the UPF to analyse traffic and correctly identify EOB. In cases where multiple bursts take too long to transmit, they can be merged with, and indistinguishable from each other. In such a case, a second burst may follow the first immediately and it may not be useful to signal an EOB to the RAN for the first burst. It can therefore be useful for the UPF to identify individual bursts, to be able to determine when a burst takes longer than a period to transmit, and therefore when a second burst is expected to follow the first immediately.

Typically, all PDUs that are associated with a burst should be associated with the same presentation time stamp. In case of multiple media streams present in a data burst, directly comparing RTP timestamps from different media is not effective for end-of-burst indication. Instead, for each different type of media, the RTP timestamp is related to the sampling instant by pairing it with a timestamp from a reference clock (wall clock) that represents the time when the data corresponding to the RTP timestamp was sampled. In other words, when determining if two or more media flows are part of the same burst, the timestamps in the RTP packet header are not compared directly to each other. Rather, they are compared to a reference clock to determine if they were transmitted at the same time. To save processing time at the UPF, it is therefore useful for the sender to set a burst ID, which can be derived from the presentation timestamp, or could in some cases be a sequence number as well.

The end-of-burst may also be detected when the marker (M) bit in the RTP header is set. The marker bit is intended to allow significant events such as frame boundaries to be marked in the RTP packet stream. When a marker bit is set in an RTP header, it is interpreted that the frame boundary is reached and the corresponding PDU and/or PDU set is marked with end-of-burst indication in the RTP HE.

# Proposal

We propose to agree the proposed syntax, semantics, and guidelines for marking the end-of-burst indication to the PDU set information RTP header extension in 5G_RTP permanent document.

# References

- S2-2303879, 23.501 CR3759R6 (Rel-18, 'B'): Support of XR and Media Services, Tencent, et al.

- S2-2303845, 23.501 CR3875R14 (Rel-18, 'B'): Introduction of support for Jitter Measurement and End of Data Burst reporting to the NG-RAN, Ericsson, et al .