3GPP SA4 RTC Telco post SA4 #128	S4aR240039

Online, 26 June 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] AL-FEC awareness at RAN while considering upstream and downstream packet losses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 06/26/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This addresses Key issue #4: Application-layer FEC awareness for PDU Set handling and in particular regarding “network dropping extra PDUs in a PDU Set encoded with application-layer FEC”. When the RAN drops extra PDUs in a PDU Set, it needs to consider the packet losses that happened in the upstream links or will happen in the downstream links. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a solution by which the RAN drops extra PDUs with the consideration of packet losses that happened in the upstream links or will happen in the downstream links. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN dropping extra PDUs may make it impossible to reconstruct source packets at the application receiver. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# * * * * 1st change * * * *

## 6.x	Solution #x: AL-FEC awareness at RAN while considering upstream and downstream packet losses

### 6.x.1	Key Issue mapping

This maps to Key Issue #4.

### 6.x.2	Description

### 6.x.2.1	Motivation

As explained in clause 5.4.2.2, there is an upstream network segment before the RAN and there may be a downstream network segment after the RAN (e.g., a tethered link). When the RAN drops extra PDUs of a PDU Set encoded with AL-FEC, it needs to consider how many of the PDUs have already been lost in the upstream network segment. The RAN also needs to consider how many packets may get lost in the downstream network segment. Otherwise, the application receiver may not receive enough PDUs for reconstructing all the source PDUs.

### 6.x.2.2	Solution

Figure 6.x.2.2-1 The RAN drops extra PDUs by considering packet losses in the upstream network segment and downstream network segment.

The RAN derives the PDUs that have been lost in the upstream network segment, e.g., by taking the difference between the number of PDUs in the PDU set (NPDS) in the RTP header extension for PDU Set marking for a PDU Set and the number of PDUs of the PDU Set that it has received.

The downlink network segment informs the RAN of the packet loss rate over the tethered link. This is shown in Figure 6.x.2.2-1.

The RAN then decides on how many PDUs are extra PDUs given (1) the upstream packet losses, (2) the packet losses at the RAN (not dropped as extra PDUs), and (3) the downstream packet loss rate.

The following example shows how the RAN decides on the number of extra PDUs that it can drop while still ensuring that the receiver can reconstruct all the source packets with a high probability. Denote the following:

(1) the upstream packet losses: N1

(2) the packet losses at the RAN (not dropped as extra PDUs): N2

(3) the downstream packet loss rate: p

It is assumed that the packet losses in the downsteam are independent. Consider a PDU Set with be K=30 source PDUs, and 15 repair PDUs, hence a total of N=45 PDUs. Let the packet losses at the RAN due to wireless link error (not dropped as extra PDUs) be N2. Let the AL-FEC code be a MDS code. For simplicity each PDU is assumed to be sent in a separate transport block over the RAN air interface. Let N1=2, p=2%. Let the minimum reconstruction success probability be 0.995. Then the allowed values for the number of extra PDUs N3 that the RAN can drop must satisfy the following inequality:

n=Km-N3m-N3n(1-p)n pm-N3-n> 0.995, where m=N-N1-N2.

Figure 6.x.2.2-2 The RAN can drop up to N3=6 extra DPUs while still meeting the reconstruction probability threshold 0.995 with the packet losses in the upsteam, at the RAN (due to wireless link error, not dropped as extra PDUs), and in the downstream.

The above assumes that the extra PDUs dropped at the RAN are not taken as signals of network congestion. Network congestion can occur in the upstream network segment, at the RAN or in the downstream network segment.

FFS: Under what conditions the RAN can drop extra PDUs considering network congestion in the upstream network segment, at the RAN or in the downstream network segment.

# * * * * End of 1st change * * * *