SA4-e (AH) RTC SWG post 129-e	S4aR240048

# 25th September 2024, online

Source:	InterDigital Europe

Title:	Revised Haptic Use case - Haptic-enhanced Communication

Spec:	3GPP TR 26.854

Agenda item:	15.11

Document for:	Agreement

1. Introduction

This contribution revises the use case “haptics-enhanced communication” of section 5.2 of TR 26.854 to provide a simpler and homogeneous description as well as requirements.

2. Reason for Change

As per the above

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

## 5.2		Haptic-enhanced Communication

Use Case Name:

Haptic-enhanced Communication Including: TR 22.856 5.9 “Synchronized predictive avatars” TR 22.856 5.11 “IMS-based 3D Avatar Communication” TR 26.813 UC1 “Avatar Communication”

Description:

This use case addresses communication between people, directly or through an avatar representation. In the simple case, haptic is a new channel added to the traditional communication means, such as i) on text messaging services to add vibrations to emojis, ii) on top of an audio call to enhance the context or just for adding effects, iii) in addition to A/V in a video call to increase the emotional impact… In a more complex scenario, this use case is about one-to-one and multi-party communication (which may be an IMS multimedia telephony call using AR/MR/VR). with spatial audio and haptics rendering, where avatars audio and haptics of each participant in avatar call are transmitted and spatially rendered in the direction of their geolocation. An avatar call is similar to a video call in that both are visual, interactive, provide live feedback to participants regarding their emotions, attentiveness and other social information. Each participant is equipped with display devices (phones, AR glasses, etc.) with external or built-in headphones and haptics renderer.. Note 1: This can include help to people with disabilities. Note 2: Emotional haptic is relevant in this use case. Haptic modalities included here: mostly vibrotactile. Haptic characteristics: Includes 2-way communication (talks, social media, phone calls…) and 1-way (alerts messages, information…). Multi-users, low to high data-rates per channel, limited number of channels.

Categorization

Type: 2D/3D audio-video sessions Delivery: Conversational, Split Device: headphones, smartphones, smartwatches

Preconditions

- Video and audio communication devices with associated haptic actuators for physical feedback - Library of created effects to be sent or/and - Accessories to capture information and generate haptic effects.

Requirements

Bit-rates = 8-16 kbit/s per channel Message size = TBD Number of Channels = limited, between 1 and 4 with sparse packets. Delay = lower than 1 audio frame (< 10 ms) Format: parametric Editor’s note: to be refined when additional data is provided.

Feasibility

Enabling technologies have reached a sufficient maturity in terms of: - Haptics experiences creation, coding, transmission and rendering. Haptic effects library can be used. - Integration into communication applications - Integration into hardware devices Connected devices in various form factors exist that are capable of decoding and rendering haptics (smartwatches, mobile phones, rings…).

Interoperability considerations

Haptic data should be distributed and rendered with various applications, environments and devices. Adaptation to the rendering capabilities should be supported.

Potential Standardization Status and Needs

- Haptics data representation and coding (section 6 and 7) - Haptics support in Scene description and avatar representation - Haptics data carriage, transport and streaming (section 7) - Integration in 5G architecture and services. (Section 8)

Table 5.3: use case Haptic-enhanced Communication

* * * End of Changes * * * *