3GPP SA4 RTC Telco Post #129-e	S4aR240055

Online, 25 September 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] Traffic Characteristics for XR Split Rendering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 09/25/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This addresses Key Issue #12: Enhancements of Data Burst Marking It is important to understand the traffic characteristics for XR split rendering before we consider the potential solutions to indicating the characteristics to the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the traffic characteristics for XR split rendering to the TR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The potential solutions may not address the traffic characteristics for XR split rendering. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

[xx] 3GPP TR 26.812, Study on QoE Metrics for AR/MR Services, V18.1.0, July 2024.

# * * * * end of 1st change * * * *

# * * * * 2nd change (all new) * * * *

## 6.x Traffic Characteristics for XR Split Rendering

### 6.x.1 General description

In XR applications with split rendering, the split rendering server generates the rendered media targeting the predicted pose for a future time instant of a display refresh, also known as Vsync. The call flow is detailed in 6.3.6.3 of [xx]. The display refresh rate for displays in deployment is constant or remain contant for some extended period of time before the refresh rate is changed. To optimize the user experience, the pace of the rendering at the application server may be synchronized with the display refresh instants via a phse locked loop (PPL), hence leading to periodic traffic departing the split rendering server, as illustrated in Figurure 6.x.1.

Figure 6.x.1 The split rendering server renders a video frame for a target Vsync time instant.

### 6.x.1 Experimental results

To verify the periodic traffic pattern for split rendering, a lab experiment was performed. In the setup, the HMD is connected to a server that performs split rendering via Wi-Fi 6 (IEEE 802.11ax). The traffic pattern departing the server is measured and shown in Figure 6.x-2.

Figure 6.x.2 Packet departure time for rendered video at the split rendering server.

The distribution of the inter-frame delay (the difference between the departure time of the first packet of the current video frame and the departure time of the first packet of the next video frame) is shown in Figure 6.x.3. The display fresh rate is 90 FPS, corresponding to an inter-frame delay of 11.1ms. From the CDF we see that the inter-frame delay exhibits periodic behavor, with the median inter-frame delay 11.1ms, maximum deviations of -3ms and 1.6ms, and 90% with an error in the range [-0.9, 0.6] ms.

Figure 6.x.3 Inter-frame delay distribution

### 6.x.1 Conclusion

The packet departurue time for video frames rendered by a split rendering server may exhibit a periodic pattern in line with a constant video display refresh rate.

# * * * * end of 2nd change * * * *