3GPP SA4 RTC Telco Post #129-e	S4aR240077

Online, 9 October 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.854 | CR | pseudo | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_HapticsMedia] On haptic media formats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_HapticsMedia |  |  |  |  |  | Date: |  |  | 10/09/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The clause 6 of TR 26.854 is titled “Haptic input formats and device types”, but it does not define ‘input’. It is not clear what the input is relative to. The paper S4aR240051 defines input as “input formats are the formats representing the raw data (or source formats) and used at the production side (e.g. for encoding, packaging and delivery)”. But the input of the current module is the output of the previous module in the haptic pipeline. If we define the input format, then we also need to define the output format. This unncesarily bloats the TR. Temperature (piezzo) is wrong. Besides being captured by a sensor, time-sampled signals can be generated by a haptic renderer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Haptic input format is changed to haptic media format. Removed (piezzo) Removed ‘captured by sensors’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR will be confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

# 6	Haptic media formats and device types

## 6.1	Introduction

Haptics refers to the sense of touch, and encompasses the generation, manipulation, and perception of tactile sensations, forces, and motions. In the media domain it relates to the capture and rendering of physical information about objects and environments due to interactions with the user. The capture is generally performed with sensors and the rendering with hardware actuators allowing to render different modalities.

In general, three types of modalities are considered, tactile (vibration, temperature, pression), kinaesthetic (force) and proprioception (motion, acceleration). Those modalities are rendered in the human body through different mechanoreceptors. The density and properties of those receptors are different depending on the modality and the body parts. In addition, each individual has different sensibility, which can be managed through users’ profiles and adaptation.

Finally, the rendering is performed with specific hardware actuators embedded into user devices. The different Haptic modalities are rendered with different actuators. Typically, tactile sensations can be rendered with vibrotactile (e.g. linear resonant actuators (LRA), eccentric rotating mass (ERM), temperature, wind (fan), force) … devices.

## 6.2	Haptics media formats

### 6.2.1	Introduction

Haptic media formats are essential in capturing and transmitting tactile sensations in a variety of applications, ranging from simple vibrations in mobile devices to complex force feedback in virtual reality systems. Two types of input formats exist, time-sampled signals (typically PCM encoded), and parametric description of haptic effects.

# * * * * end of 1st change * * * *