SA4-e (AH) RTC SWG post 129-e	S4aR240093

23rd Oct. 2024, online

Source:	InterDigital Europe

Title:	Haptics media communication pipeline

Spec:	3GPP TR 26.854

Document for:	Agreement

1. Introduction

As per the minutes of the Sept 25th call, this contribution provides a haptics media communication pipeline.

2. Reason for Change

Provide initial text for Haptics media communication pipeline to populate section 4 for TR 26.854.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

## 4.3 	Haptics media pipelines

Haptics can be used as a media type at the same level as audio and video. Figure 4.3-1 depicts an end-to-end streaming pipeline associating audio, video and haptics in the most complete scenario of interactive 3D scenes. A simpler version of the pipeline consists in 2D AVH (Audio-Video-Haptics media) with only 2D assets and no interaction.

Figure 4.3-1 Audio-visual Haptic end-to-end streaming pipeline.

Figure 4.3-1 also illustrates the different formats and some existing APIs supporting haptics media.

Figure 4.3-2 depicts an end-to-end real-time communication pipeline associating audio, video and haptics. In a real-time communication use case, the haptic media is managed similarly to audio and video. The input is captured from input sensors, a camera for the video, a microphone for audio and dedicated haptic sensors (motion, pressure…) for haptics. Alternatively the signal can be loaded from the storage, typically a library of encoded effects can be stored and retrieved on the fly and sent to the receiver. The signals are then encoded and distributed using existing coded formats and protocols.

On the receiver side, the various media are decoded and rendered with the appropriate devices, loudspeaker, screen or vibrotactile devices.

Figure 4.3-2- Audio-visual Haptic real-time communication pipeline.

Figure 4.3-2 also illustrates the different formats and some existing APIs supporting haptics media.

* * * End of Changes * * * *