3GPP TSG-SA4 Meeting #(AH Video SWG post 129-e) 	S4aV240060

# online,  , 19 - 29 Oct 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.265 | CR | pseudo | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [VOPS] Video Signal Parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VOPS |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4, 6, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[h264]	ITU-T Recommendation H.264 (08/2021): "Advanced video coding for generic audiovisual services".

[h265]	ITU-T Recommendation H.265 (09/2023): "High efficiency video coding".

[CMAF]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[CENC]	ISO/IEC 23001-7: "MPEG systems technologies - Part 7: Common encryption in ISO base media file format files".

[DPC]	CTA-5003-A & Errata: "Web Application Video Ecosystem (WAVE): Device Playback Capabilities Specification", available at https://cdn.cta.tech/cta/media/media/resources/standards/pdfs/cta-5003-final.pdf.

[6381]	IETF RFC 6381: The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types.

[MSE]	3GPP TR 26.857, "5G Media Service Enablers"

[h274]	ITU-T Recommendation H.274 (09/2023): "Versatile supplemental enhancement information messages for coded video bitstreams".

[h273]	ITU-T Recommendation H.273 (09/2023): "Coding-independent code points for video signal type identification".

## =====  CHANGE =====

## 4.2	Reference architectures and definitions

In order to define the normative aspects of this specification, reference architectures are defined. The core architecture is provided in Figure 4.2-1. The workflow addresses the generation of a video bitstream from a video signal using a video encoder as well as the decoding of a video bitstream by a video decoder and providing the resulting decoded video as well as associated metadata to a rendering and display process. The video encoder as well as the video decoder may be configured to certain operations indicated by APIs in Figure 4.2-1. These APIs are not normatively specified but serve as an example reference to configure encoders and decoders as documented in Annex [A].

Figure 4.2-1 Reference architecture for video operating points and capabilities

A more system-centric architecture is provided in Figure 4.2-2. The workflow addresses the generation of a transport stream from a video signal using a video encoder and a packager. The package may include for example timing and metadata information. The de-packaging and decoding of the transport stream by a de-packager and a video decoder, respectively, allows for providing the resulting video signal as well as associated metadata to a rendering and display process. Again, the packager/encoder as well as the de-packager/decoder may be configured to certain operations indicated by APIs in Figure 4.2-2.

Figure 4.2-2 Reference architecture for system operating points and capabilities

Editor’s Note: A reference architecture for multiple decoders still needs to be defined.

Based on this introduction, the following terms are defined

Operating Point: A collection of different possible video formats including spatial and temporal resolutions, colour mapping, transfer functions, etc. and a video encoding format.

Bitstream: A compressed media representation presented as a sequence of bits that conforms to a particular video coding specification/format and one or more Operating Points.

Receiver: A device that can ingest and decode any bitstream that is conforming to a particular video coding specification and Operating Point, and optionally render it.

In addition, on system level the following terms are defined:

System Operating Point: A collection of different possible video formats including spatial and temporal resolutions, colour mapping, transfer functions, etc., a video encoding and a packaging format.

Transport Stream: A packaged media bitstream that conforms to a particular video coding and packaging specification/format and one or more Operating Points.

System Receiver: A receiver that can de-package and decode any system bitstream that is conforming to a particular System Operating Point, and optionally render it.

## =====  CHANGE =====

### 4.4.2	Video signal parameters

Video signals considered in this specification are represented by a sequence of pictures, where a picture represent an array of luma samples in monochrome format or an array of luma samples and two corresponding arrays of chroma samples in 4:2:0, 4:2:2 and 4:4:4 colour format. Only progressive signals are considered, not interlaced. Luma represents a sample array or single sample representing the monochrome signal related to the primary colours (denoted with the symbol Y), and chroma represents a sample array or single sample representing one of the two colour difference signals related to the primary colours, represented by the symbols Cb and Cr. A component refers to an array or single sample from one of the three arrays (luma and two chroma) that compose a picture.

Video signals are typically described by a set of parameters that are required for properly rendering the decoded signal. Table 4.4.2-1 documents typical video signal parameters and provides a definition and/or reference.

Table 4.4.2-1	Video Signal Parameters

| Parameter |  | Definition |  |  | 3GPP restrictions |
| --- | --- | --- | --- | --- | --- |
| Spatial Resolution width |  | the number of samples per active line for the luma component. Typical values are 1280 or 1920. |  |  | no direct restrictions, but services may provide subsets. |
| Spatial Resolution height |  | the number of active lines per picture for the luma component. Typical values are 720 or 1080. |  |  | no direct restrictions, but services may provide subsets. |
| Scan Type | indicates the the source scan type of the pictures as defined in clause 7.3 of ITU-T H.273. Typical value is progressive | progressive only |
| Chroma format indicator |  | indicates whether the picture has only a luma component or that the picture has three colour components that consist of a luma component and two associated chroma components, such that the width and height of each chroma component are the width and height of the luma component divided by a factor defined by the chroma format as defined in ITU-T H.274, clause 7.3. The chroma format indicator when set to 4:2:0 may be augmented with a locator indicating the offset of the sampled chroma component from the corresponding luma position as defined in ITU-T H.274, clause 7.3. Typical values are 4:2:0, for which the chroma components are subsampled by a factor of 2 in both directions and the indicator to set to half sample offset in both directions. |  |  | 4:2:0, half sample offset |
| Bit depth |  | Indicates the bit depth for the samples of the luma component and the samples of the two associated chroma components. Typical values are 8 or 10 bit. |  |  | 8 or 10 bit |
| Colour primaries |  | indicates the chromaticity coordinates of the source colour primaries as specified in clause 8.1 of ITU-T H.273. Typical values are 1 to refer to ITU-R BT.709-2 or 9 to refer to ITU-R BT.2020-2 and ITU-R BT.2100-2. |  |  | 1 or 9 |
| Transfer Characteristics |  | either indicates the reference opto-electronic transfer characteristic function of the source picture as a function of a source input linear optical intensity input, or indicates the inverse of the reference electro-optical transfer characteristic function as a function of an output linear optical intensity as defined in clause 8.2 of ITU-T H.273. Typical values are 1 to refer to ITU-R BT.709-2, 14 to refer to ITU-R BT.2020-2 (10 bit), 16 to refer to ITU-R BT.2100-2 perceptual quantization (PQ) system, or 18 to refer to Rec. ITU-R BT.2100-2 hybrid log-gamma (HLG) system |  |  | 1, 14, 16, or 18 |
| Matrix Coefficients |  | describes the matrix coefficients used in deriving luma and chroma signals from the green, blue and red primaries. A video full range flag may be supplied with this parameter specifying the scaling and offset values applied in association with the Matrix coefficients. For detailed definition refer to clause 8.2 of ITU-T H.273. Typical values are 1 to refer to ITU-R BT.709-2 or 9 to refer to ITU-R BT.2020-2 and ITU-R BT.2100-2. |  |  | 1 or 9 |
| Frame rate |  |  |  |  | No specific restrictions |
| Frame packing | indicates a frame packing arrangement, if present, as defined in ITU-T H.274, clause 7.3, and typically refers to packing arrangements in clause 8.6 of ITU-T H.274. Typical values are no frame packing. | Typically restricted to no frame packing, but applications may use frame packing. |
| Projection | indicates a projection, if present, as defined in ITU-T H.274, clause 7.3, and typically refers to packing arrangements in clause 8.6 of ITU-T H.274. | Typically restricted to no projection, but applications may use projections. |
| Sample aspect ratio |  | indicates width-to-height aspect ratio of the luma samples of the associated pictures as defined in clause 7.3 of ITU-T H.273. Typical values are 1 |  |  |  |

## =====  CHANGE =====

### 4.4.3	Video Representation Formats

Video representation formats are described by a set of well-defined video signal parameters as defined in clause 4.4.3.

Editor’s Note:

- check what is available in TS 26.116, also from S4-240619

- Standard Definition and SDR:

- High Definition and SDR  BT.709-1

- Ultra-High Definition and HDR PQ  BT.2100-1

- Ultra-High Definition and HDR HLG  BT.2100-1