3GPP SA4-MBS SWG AH on 5GMS3	S4-AHI886

# The Hague, Netherlands, 12th-14th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | - | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposed updates on the TV profile for 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | November 2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed implementation of the Television profile for 5GMS unicast downlink with considerations of either applying the PSS capabilities as defined in the 5GMS3 WID or upgrading the decoding capability requirements to noawadays general support of media formats particularly on video formats. 2 options are provided under square brackets for further considerations in SA4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 ; 3.3 ; 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5GMS2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[5Gaudio]	3GPP TS 26.117,"5G Media Streaming (5GMS); Speech and audio profiles".

[html5]	3GPP TS 26.307, "Presentation Layer for 3GPP Services".

[vr]	3GPP TS 26.118, "3GPP Virtual reality profiles for streaming applications".

[tv]	3GPP TS 26.116, "Television (TV) over 3GPP Services; Video Profiles".

End of change 1

Start of change 2

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

5GMSA	5G Media Streaming Architecture

AS	Application Server

DASH	Dynamic Adaptive Streaming over HTTP

HD	High Definition

HDR	High Dynamic Range

OMAF	Omnidirectional Media Application Format

TV	Television

UE	User Equipment

UHD	Ultra High Definition

VR	Virtual Reality

End of change 2

Start of change 3

## 4.3	Television profile

### 4.3.1	Video decoding

[

Option A (PSS capabilities)

#### 4.3.1.1	General

If the 5GMS client supports the Television profile, the following requirements on video decoding capabilities as defined in clause 4.3.1.2 for H.264 (AVC) and clause 4.3.1.3 for H.265 (HEVC) apply.

#### 4.3.1.2	H.264 (AVC)

The 5GMS UE shall support the H.264/AVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.4.2.6.

The 5GMS UE should support the H.264/AVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.4.3.6.

#### 4.3.1.3	H.265 (HEVC)

The 5GMS UE should support:

-	H.265/HEVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.2.7,

-	H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.3.7,

-	H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.4.7,

-	H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.5.8, and

-	H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.6.8.

End of option A

Option B (HEVC only)

The 5GMS UE shall support:

-	H.265/HEVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.2.7,

-	H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.3.7.

The 5GMS UE should support:

-	H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.4.7,

-	H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.5.8, and

-	H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.6.8.

End of option B

]

### 4.3.2	Audio decoding

If the 5GMS client supports the Television profile, Enhanced aacPlus decoder shall be supported as defined in 3GPP TS 26. 117 [5Gaudio], clause 5.3

End of change 3