3GPP TSG-4 MBS SWG AH on 5GMS3	S4-AHI888

# The Hague, Netherlands, 12th-14th Nov. 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.117 | pCR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR regarding AAC family |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Mobile Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. The scope is adjusted to refer to 5GMS, otherwise it is too open – “and other applications” removed. 2. A reader interested in high-quality audio streaming would not easily understand what further suitable audio codecs are supported implicitly. 3. The word “Encoding” in the heading appears only for EVS in 6.2.4.1 while it should be relevant also for other codecs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the requriements in practice encompass not only eAAC+ but also other members of that codec family, which might be preferred for services using higher bit-rates. Add the word “Encoding” to appropriate places |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes are implemented on top of output draft TS from SA4#106 (S4-191315) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------START OF CHANGE -----------------------------

# 1	Scope

The present document specifies speech and audio media capabilities, operation points and media profiles for 5G Media Streaming in the context of 3GPP services and deployments.

-------------------------END OF CHANGE -------------------------

-------------------------START OF CHANGE -----------------------------

## 5.3	Audio Media Decoding Capabilities

The following audio media decoding capabilities are defined:

-	eAAC+: All decoding requirements for the eAAC+ audio codec as specified in 3GPP TS 26.401 [19], 3GPP TS 26.402 [20], 3GPP TS 26.410 [24] and 3GPP TS 26.411 [25].

-	AMR-WB+: All decoding requirements for the AMR-WB+ audio codec as specified in 3GPP TS 26.290 ‎‎[26], 3GPP TS 26.304 ‎[27] and 3GPP TS 26.273 [28].

-------------------------END OF CHANGE -------------------------

------------------------- START OF CHANGE -------------------------

#### 6.2.3.1	Bitstream Encoding Requirements

The following requirements apply to the AMR-WB Operation Point.

- The sampling frequency shall be 16 kHz.

- The bitstream shall either be encoded

  - according to 3GPP TS 26.173 ‎[10] [ or 3GPP TS 26.204 [11]],. or

  - [if the EVS codec is supported, EVS AMR-WB IO may serve as an alternative implementation of the AMR-WB codec as described in TS 26.445 [16] and TS 26.447 [35], the bitstream may also be encoded using the AMR-WB IO mode according to TS 26.442 [14], TS 26.443 [15], or TS 26.452 [34].]

Note that the bitstream produced by the AMR-WB encoder consists of 20ms encoded speech frames.

-------------------------END OF CHANGE -------------------------

-------------------------START OF CHANGE -----------------------------

#### 6.2.5.1	Bitstream Encoding Requirements

The following requirements apply for the EVS-Dual-Mono Operation Point.

- The sampling frequency shall be one of the following: 8, 16, 32, 48 kHz.

- The bitstream shall be encoded according to either TS 26.442 [14], TS 26.443 [15], or TS 26.452 [34] encoding functions.

Note that the bitstream produced by the EVS encoder consists of 20ms encoded speech frames. Two EVS encoders are required to generate the speech frames for the left and right channel independently.

The packetization of EVS dual-mono is described in [16] (see clause A.2.6.2).

-------------------------END OF CHANGE -------------------------

-------------------------START OF CHANGE -----------------------------

### 6.3.2	eAAC+ stereo

#### 6.3.2.1	Bitstream Encoding Requirements

The following requirements apply to the eAAC+ stereo Operation Point.

- The sampling frequency shall be either 32kHz, 44.1kHz or 48kHz.

- The bitstream shall be encoded according to 3GPP TS 26.401 [19], clause 7, as well as 3GPP TS 26.403 [21], 3GPP TS 26.404 [22] and 3GPP TS 26.405 [23].

NOTE: The specified eAAC+ encoder consists of AAC-LC with additional tools that can be enabled (SBR, PS and more), see [19].

#### 6.3.2.2	Receiver Requirements

Receivers conforming to the eAAC+ stereo Operation Point shall support the eAAC+ media decoding capability according to clause 5.3 and shall support playback of the decoded signal.

NOTE: The eAAC+ decoder supports decoding of streams encoded with AAC-LC or aacPlus, see [19].

-------------------------END OF CHANGE -------------------------

-------------------------START OF CHANGE -----------------------------

#### 6.3.3.1	Bitstream Encoding Requirements

The following requirements apply to the AMR-WB+ Operation Point.

- The sampling frequency shall be either 8, 16, 32 or 48 kHz.

- The bitstream shall be encoded according to either 3GPP TS 26.273 [28] or 3GPP TS 26.304 [27].

-------------------------END OF CHANGE -------------------------