3GPP TSG-SA4 MBS AHG Meeting	S4-AHI893

# The Hague, The Netherlands, 12-14 November, 2019					 revision of S4-AHI874

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RAN Signaling based Downlink Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 is currently missing description and call flow on the RAN signaling based method for downlionk network assistance. This CR proposes additional text to be added on this matter to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 5.X on RAN signaling based downlink assistance functionality in the context of DASH streaming service delivery, and new clause 6.X on high level system architecture illustration of RAN signaling based downlink network assistance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | System architecture and interaction between relevant entities in support of RAN signaling based downlink network assistance remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses 5.X and 6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

## 5.X	Establish a Unicast Media Downlink Streaming Session with RAN signaling based network assistance

### 5.X.1	General

This clause describes the establishment of a unicast downlink streaming session with RAN signaling based interactions for network assistance. Clause 6.X provides an descriptive overview and network architecture for RAN signaling based network assistance. The UE (its MAC layer entity) communicates with the RAN to request/receive recommendation of the maximum bitrate supported by the RAN for the media content carried on the radio bearer associated with the media stream of interest.

### 5.X.2	DASH Streaming

The message sequence diagram and corresponding description below pertain to the establishment of a unicast media downlink streaming session with RAN signaling based interactions for network assistance. A streaming session may use DASH format or other (potentially non-3GPP defined) formats.

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.X-1: High Level Procedure for DASH content

Prerequisite:

-	The 5GMS Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMS Aware Application has received the service announcement from the 5GMS Application Provider.

-	The 5GMS Application Provider has established agreement of one or more policies with the network provider, and the 5GMS Application Provider is authorized to apply QoS policy dynamically during a downlink streaming session.

-	The service announcement information contains the URL of the 5GMSd AF.

Steps:

1: The 5GMSd Aware Application triggers the Content Discovery procedure. This procedure only involves the 5GMSd Application and the external Application Server.

2: A media content item is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start media playback. Access information to the Media Player is provided to the Media Session Handler.

4: The Media Session Handler is assumed to be aware of, and interacts with, the UE MAC Entity to activate bitrate recommendation functionality.

5: The UE MAC Entity is prepared to send bitrate recommendation information and/or receive bitrate boost request from the Media Session Handler.

6: (Optional) Should the 5GMSd Aware Application have previously received a reference to the service access information, the Media Session Handler interacts with the 5GMSd AF to acquire the entire service access information.

7: The Media Session Handler triggers the Media Player to start the session.

8: The Media Player establishes the transport session for acquiring the MPD whose URL is contained in the Media Player access information.

9: The Media Player requests the MPD.

10: The media access client receives the MPD.

11: The Media Player processes the MPD. It determines for example the number of needed transport sessions for media acquisition. The Media Player should be able to use the MPD information to initialize the media pipeline for each media stream. The MPD should also contain information to start the DRM client initialization, when DRM is used.

12: The Media Player notifies the media session handler about MPD reception. The notification contains parameters from the MPD.

13: Optional: the Media Player acquires the necessary DRM information such as a DRM License.

14: The Media Player configures the media rendering pipeline.

15: The Media Player establishes the necessary transport sessions with the 5GMSd AS for streaming content delivery (e.g. TCP connections). For example, the Media Player may establish one transport session for each media component (audio, video, etc.), and possibly additional transport sessions for other media representations.

16: The Media Session Handler is notified about the Flow description(s).

17: The Media Player requests initialization information, and repeats this step for each required Initialization Segment.

18: The Media Player receives the initialization information.

19: The Media Player requests Media Segments of a particular Adaptation Set at bitrate R0 according to the MPD.

20: Downlink streaming content is received at bitrate R0 by the Media Player and is Optional: placed in the corresponding media rendering pipeline.

21: Steps 19 and 20 are repeated.

One of the following steps subsequently occurs:

22a: (Optional) The RAN provides a higher bitrate recommendation resulting in the Media Player taking advantage of such recommendation in subsequently acquiring content at a higher bitrate. In this step, the RAN sends recommended bitrate R2, with R2>R1, to the UE MAC Entity, which in turn relays that information to the Media Player via the Media Session Handler. T

he Media Player performs an evaluation of bitrate adaptation, and in accordance to the MPD, requests for/receives Media Segments of a different Adaptation Set at bitrate R1’, with R1<R1’<R2.

22b: (Optional) The Media Player requests a bitrate boost which is granted by the RAN. In this step, in accordance to the MPD, the Media Player wishes to obtain a bitrate boost from R1 to R2’. Due to the RAN responding with a bitrate recommendation of R3>R2’, the Media Player subsequently requests and receives Media Segments at bitrate R2’.

22c: (Optional) The RAN determines that the previous bitrate R1 can no longer be supported, and sends a lower bitrate recommendation resulting in the Media Player requesting/receiving content at a lower bitrate than R1. In this step, the RAN sends recommended bitrate R0, with R0<R1, to the UE MAC Entity, which in turn relays that information to the Media Player via the Media Session Handler. The Media Player performs an evaluation of bitrate adaptation, and in accordance to the MPD, requests for/receives Media Segments of a different Adaptation Set at bitrate R0’, with R0’<R0<R1.

END OF FIRST CHANGE

START OF SECOND CHANGE

## 6.X	RAN Signaling based Support for Downlink Network Assistance

The diagram in Figure 6.X-1 depicts a downlink streaming architecture where RAN signaling is employed to support network assistance functionality.

The RAN in the architecture contains control plane and user plane entities that interact with peer control and user plane entities in the UE. The control plane interactions involve the sending by the RAN of downlink bit recommendations to the UE, which may occur either independently, or in response to a boost request from the UE, via the exchange of the bit rate recommendation message and bit rate recommendation query message between the RAN and UE as described in TS 38.321 [X]. The user plane interaction pertains to downlink streaming media transport. This control interface is outside the scope of 5GMSA. The user plane functionality as shown in Figure 6.X-1 (solid line going from DL Media AS to RAN to UE) illustrates the scheduling/pass-through functionality associated with user plane communications, governed by the RAN and which may occur as result the bitrate recommendation messaging over the control plane (shown by dotted line between the UE and RAN).

It is assumed in this case that application-level network assistance, i.e., as supported by the DL Media AF is not utilized. However, it should be also assumed that when RAN signaling based network assistance is performed, there is a higher level network entity which coordinates and tracks network assistance performed using RAN signaling vs. application signaling for individual UEs.

Figure 6.X-1: RAN Signaling based Downlink Network Assistance

END OF SECOND CHANGE