Source:	VoiceAge Corporation

Title:	Proposal to progress the IVAS Characterization Test Plan

Agenda Item:	2.4

Document for:	Discussion and Agreement

Introduction

At the last SA4 meeting in Sophia Antipolis, contribution [1] presented an alternative approach to the Characterization testing of the IVAS codec based on comparing the new codec to previous service quality. The proposal is elegant in the sense that the same test structure is proposed for all experiments. Hower, it also implies comparing the IVAS immersive formats rendered to loudspeakers or stereo headphones with the mono rendering of the EVS codec. The proposal comprised both MUSHRA tests and P.SUPPL800 tests with emphasis on MUSHRA.

As claimed in [1], comparing different spatial formats in a MUSHRA test seems reasonable, given the test methodology and the usage of experimented listeners. However, concerns were expressed during the Sophia Antipolis meeting, and also at previous Audio SWG calls, whether a P.800 type of test is appropriate for such a task which might be confusing for naïve listeners. It is the opinion of the source that there is not enough data to be confident that comparing different spatial formats in a P.800 type of test, using naïve listeners, would accurately reflect the leap the immersive audio brings compared to mono.

Additionally, the proposal [1] does not include test conditions evaluating combinations of floating-point encoder and fixed-point decoder and fixed-point encoder and floating-point decoder. While it was argued that objective evaluation of these configurations might be sufficient if at least the fixed-point encoder / fixed-point decoder conditions are assessed also subjectively, we consider that at least a limited subjective cross-check would be beneficial: given the complexity of the IVAS codec, the encoder or the decoder can take on different processing paths which can bias the objective comparisons.

To progress the Test Plan, this contribution builds on the proposal in [1]. Based on the discussion above, it is proposed to remove the mono conditions from the P.SUPPL800 experiments and fill up the free condition spaces with combinations of floating/fixed-point encoder and decoder. Such an experiment setup will still provide the rate-distortion curve for IVAS while avoiding the problematic comparison of different levels of audio immersion, and at the same time it will more comprehensively assess the performance of the fixed-point code.

Modified condition list for P.SUPPL800 experiments

Based on Table 2, section 4.3.1, of the last revision of the IVAS Characterization Test Plan [2], the proposed modifications are highlighted in Table 1 below:

Table 1 - Conditions for P.SUPPL800 experiments

| Label | Condition | Bitrate [kbps] | DTX |
| --- | --- | --- | --- |
| c01 | Reference | - | - |
| c02 | MNRU Q=xx dB | - | - |
| c03 | MNRU Q=xx dB | - | - |
| c04 | MNRU Q=xx dB | - | - |
| c05 | MNRU Q=xx dB | - | - |
| c06 | ESDRU α=xx | - | - |
| c07 | ESDRU α=xx | - | - |
| c08 | ESDRU α=xx |  |  |
| C09 | ESDRU α=xx | - | - |
| c10 | IVAS FL enc / FX dec | 13.2 |  |
| c11 | IVAS FX enc / FL dec | 16.4 |  |
| c12 | IVAS FL enc / FX dec | 24.4 |  |
| c13 | IVAS FX enc / FL dec | 32.0 |  |
| c14 | IVAS FL enc / FX dec | 48.0 |  |
| c15 | IVAS FX enc / FL dec | 64.0 |  |
| c16 | IVAS FL enc / FX dec | 80.0 |  |
| c17 | IVAS FX enc / FL dec | 96.0 |  |
| c18 | IVAS FL enc / FX dec | 128.0 |  |
| c19 | IVAS FL | 13.2 |  |
| c20 | IVAS FL | 16.4 |  |
| c21 | IVAS FL | 24.4 |  |
| c22 | IVAS FL | 32.0 |  |
| c23 | IVAS FL | 48.0 |  |
| c24 | IVAS FL | 64.0 |  |
| c25 | IVAS FL | 80.0 |  |
| c26 | IVAS FL | 96.0 |  |
| c27 | IVAS FL | 128.0 |  |
| c28 | IVAS FX | 13.2 |  |
| c29 | IVAS FX | 16.4 |  |
| c30 | IVAS FX | 24.4 |  |
| c31 | IVAS FX | 32.0 |  |
| c32 | IVAS FX | 48.0 |  |
| c33 | IVAS FX | 64.0 |  |
| c34 | IVAS FX | 80.0 |  |
| c35 | IVAS FX | 96.0 |  |
| c36 | IVAS FX | 128.0 |  |

Testing different Input levels

The current experiment proposals assume the input signal level of -26 LKFS. As different input signal levels can stress the fixed-point implementation performance, traditional input level variations of -10 dB and +10 dB should be integrated in the Test Plan. Different input levels can be integrated for different audio samples within source material categories, or alternatively different input signal levels can be assigned to particular categories.

DTX

Currently DTX operation is missing in most of the P.SUPPL800 experiment proposals (see Table 2). It might be good to cover the DTX operation more extensively.

Question of replication of the Characterization experiments

During the Selection test, all experiments were replicated twice in two different LLs, generally with good correlation of the results in both LLs. I was thus suggested that replicating each experiment twice might not be needed, and that the listening resources might be better spent on larger coverage of different IVAS configurations. This applied in particular for MUSHRA tests using expert, or at least experimented, listeners. In the discussions during the Chicago SA4 meeting, this assumption was questioned with the argument that in case of unexpected results it will not be clear whether an issue is related to the codec or to an issue in running the experiment in a LL. As a solution it was proposed that a potential subsequent verification phase could be scheduled to verify experiments with unexpected results. Nevertheless, with the alternative proposal of [FhG], the number of P.SUPPL800 experiments was reduced, consequently reopening the possibility of running some of the experiments in two LLs.

Assignement of experiments to external LLs

With the limited budget of 192000 E for testing at external LLs, it is proposed that the external LLs run exclusively P.SUPPL800 experiments. If there is remaining testing capacity, some of the experiments can be then x-checked by volunteering companies. Under this assumption, and considering the same price per experiment as in Selection, this would mean 5 P.SUPPL800 experiments per external LL, leaving 12000 E. If agreeable, the overall testing capacity would then be updated as follows:

External LLs:

10 x P.SUPPL800

Volunteering LLs (last estimate):

14 x P.SUPPL800

21 x BS.1534

Details (extracted from [ ]):

Dolby – 4-6 BS.1534 tests, both loudspeaker and binaural rendering is possible, tentatively 2 P.SUPPL800 tests

Ericsson – 2 P.SUPPL800 tests, and at minimum 2 BS.1534 tests, both loudspeaker and binaural rendering is possible.

FhG – tentatively 2 P.SUPPL800 tests, and at minimum 2 BS.1534 tests, both loudspeaker and binaural rendering is possible.

Huawei – 2 BS.1534 tests, binaural rendering.

Nokia – 1 ACR, 3 P.SUPPL800 tests, and 6 BS.1534 tests, both loudspeaker and binaural rendering is possible.

NTT – 1 P.SUPPL800 test, 1/2 BS.1534 (shared with Panasonic), binaural rendering.

Orange –2 P.SUPPL800, 1 BS.1534.

Panasonic – 1/2 BS.1534 (shared with NTT), binaural rendering.

Philips – tentatively 1 BS.1534, 1 room acoustics testing.

QC – tentatively 2 BS.1534.

VoiceAge – 2 P.SUPPL800 tests.

Total:

24 x P.SUPPL800

21 x BS.1534

Updated tables on Experiment allocation

Taking as a basis the list of already proposed experiments in the Test Plan [2] including the Alternative testing proposal in [1], the updated tables would look as follows (after a split for P.SUPPL800 experiments and BS.1534 experiments):

Table 2: Allocation of P.SUPPL800 experiments

| Exp | Objectif | Input format | Source material | FX validation | DTX | FER | Language | LL 1 | LL 2 | Pricing [Euros] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P800-1 |  | Stereo | All | Yes | Off | 0% |  |  |  | 18000 |
| P800-2 |  | FOA | All | Yes | Off | 0% |  |  |  | 18000 |
| P800-3 |  | MC 5-1 | Clean speech, mixed/music | Yes | Off | 0% |  |  |  | 18000 |
| P800-4 |  | 1-4 Objects | Clean speech, speech + effects, speech + music, music | Yes | Off | 0% |  |  |  | 18000 |
| P800-5 |  | MASA 1 TC | All | Yes | Off | 0% |  |  |  | 18000 |
| P800-6 |  | OSBA 1-4 Objects | All | Yes | Off | 0% |  |  |  | 18000 |
| P800-7 |  | MASA 1-TC | Clean speech | Yes | Off | 0%, 5% |  |  |  | 18000 |
| P800-8 |  | MASA 1-TC | Speech+bckg | Yes | On/Off | 0% |  |  |  | 18000 |
| P800-9 |  | OSBA | Speech+bckg | Yes | Off | 0%, 5% |  |  |  | 18000 |
| P800-10 |  | OMASA | Speech+bckg | Yes | Off | 0%, 5% |  |  |  | 18000 |
| P800-11 | Stereo downmix | Stereo | All |  |  |  |  |  |  |  |
| P800-12 | JBM/FER/Tan | Stereo | Clean speech |  |  |  |  |  |  |  |
| P800-13 | JBM/FER/Tan | 2 Objects | Clean speech |  |  |  |  |  |  |  |
| P800-14 |  |  |  |  |  |  |  |  |  |  |
| P800-15 |  |  |  |  |  |  |  |  |  |  |
| P800-16 |  |  |  |  |  |  |  |  |  |  |
| P800-17 |  |  |  |  |  |  |  |  |  |  |
| P800-18 |  |  |  |  |  |  |  |  |  |  |
| P800-19 |  |  |  |  |  |  |  |  |  |  |
| P800-20 |  |  |  |  |  |  |  |  |  |  |
| P800-21 |  |  |  |  |  |  |  |  |  |  |
| P800-22 |  |  |  |  |  |  |  |  |  |  |
| P800-23 |  |  |  |  |  |  |  |  |  |  |
| P800-24 |  |  |  |  |  |  |  |  |  |  |
| Total price |  |  |  |  |  |  |  |  |  | 180000 |

Notes:

All P.SUPPL800 experiments are rendered binaurally via stereo headphones.

Experiments P800-7 and P800-8, largely overlap with experiment P800-5. Depending on the desired depth of testing coverage, it is suggested these three experiments are reduced to one or two MASA 1 TC experiments.

Similarly, experiment P800-9 overlaps with experiment P800-6. It is suggested to review whether two OSBA experiments are needed. If so, the redundances should be addressed. If not, both experiments should be reviewed into a single experiment proposal.

Table 3: Allocation of MUSHRA experiments

| Exp | Objectif | Input format | Source material | Listening environment | LL 1 | LL 2 |
| --- | --- | --- | --- | --- | --- | --- |
| BS1534-1 |  | Stereo | Generic audio | Headphones |  |  |
| BS1534-2 |  | Stereo | Generic audio | Headphones |  |  |
| BS1534-3 |  | FOA | Generic audio | Headphones |  |  |
| BS1534-4 |  | FOA | Generic audio | Headphones |  |  |
| BS1534-5 |  | HOA3 | Generic audio | Headphones, 7.1 + 4 |  |  |
| BS1534-6 |  | 5.1 | Generic audio | Headphones, 5.1 |  |  |
| BS1534-7 |  | 5.1, 7.1 | Generic audio | Headphones |  |  |
| BS1534-8 |  | 5.1+2, 5.1+4 | Generic audio | Headphones |  |  |
| BS1534-9 |  | 7.1+4 | Generic audio | Headphones, 7.1 + 4 |  |  |
| BS1534-10 |  | 1-2 Object | Generic audio | Headphones |  |  |
| BS1534-11 |  | 3-4 Objects | Generic audio | Headphones |  |  |
| BS1534-12 |  | 3-4 Objects | Generic audio | Headphones |  |  |
| BS1534-13 |  | MASA 1 TC | Generic audio | Headphones |  |  |
| BS1534-14 |  | MASA 1 TC | Generic audio | Headphones |  |  |
| BS1534-15 |  | MASA 2 TC | Generic audio | Headphones |  |  |
| BS1534-16 |  | OSBA 1-4 Objects | Generic audio | Headphones |  |  |
| BS1534-17 |  | OSBA 1-4 Objects | Generic audio | Headphones |  |  |
| BS1534-18 |  | OMASA 1-4 Objects | Generic audio | Headphones |  |  |
| BS1534-19 |  | OSBA, OMASA | Speech+bckg |  |  |  |
| BS1534-20 | JBM/FER/Tan | FOA, MASA | Clean speech | Headphones |  |  |
| BS1534-21 |  |  |  |  |  |  |

Additionally, there were also several proposals for experiments using different testing methodologies, or where the methodologies were not clarified yet:

P.800 ACR experiment to compare different bandwidth

Room acoustics experiment

Experiment on 6 DoF and directivity.

Proposals

- Agree on the principle in Table 1 and integrate the agreement in Section 4 of the Test Plan. Update Annex F accordingly.

- Agree on the principle of external LLs running P.SUPPL800 experiments only. If agreeable, decide what to do with the remaining 12000 E.

- Review the volunteering LLs’ capabilities and, if updated, reflect the agreement in the Test Plan.

- Review the list of experiments in Section 7 and reflect the agreement in the Test Plan.

References

S4-240298 - Updated Proposal for IVAS Characterization Tests. Source: Fraunhoffer IIS.

S4-240367 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v.0.2.0.