## Source:	Fraunhofer IIS

## Title:	On the Importance of the Reference for IVAS Testing

## Document for:	Agreement

## Agenda Item:	7.5

# 1	Introduction

Recent discussions revealed that there is no clear understanding yet, whether IVAS codec candidates can be evaluated in a similar fashion as in the previous speech codec exercises, specifically, whether a reference will be part of the evaluation and thus candidates will be tested against this reference. The source sees no alternative to comparison to a reference, as outlined in the following.

# 2	Background

During the course of evaluation of the previous 3GPP communication codecs, a reference has always been available (the uncoded DIRECT signal), which is, as the input to the encoder, by definition the maximum quality signal during testing. The same concept is also applicable to IVAS, where the input signal is the reference and any IVAS candidate will aim for faithful reproduction of the input signal after encoding and decoding. The source notes that there is one complication in IVAS testing, which has already been observed for the VRStream audio profile selection process: some input signals cannot be directly listened to, and thus a rendering step is necessary. To overcome this for VRStream, both a common loudspeaker renderer (VBAP) and a common informative binaural renderer (CIBR) were defined, which allowed to evaluate the candidate codecs’ performance.

NOTE: There have been different interpretations how the reference would have to be rendered. It is envisioned that for IVAS a more explicit test and processing plan will be developed, avoiding such issues.

Contrary to this traditional way of evaluation, there seems to be the vague idea of evaluation without any reference. This would reduce the effort of testing but only provide little insight into a candidate’s performance. It thus seems unlikely that such procedure would serve 3GPP’s interest of high-quality standards that have been thoroughly tested.

Any IVAS solution shall aim for faithful reproduction of the input content. Only then the original intent can be preserved. If a reference would be omitted, test results become a matter of the listeners’ taste and the original intent could be compromised, potentially leading to effects such as a vastly different spatial image, which is especially important in case of audio-visual content, where the audio must match the visual content.

## 2.1	Evaluation and Reference condition for Stereo Content

Since the input signal can readily be evaluated over a stereo pair of loudspeakers or with dichotic headphone listening, the input signal can be directly used as the reference, thus the DIRECT signal is the reference.

Figure 1: Evaluation of Stereo content over loudspeakers

Such schema is also valid for Binaural Audio, but limited to listening over headphones.

## 2.2	Reference condition for Multi-Channel Content

Multi-Channel input signals can be directly evaluated over a loudspeaker setup with the configuration of the input signal, e.g. a 5.1 multi-channel input signal (CICP 6) can be directly evaluated with the corresponding 5.1 loudspeaker setup.

For listening over headphones, multi-channel audio needs to be rendered. Using the pass-through mode, a common renderer for the input signal and the decoded output signal can be defined, which eliminates rendering as a factor.

Using the built-in rendering, it becomes necessary to compare this rendering and the common rendering. Since different renderers may have different characteristics, e.g. timbre, localization, artefacts, etc. To ensure faithful rendering, an additional test would need to be run, e.g. similarly to the test described in TS 26.261 (Test 3 Renderer Test for VRStream).

Figure 2a: Evaluation of multi-channel content over loudspeakers

Figure 2b: Evaluation of multi-channel content over headphones

## 2.3	Evaluation and Reference condition for scene-based Content

Scene-based audio needs to be rendered to enable listening over loudspeakers or headphones. Using the pass-through mode, a common renderer for the input signal and the decoded output signal can be defined, which eliminates rendering as a factor.

Using the built-in rendering, it becomes necessary to compare this rendering and the common rendering. Since different renderers may have different characteristics, e.g. timbre, localization, artefacts, etc. To ensure faithful rendering, an additional test would need to be run, e.g. similarly to the test described in TS 26.261 (Test 3 Renderer Test for VRStream).

Figure 3a: Evaluation of scene-based content over loudspeakers

Figure 3b: Evaluation of scene-based content over headphones

## 2.4	Evaluation and Reference condition for Object-based Content

For the VRStream audio media profile selection a common object rendering scheme has been defined (VBAP). Ignoring object modification for the moment, a similar common rendering scheme could be employed for pass-through of objects, allowing comparison of the IVAS candidate coding with the input signal, eliminating the factor of rendering. Using the built-in rendering, a similar concept as for scene-based audio could be used.

Figure 4a: Evaluation of Object-based content over loudspeakers

Figure 4b: Evaluation of Object-based content over headphones

## 2.5	Evaluation and Reference condition for Mixed-format Content

The same concept as outlined in 2.3 and 2.4 can also be applied for mixed-format inputs, e.g. scene-based audio with additional objects.

# 3	Summary

This contribution outlined the risks associated with reference-less testing and provides example reference conditions for the proposed input formats. It is proposed to agree on the following:

- For input formats that can be readily evaluated (stereo contents, multi-channel contents over loudspeakers) the DIRECT signal is the reference condition

- For input formats that need to be rendered (e.g. scene-based audio, objects), the rendered DIRECT signal is the reference. The same renderer is also used for the IVAS processed file in pass-through mode. The definition of the renderer for evaluation (“common evaluation renderer”) is TBD.

- Evaluation of the IVAS Rendered files for binaural or loudspeaker listening can be performed against the files rendered with the renderer for evaluation (“common evaluation renderer”).

- Initiate a draft test plan permanent document, containing section 2.