3GPP TSG-WG SA4 Meeting #106	S4-191127

# Busan, KR, 21 – 25 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pseudo CR to TS 26.501 on DASH Ingest |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The stage 3 specification for the Ingest interface is specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR specifies the Ingest interface for 5GMSA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The content ingest for 5GMSA will be lacking. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[X]					DASH-IF, Specification of Live Media Ingest, https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

Second Change

## 4.2.1  DASH Ingest

#### 4.2.1.1	Overview

The DASH Ingest API offers external Media Application Servers (eMAS) access to the media distribution services offered by the network. The eMAS creates an Ingest and Distribution Configuration using the API and can start ingesting media into the assigned Media AS, which will then distribute it based on the created Distribution configuration. The API is designed to match the functionality of public CDNs.

#### 4.2.1.2		Control interface

##### 4.2.1.2.1	RESTful Resource

The API defines a main REST resource, the Distribution. The Distribution is defined in the following table:

| Name | Type | Description |
| --- | --- | --- |
| Origin | Object | Describes the origin that will be used to ingest the content for distribution. |
| Name | string | A name associated with this origin |
| Path | string | The relative path which will be used to address the media resources. This Path is provided by the Ingest receiver endpoint in the case of Push. |
| Pull | boolean | Indicates whether to use Pull or Push for ingesting the content. |
| Protocol | string | Identifier of the ingest protocol. The set of supported protocols are defined in section 5.4.2.3. |
| EntryPoint | string | An entry point to ingest the content. The semantics of the entry point are dependent on the selected Protocol. |
| Cache | [Object] | Defines the Cache configuration for the content of this Distribution. |
| Pattern | string | A pattern that will be used to match the object’s URL to the Cache settings |
| OriginCacheHeaders | boolean | Indicates whether to apply the cache directives for the object at ingest to the distribution. This is only applicable if the Ingest protocol is HTTP. |
| CacheDirectives | Object | If the cache directives are not inherited from the origin, this provides instructions on how to cache the content that matches the pattern. |
| no-cache | boolean | If set to true, this indicates that the objects that match this pattern are not be cached. |
| HTTP-Method-Filter | [string] | The methods to which the Cache rules apply |
| Status-Code-Filter | [integer] | The status codes to which the Cache rules apply |
| CDN_TTL | [integer] | The TTL value to set to the objects that match the pattern and the other filters. |
| Configuration | Object | Specifies the distribution method and configuration for the ingested content |
| ContentPreparation | Object | Defines the requested content preparation. A set of templates for content preparation may be provided for selection. |
| TargetDomain | string | The target domain name, which will be used to access the content from the CDN. This will replace the domain name of the |
| OriginDomain | string | The origin domain that is redirected to the target domain. This domain name can be exposed to the UEs instead of the target domain. |
| GeoFencing | Object | Limit access to the content to the indicated geographic areas. |
| URLSignature | Object | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content. |
| tokenName | string | The name of the query parameter that carries the token. |
| passphraseName | string | The name of the query parameter that carries the passphrase. |
| passphrase | string | The shared secret between the eMAS and the MAS for this Distribution. |
| ttlName | string | The name of the query parameter that carries the TTL field. |
| UseIPAddress | boolean | If set to true, the IP address of the UE is matched to the IP address in the signature. Access to the object is only allowed when they match. |
| ipAddressName | string | The name of the query parameter that carries the IP address of the client. |
| Certificate | Object | When content is distributed using TLS, the certificate for the Origin Domain is shared with the MAS. |

##### 4.2.1.2.2 	RESTful Operations

The Ingest and Distribution API is accessible through this URL paths:

/v1/distribution/distribution_ID and /v1/distributions

The following operations and the corresponding HTTP methods are supported:

| Operation | HTTP Method | Description |
| --- | --- | --- |
| Create a new Distribution | POST | This is used to create a new Distribution resource. |
| Fetch Distribution Configuration | GET | This operation is used to read the distribution configuration. |
| Update Distribution | PUT | This operation is used to update the distribution configuration. |
| Delete Distribution | DELETE | This operation is used to delete an existing Distribution. |
| Purge Cache | POST | This operation is used to invalidate some or all cached objects of this Distribution. |
| Create a new Content Preparation Template | POST | This operation is used to create a new Content Preparation Template that defines how the ingested content is to be prepared for distribution. |
| Create a new Certificate | POST | This operation is used to add a new certificate that is used for the distribution of the content of a Distribution using TLS. |

#### 4.2.1.3	Media Ingest Interface

This specification defines a single DASH Ingest protocol, the “dash-if-ingest” as specified by [8]. The Pull flag of the Origin element shall be set to false, indicating that a Push-based protocol is used. The entry point element shall be set to the URL that will be used to upload the DASH segments and MPD. The entry point URL shall not contain a path. The path for the URL is provided by the Path element.