3GPP TSG-SA4 Meeting #106                                                                           S4-191143

Busan, South Korea, Oct 21-25, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | <CR#> | rev | <Rev#> | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VR QoE Metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI, InterDigital Communications, Inc., Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2019-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With approval of the work item VRQoE, definition of new QoE metrics for VR services is necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal of new metric partly based on MPEG-I Part 6 WD metrics proposal |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================== CHANGE 1  ==============================

### 9.3.2 Comparable quality viewport switching latency

The comparable quality viewport switching latency metric reports the latency and the quality-related factors when viewport movements causes quality degradations, such as when low-quality background content is briefly shown before the normal higher-quality is restored.

The viewport quality is represented by two factors; the average quality ranking (QR) value (see clause X.2), and the effective pixel resolution of the viewport (see clause X.3). The resolution is defined by the orig_width and orig_height values in ISO/IEC 23090-2 in Spherical-Region Quality Ranking or 2-Dimensional Quality Ranking. The resolution corresponds to the monoscopic projected picture from which the packed region covering the viewport is extracted.

If more than one quality ranking region is visible inside the viewport, the viewport quality factors are calculated as the area-weighted average for QR and the area-weighted (effective) pixel resolution, respectively. For instance, if 60% of the viewport is from a region with QR=1, Res=3840 x 2160, and 40% is from a region with QR=2, Res=960 x 540, then the average QR is 0.6 x 1 + 0.4 x 2, and the effective pixel resolution is 0.6 x 3840 x 2160 + 0.4 x 960 x 540 (also see Annex X for more examples).

The client shall continously evaluate the average QR and the effective pixel resolution for the current viewport at regular intervals (for instance every X milliseconds, or every X degrees). If the current viewport includes at least one new quality ranking region (i.e. a quality ranking region not included in the previous viewport), a switch event is started. The last evaluated viewport quality before the switch is taken as the initial viewport quality, and quality factors related to that viewport is assigned to the firstViewport log entry. The start time of the switch is also set to the time of the last evaluated viewport before the switch.

The end time for the switch is defined as the first evaluated viewport after the start time, where  both the average QR and the effective resolution for the viewport is less than Y% worse than the first viewport values, or until a timeout of N milliseconds occurs (for instance when an adaptation to a lower bitrate occurs, and the viewport never reaches comparable quality). The evaluated viewport data is assigned to the secondViewport log entry, and the latency (end time minus start time) is assigned to the latency log entry. In case of a timeout, this is indicated under the cause log entry.

During the duration of the switch the worst evaluated viewport is also stored, and assigned to the worstViewport log entry.

If a new viewport switching event occurs (e.g. yet another new region becomes visible) before an ongoing switch event has ended, only the N milliseconds timeout is reset. The ongoing measurement process continues to evaluate the viewport quality until a comparable viewport quality value is achieved (or a timeout occurs).

The data type, ViewportDataType, is defined as shown in Table 1. ViewportDataType identifies the direction and coverage of the viewport.

Table 1 ViewportDataType

| Key |  | Type | Description |
| --- | --- | --- | --- |
| ViewportDataType |  | Object |  |
|  | viewpoint_id | Integer | Specifies the identifier of the viewpoint to which the viewport belongs. |
|  | centre_azimuth | Integer | Specifies the azimuth of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | centre_elevation | Integer | Specifies the elevation of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −90 * 216 to 90 * 216, inclusive. |
|  | centre_tilt | Integer | Specifies the tilt angle of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | azimuth_range | Integer | Specifies the azimuth range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |
|  | elevation_range | Integer | Specifies the elevation range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |

The data type Viewport-Item is defined as shown in Table 2. Viewport-Item is an Object which identifies a viewport and quality-related factors for the region(s) covered by the viewport.

Table 2. Viewport item

| Key |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- |
| Viewport-Item |  |  |  | Object |  |
|  | Position |  |  | ViewportDataType | Identifies the viewport |
|  | QualityLevels |  |  | List | List of different quality levels regions within the viewport |
|  |  | Coverage |  | Float | Percentage of the viewport area covered by this region |
|  |  | QR |  | Integer | Quality ranking (QR) value of this region |
|  |  | Resolution |  | Object | Resolution for this region |
|  |  |  | Width | Integer | Horizontal resolution for this region |
|  |  |  | Height | Integer | Vertical resolution for this region |

The comparable quality viewport switching latency metric is specified in Table 3.

Table 3 Comparable quality viewport switching latency metric

| Key |  |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- | --- |
| CQViewportSwitchingLatency |  |  |  |  | List | List of comparable quality viewport switching latencies |
|  | Entry |  |  |  | Object |  |
|  |  | firstViewport |  |  | Viewport-Item | Specifies information about the first viewport |
|  |  | secondViewport |  |  | Viewport-Item | Specifies information about the second viewport |
|  |  | worstViewport |  |  | Viewport-Item | Specifies information about the worst viewport seen during the switch duration |
|  |  | time |  |  | Real-Time | Wall-clock time when the switch started |
|  |  | Mtime |  |  | Media-Time | Media presentation time when the switch started. |
|  |  | Latency |  |  | Integer | Specifies the switching delay in milliseconds. |
|  |  | Cause |  |  | List | Specifies a list of possible causes for the latency. |
|  |  |  | Entry |  | Object |  |
|  |  |  |  | code | Enum | A possible cause for the latency. The value is equal to one of the following: - 0: Segment duration - 1: Buffer fullness - 2: Availability of comparable quality segment - 3: Timeout |

=========================== END OF CHANGE 1  ==========================

============================== CHANGE 2  ==============================

# X. Examples for comparable quality viewport switching latency

## X.1 Introduction

This sub-clause illustrates how the viewport quality and effective resolution are calculated.

## X.2 Viewport quality

The quality level of each region is determined with its respective quality ranking (QR) value. A viewport can be covered with multiple regions. A quality level value for the viewport can be derived as weighted average of the QR values of the regions covering the viewport. The weight of each region is defined as the percentage of the viewport area covered by the corresponding region. The viewport quality level can be calculated by the following equation.

QualityLevelviewport=i=1NQRi×Coveragei/100

N: number of regions covering the viewport,

QR[i]: QR value of i-th quality ranking region

Coverage[i]: the viewport coverage value of i-th quality ranking region.

Figure X.1 – An example of a viewport covered by four quality ranking regions

Figure X.1 is an example of a viewport covered by four quality ranking regions. The quality of the viewport is equal to the weighted sum of the quality ranking value and coverage percentage value of each quality ranking region.

## X.3 Effective Viewport Resolution

The resolution of each region is determined by its respective width and height values in pixel which are available in the quality ranking box under the name orig_width and orig_height. Note that these values are already normalized to represent the full-sphere resolution you would get if the resolution of this region would be used for the full sphere.

The effective resolution for the viewport can be derived as the weighted average of the resolution of each region covering the viewport. The weight of each region is defined by the percentage of the viewport area covered by the corresponding region. The effective viewport resolution can be calculated by the following equation.

EffectiveResolutionviewport=i=1Nwidthi×heighti×Coveragei/100

N: number of regions covering the viewport,

width: width component of the original source resolution in pixels of ith Spherical or 2D region signaled in the Sphericial Region or 2D quality ranking,

height: height component of the original source resolution in pixels of ith Spherical or 2D region signaled in the Sphericial Region or 2D quality  region,

Coverage: the percentage value of the viewport area being covered by i-th Spherical or 2D region signaled in the Sphericial Region or 2D quality region.

Resolution [3]

Resolution [2]

Resolution [4]

Coverage [1]

Coverage [3]

Coverage [4]

Region [1]

Region [2]

Region [4]

Region [3]

Viewport

35%

15 %

40 %

10%

Coverage [1]

Coverage [2]

Coverage [3]

Coverage [4]

Coverage [2]

Resolution [1]

Figure X.2 (a)– An example of source packed image with four quality ranking 2D regions with different resolution

Figure X.2 (b)– An example of a viewport covered by four different quality ranking 2D regions

Figure X.2(a) is an example of a source with four regions with different resolution. Figure X.2.(b) represents an example of a viewport which is covered by the four quality ranking 2D regions. The effective viewport resolution is equal to the sum total of the resolution for each quality-ranking 2D region and its corresponding viewport coverage percentage value.

## X.4. Comparable quality viewport switching latency measurement

An example of viewport switching is shown in figure X.3.

Figure X.3 – Comparable quality viewport switching latency measurement example

Figure X.3 presents an example of the metric measurement operation. The viewport quality is evaluated at time t0, and then again at time t1. The media playback module renders the high-resolution sub-picture #1 at time t1. The user viewing orientation is gradually changing from sub-pic#1 to sub-pic#2 as the time progresses.

At time t2, the media playback module starts to render the buffered low-quality representation of sub-pic#2 as the viewport moves into sub-picture #2. At time t2, the viewport quality drops in values as compared to the viewport quality at time t1, and a new sub-picutre (sub-pic #2) is rendered. A viewport switching event is identified at time t2.

The viewport quality values evaluated at t1 identifies the first viewport. The viewport position and viewport quality level list are assigned to the attributed Position and QualityLevel of the firstViewportItem.

An effective viewport resolution and viewport QR quality value for the new viewport that is comparable to that of the firstViewportItem after viewport switching time is logged at time t4.  The new viewport position identifies the Position of the secondViewportItem. The corresponding QualityLevel list for the secondViewportItem is assigned.

The associated viewport values stored for the worst viewport quality during the switch is assigned to the field Position of the worstViewportItem. The corresponding QualityLevel list for the worstViewportItem is also assigned.

The comparable-quality viewport switching latency is measured as the time interval between the logged times for firstViewportItem (t1 in this example) and secondViewportItem (t4 in this example).

=========================== END OF CHANGE 2  ==========================