3GPP TSG-SA4 Meeting #106 	S4-191149

# Busan, Republic of Korea, Oct 21 - Oct 25, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | xxx | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Amended QoS Signaling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Samsung Electronics Co., Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_FLUS |  |  |  |  |  | Date: |  |  | 2019-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current 3GPP resource reservation mechanisms rely on identification of application and media type, making a 1:1 QoS mapping based on that. In the MTSI instantiation of FLUS, the application would still be MTSI with an additional FLUS media indication ("a=label:flus") keeping separation per media type as before, but the 1:1 mapping would no longer be possible since there are at least the 5 defined live uplink streaming 5QIs/QCIs in TS TS 23.501 (Table 5.7.4-1) and 23.203 (Table 6.1.7-A) to choose from. It is assumed that it would neither be desirable nor sufficient for an operator to always choose a single one of the 5 FLUS 5QIs/QCIs for a certain media for all FLUS applications, since that would be suitable only for a fraction of the possible FLUS use cases, and sub-optimal at best for the other FLUS use cases. It is also assumed that it would be infeasible to define new, separate (non-MTSI) applications for the different FLUS use cases to be able to keep the 1:1 mapping approach between 5QI/QCI and the {application, media type} tuple. A method to provide the necessary detailing of QoS signaling, using a new "a=3gpp-qos-hint" SDP attribute, is defined in a related CR to TS 26.114. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Recommending use of "a=3gpp-qos-hint" SDP attribute for MTSI instantiations of FLUS media where QoS is required, adding a reference to its definition in TS 26.114 clause 6.2.7.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to apply appropriate QoS for MTSI instantiation of FLUS media. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2, 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-AHM497 Initial draft S4-191149 2nd draft |  |  |  |  |  |  |  |  |

First Change

### 4.5.2	General source systems

A source system provides a collection of media streams that can be combined by the FLUS sink to create a full media experience. FLUS provides the ability to stream continuous media data from a source to a sink.

Media streams within one source system are time-synchronized and self-descriptive in the source system. Media streams are identified by a unique identifier in a source system.

Each media stream shall be self-declarative in the announced system.

Media streams may be of the following types: video, audio, text or metadata.

A source system may declare additional metadata in the namespace of the source system.

Table 4.5-1: General source system description

| Source System |  |  |  | Description | IMS-based Instantiation SDP Parameters | Non-IMS-based Instantiation Parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  | Source System Identifier |  |  | A URI that uniquely identifies the source system | a=3gpp-flus-system:<urn> | JSON Object (SourceSystemIdentifier) |
|  | FLUS media stream to session mapping |  |  | Grouping description needed to identify the FLUS media streams when a FLUS session is part of an MTSI session | a=group:FLUS <mid_1> <mid_2>... | Not applicable |
|  | Configuration |  |  | Provides source system-specific configuration parameters for the source system. | a=3gpp-flus-configuration:<base 64 encoded> | Not applicable |
|  | Media Stream |  |  | Descriptions of the media streams that are defined by the source system | m= (one per stream) | Defined by the F-U instantiation |
|  |  | Identifier |  | A unique identifier shall be associated with every media stream. If not present, then the source system description should contain sufficient information to uniquely identify the media stream in the system. | a=mid:<media_id> | Defined by the F-U instantiation (may be implicit) |
|  |  | Stream Configuration |  | Describes the details of the media stream, including the encoding, the metadata, etc. The media stream is self-describing in the system. The definition of a single media stream, in the context of session description information, is provided during session establishment, e.g. by a media line in SDP. | a=3gpp-flus-media-configuration:<base 64 encoded> | Defined by the F-U instantiation |
|  |  | Bandwidth Requirement |  | Indication of the required network bandwidth to transport the media streams of the source system. Mandatory if QoS is required | b=AS:<bw> a=bw-info: | Defined by the F-U instantiation |
|  |  | QoS Requirement |  | Indication of the required network QoS parameters to transport the media streams of the source system. Recommended if specific loss and/or latency are required | a=3gpp-qos-hint: | Defined by the F-U instantiation |
|  |  | Transmission Direction |  | Optional indication of the transmission direction of the media streams | a=sendonly (mandatory) | Defined by the F-U instantiation |
|  |  | Codec |  | Codec Identifier | a=rtpmap: (mandatory) | Defined by F-U instantiation. |
|  |  | Codec configuration |  | Codec-specific configuration information for the set of media streams in the source system | a=fmtp: | Defined by F-U instantiation |
|  |  |  | Media Type | Media type of each media stream: audio, video, text, etc... | m=<type> (mandatory) | Defined by F-U instantiation |
|  |  |  | Media Transport and Control | Identification of the media transport protocol, operating over IP, for this media stream in the source system | m=<type> <port> RTP/AVP (mandatory) RTP/AVPF (if video rate adaptation is desired) | Defined by F-U instantiation. |

Next Change

### 5.2.2	Session management

The IMS-based FLUS System uses the SIP protocol for all session management. The FLUS Sink and FLUS Source shall support the IMS procedures as defined by TS 24.229 [12].

Use of the "a=3gpp-qos-hint:" SDP attribute, as described by TS 26.114 [4] clause 6.2.7.x, is recommended for FLUS media descriptions if specific QoS in the form of maximum loss and/or latency is required.

End of document