3GPP TSG-SA4 Meeting #106 	S4-191150

# Busan, Republic of Korea, Oct 21 - Oct 25, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | 0011 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of FLUS Media Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Samsung Electronics Co., Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 2019-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current method used in clause 4.5.2 to distinguish FLUS media in SDP for MTSI-based FLUS is “a=group:FLUS” together with per-media “a=mid” identification, meaning that any media that is listed as part of that FLUS group is a FLUS-specific media. However, defining new "a=group" grouping semantics in addition to the ones in RFC 5888 section 5 formally requires “Standards Action” (see RFC 5226 section 4.1 for term definition), which effectively requires to create a new IESG-approved Standards Track RFC for the “FLUS” grouping. Such RFC is missing and the current text is silent on that and seems to incorrectly assume that a simple IANA registration is enough. While it is clearly possible to create a new RFC to describe the new “FLUS” grouping semantics and such document would hardly be more than a few pages, that would require more administrative effort and be much more time-consuming than what is warranted in this case. A different FLUS media identification method than “a=group” would therefore be preferable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changing the method of indicating FLUS media in SDP from "a=group" to "a=label", which does not require any IETF action but RFC 4574 requires defining SDP offer/answer separately for each specific use of "a=label", here provided in clause 5.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A new standards track IETF RFC describing FLUS grouping semantics with "a=group" will have to be created. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5.2, 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-AHM496 Initial draft S4-191150 Initial version |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R BS.2051-1 (06/2017): "Advanced Sound System for Programme Production".

[3]	ISO/IEC 23090-2:2019: "Information technology - Coded representation of immersive media - Part 2: Omnidirectional media format".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[5]	3GPP TS 26.235: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[6]	3GPP TS 24.147: "Conferencing using the IM Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[9]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[10]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[11]	IETF RFC 7231: "Hypertext transfer protocol (HTTP/1.1): Semantics and Content".

[12]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	3GPP TS 23.682: " Architecture enhancements to facilitate communications with packet data networks and applications".

[16]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[17]	ISO/IEC CD 23090-8: "Information technology -- Coded representation of immersive media -- Part 8: Interfaces for network media processing" (under development).

[18]	IETF RFC 4574: "The Session Description Protocol (SDP) Label Attribute".

Next Change

### 4.5.2	General source systems

A source system provides a collection of media streams that can be combined by the FLUS sink to create a full media experience. FLUS provides the ability to stream continuous media data from a source to a sink.

Media streams within one source system are time-synchronized and self-descriptive in the source system. Media streams are identified by a unique identifier in a source system.

Each media stream shall be self-declarative in the announced system.

Media streams may be of the following types: video, audio, text or metadata.

A source system may declare additional metadata in the namespace of the source system.

Table 4.5-1: General source system description

| Source System |  |  |  | Description | IMS-based Instantiation SDP Parameters | Non-IMS-based Instantiation Parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  | Source System Identifier |  |  | A URI that uniquely identifies the source system | a=3gpp-flus-system:<urn> | JSON Object (SourceSystemIdentifier) |
|  | Configuration |  |  | Provides source system-specific configuration parameters for the source system. | a=3gpp-flus-configuration:<base 64 encoded> | Not applicable |
|  | Media Stream |  |  | Descriptions of the media streams that are defined by the source system | m= (one per stream) | Defined by the F-U instantiation |
|  |  | Identifier |  | A unique identifier shall be associated with every FLUS media stream. If not present, then the source system description should contain sufficient information to uniquely identify the media stream in the system. | a=label:flus (RTP SSRC being the unique identifier, may be implicit) | Defined by the F-U instantiation (may be implicit) |
|  |  | Stream Configuration |  | Describes the details of the media stream, including the encoding, the metadata, etc. The media stream is self-describing in the system. The definition of a single media stream, in the context of session description information, is provided during session establishment, e.g. by a media line in SDP. | a=3gpp-flus-media-configuration:<base 64 encoded> | Defined by the F-U instantiation |
|  |  | Bandwidth Requirement |  | Indication of the required network bandwidth to transport the media streams of the source system. Mandatory if QoS is required | b=AS:<bw> a=bw-info: | Defined by the F-U instantiation |
|  |  | Transmission Direction |  | Optional indication of the transmission direction of the media streams | a=sendonly (mandatory) | Defined by the F-U instantiation |
|  |  | Codec |  | Codec Identifier | a=rtpmap: (mandatory) | Defined by F-U instantiation. |
|  |  | Codec configuration |  | Codec-specific configuration information for the set of media streams in the source system | a=fmtp: | Defined by F-U instantiation |
|  |  |  | Media Type | Media type of each media stream: audio, video, text, etc... | m=<type> (mandatory) | Defined by F-U instantiation |
|  |  |  | Media Transport and Control | Identification of the media transport protocol, operating over IP, for this media stream in the source system | m=<type> <port> RTP/AVP (mandatory) RTP/AVPF (if video rate adaptation is desired) | Defined by F-U instantiation. |

Next Change

### 5.2.2	Session management

The IMS-based FLUS System uses the SIP protocol for all session management. The FLUS Sink and FLUS Source shall support the IMS procedures as defined by TS 24.229 [12].

In addition, the following SDP offer/answer procedures apply for "a=label:flus" (see also [18]):

- Generating the SDP offer:

- "a=label:flus" shall be included in every media description used for FLUS media in the SDP offer.

- Generating the SDP answer:

- "a=label:flus" shall be kept in every media description in the SDP answer where "a=label:flus" was received in the SDP offer and that is accepted by the SDP answerer to be used for FLUS media.

- If "a=label:flus" is not included in a media description in the received SDP offer, "a=label:flus" shall not be included in the corresponding media description in the SDP answer.

- Receiving the SDP answer:

- If an accepted SDP media description in the received SDP answer contains "a=label:flus", and if "a=label:flus" was also included in the corresponding media description in the SDP offer, the media description shall be considered as negotiated for use with FLUS media.

- If an accepted SDP media description in the received SDP answer does not contain "a=label:flus", and if "a=label:flus" was included in the corresponding media description in the SDP offer, the SDP answerer likely does not know of FLUS media and the SDP offerer should send an SDP re-offer with this media description disabled (port set to 0).

- If an accepted SDP media description in the received SDP answer contains "a=label:flus", and if "a=label:flus" was not included in the corresponding media description in the SDP offer, the SDP answerer is using FLUS media "a=label:flus" incorrectly and the SDP offerer shall send an SDP re-offer with this media description disabled (port set to 0).

End of document