Source:	Nokia Corporation

Title:	On voice and priority information for IVAS audio inputs

Document for:	Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

The source previously presented for consideration real-time conversational use cases and discussion relating to IVAS object-based audio input control metadata [1]. The source considers it necessary to specify sufficient control metadata for audio input streams to enable consistent experiences for new immersive services targeted by the IVAS codec. In [2], the source therefore further discussed control metadata requirements for IVAS audio input formats and combinations of them. It was proposed to define IVAS support for a minimum set on control metadata spanning all codec audio inputs.

In this input contribution, we provide further motivation and use cases for the voice signal input flag and audio input priority information that were proposed in [2]. It is furthermore proposed IVAS design constraints to accommodate for the two types of audio input stream information.

# 2. Definitions

Voice signal input

The voice signal input information can be a flag that is optionally activated for at least one of the input audio formats in case of any combination of audio input streams. When this flag is active, the IVAS encoder will know which input audio stream or streams carry the main communications signals. The purpose of the flag on the encoder-side can be seen to allow for prioritization or any mode selection relating to the audio input streams. This information is also passed to the decoder if more than one input audio stream is transmitted.

As this is an optional control metadata, it would be allowed to provide an audio input with no active voice signal input flag even if at least one of the audio streams actually carries voice.

Alternatively, the voice signal input information could also be defined for individual channels of multi-channel inputs. For example, for a regular (non-binaural) stereo input it could be indicated whether just one of the channels (L/R) carries the user voice. In this discussion, however, it is not considered the channel-level flag further.

Audio input priority

Audio input priority information can be defined, e.g., as a numeric value for each input providing a “ranking of importance” across the audio input streams. The priority information can be used in the IVAS encoder, e.g., to determine which audio input streams can be combined during encoding and which are to be maintained separable. The degree to which such combining (downmixing) is performed may depend on the number of priority levels present in the audio input priority information. It is not required to transmit the priority information to the decoder/renderer, although there may be some use cases where that could be useful.

# 3. Use case motivation

Voice signal input

The immersive audio use cases hold a lot of promise for increased user control of the listening experience. However, such control needs to be simple and intuitive for any significant adoption by users. Little control is achieved in practice if it is not clear for the listener what they are capable of controlling and how. Thus, while we do not know what the user interfaces of the future will exactly incorporate, we need to make sure the most obvious audio rendering controls are made possible using the IVAS codec.

While encoder prioritization for the voice signal can in many cases be achieved equally well using the audio input priority information, the voice signal input information is required also at the decoder. This is understood by considering the aspects that a listener needs to be able to control for a personalized audio presentation in different communications scenarios and in different real-world surroundings.

In case of stereo playback of a spatial audio call it can be advantageous for the user to be able to set a caller position (or balance) between left and right channels. From the IVAS codec point of view, this is possible as long as the voice is transmitted separate from any other components. However, the rendering device needs to understand this. For example, the decoded bitstream may include two audio objects. The rendering device thus needs a way to communicate to the user there is a voice signal and also know which one of the two it is. If both are voice signals, the user interface can adapt to this information. Instead of providing controls, e.g., for voice channel choice, it can be provided, e.g., volume control for caller 1 and caller 2. Information about caller 1 being “John” and caller 2 being “Mary” is not relevant for the transmission; information about there being a specific voice channel is important.

Figure 1 provides an example user interface that allows the listener to adapt the playback of a spatial audio call. Since the IVAS codec may transmit many different immersive audio configurations, the rendering device user interface needs to be able to adapt to the current configuration. For example, here we may consider a combination of input formats where it is transmitted at least one audio object (carrying the user voice signal) and spatial audio (e.g., ambisonics or MASA) that provides mainly the surrounding audio scene. For simplicity, we call it here the ambience.

The example UI features a slider for adjusting the level of ambience playback. The listener may in real life be, e.g., switching between noisy and more quiet environments and therefore wishes to adjust the speech volume relative to the ambience. Various alternatives for that control exist, however we believe they all require knowledge of the voice signal stream. Other controls in this example relate to the spatial positions of scene components. For example, it may be provided the listener overall control of the scene orientation (irrespective of any head-tracking capability). In addition, it may be provided ability to select the position of the caller. This again requires information about which stream actually is the user voice.

Figure 1. Example user interface elements for immersive audio call on IVAS device.

Audio input priority

Audio input priority information can be obtained from various sources. For example, user interface on a mobile device may allow the user to place certain elements in their audio scene. Such elements may include, e.g., the user voice, a sound clip, background music, etc. The user may also wish to prioritize the elements. From the user viewpoint such prioritization may relate, e.g., to the ability of the listener to affect the playback of the audio scene. Another example of the audio input priority source is a conferencing system that may provide individual talkers as audio objects. Each participant may be given a priority that can relate to their role or affiliation or any other compelling criteria for the teleconference. For example, a moderator or chair may be given higher priority than other participants.

One aspect under discussion for the IVAS codec design constraints is the so-called pass-through operation. According to pass-through operation, the decoder provides an output corresponding to the encoder input formats. Thus, in pass-through operation an audio input of, e.g., ambisonics and two audio objects would result in decoding of ambisonics and two audio objects. This would allow, e.g., manipulation of the two audio objects relative to the ambience signal. For example, we may consider the user interface example in Figure 1.

On the encoder side, the audio input priority allows to determine which audio inputs should be provided separately (e.g., when bitrate does not allow for separate transmission of all audio inputs). In addition, audio input priority may be used to determine which of the active talkers gets through in case the IVAS encoder prioritizes teleconference participants.

Audio input priority can also be used on the decoder side. For example, a mobile device user interface for audio object manipulation may be populated based on the priority information. For a teleconferencing scenario it may be that the IVAS encoder transmits all inputs it is provided. In other words, any removal of participant voices based on prioritization would be carried out prior to IVAS encoder. In this case, the prioritization information may determine which talkers are activated in the presentation by default. For example, a listener may then be able to add or switch talkers.

# 4. Summary and proposal

The source considers it very important to define some control metadata particularly for combinations of encoder input audio formats. In the current document, we discussed indication of voice signal input and audio input priority information.

The two control metadata types are needed especially if pass-through operation is not required for IVAS. When pass-through operation is not used, user control of audio objects and spatial audio in the rendering generally cannot be guaranteed.

Even in case of pass-through operation, it remains useful to indicate certain input properties, e.g., for advanced spatial audio rendering user interfaces. In general, it should be understood that simplicity and adaptability of the user interfaces will be of high importance in practical user adoption of features and capabilities that are foreseen for the IVAS codec use cases.

We propose to agree the following for IVAS-4:

- The IVAS codec shall support transmission of voice input indication.

- The IVAS codec shall support prioritization information for audio inputs.

Both proposed requirements are intended as optional information, when IVAS encoder receives more than one audio input. Thus, it is not mandatory to provide the information for all sessions. It is the capability that is considered mandatory.

# References

[1]	Tdoc S4-190396: “Examples of object-based audio control metadata for IVAS”. Source: Nokia Corporation

[2]	Tdoc S4-190938: “On shared control metadata for IVAS input audio formats”. Source: Nokia Corporation