| 3GPP TS 26.139 V0.0.1 (2019-10) |  |  |
| --- | --- | --- |
| Technical Specification |  |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; RTP/RTCP Verification Procedures (Release 16) |  |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	8

5	Test Architectures	9

5.1	General	9

6	Verification Tests	9

6.1	General	9

6.2	RTCP Tests	10

6.2.1	General	10

6.2.2	Basic RTCP Tests	10

6.2.2.1	Initial Sending No Data RTCP Test	10

6.2.2.2	Initial Receiving No Data RTCP Test	11

6.2.2.3	Sending Data RTCP Test	11

6.2.2.4	Mid-Session Sending No Data RTCP Test	11

6.2.2.5	Mid-Session Receiving No Data RTCP Test	11

6.2.2.6	Compound RTCP Packet Format Test	12

6.2.2.7	RTCP Report Count Test	12

6.2.3	RTCP Bandwidth and Transmission Timing Tests	13

6.2.3.1	RTCP Disable Test	13

6.2.3.2	RTCP Basic Interval Test	13

6.2.3.3	RTCP Low Bandwidth Test	13

6.2.3.4	RTCP High Bandwidth Test	14

6.2.3.5	RTCP Explicit Bandwidth Test	15

6.2.3.6	RTCP Timeout Test	15

6.2.3.7	RTCP Step Join Backoff Receiver Test	16

6.2.3.8	RTCP Step Join Backoff Sender Test	16

6.2.4	Sender Report Sender Info Tests	16

6.2.4.1	Sender SSRC Test	16

6.2.4.2	NTP Test	16

6.2.4.3	Wrapped NTP Test	17

6.2.4.4	Time Stamp Test	18

6.2.4.5	Wrapped Time Stamp Test	18

6.2.4.6	Packet Count Test	19

6.2.4.7	Wrapped Packet Count Test	19

6.2.4.8	Octet Count Test	19

6.2.4.9	Wrapped Octet Count Test	19

6.2.5	Source Description (SDES) Tests	19

6.2.5.1	Basic SDES Test	19

6.2.5.2	Canonical End-point Identifier (CNAME) Test	20

6.2.6	Receiver Report Block Tests	20

6.2.6.1	SSRC Consistency Test	20

6.2.6.2	SSRC Completeness Test	20

6.2.6.3	SSRC Change Test	21

6.2.6.4	Initial Zero Loss Test	21

6.2.6.5	Zero Loss Test	21

6.2.6.6	Loss Test	21

6.2.6.7	Wrapped Loss Test	22

6.2.6.8	Duplicate Loss Test	22

6.2.6.9	All Loss Test	22

6.2.6.10	Extended Highest Sequence Number Received Test	22

6.2.6.11	Wrapped Extended Highest Sequence Number Received Test	22

6.2.6.12	Interarrival Jitter Test	22

6.2.6.13	Duplicate Interarrival Jitter Test	23

6.2.6.14	Last Sender Report Timestamp Test	23

6.2.6.15	Delay Since Last SR Test	23

6.2.7	BYE Tests	23

6.2.8	Feedback Report Block Tests	23

6.2.8.1	General	23

6.2.8.2	AVPF Transmission Timing Tests	23

6.2.9	Extended Report Block Tests	23

6.2.9.1	General	23

6.2.10	APP Tests	23

6.2.11	Reduced-Size Packet Tests	23

6.3	RTP Tests	24

6.3.1	General	24

6.3.2	RTP Bandwidth and Transmission Timing Tests	24

6.3.3	RTP Header Extension Tests	24

6.3.4	RTP Contributing Source Tests	24

6.3.5	RTP Voice Tests	24

6.3.6	RTP Video Tests	24

6.3.7	RTP Text Tests	24

6.4	SRTCP Tests	25

6.4.1	General	25

6.5	SRTP Tests	25

6.5.1	General	25

6.6	RTP Translator Tests	25

6.7	RTP Mixer Tests	25

Abstract Test Suite (ATS) text block	25

Y	Abstract Test Suite (ATS)	25

Y.1	Introduction	25

Y.2	The TTCN Graphical form (TTCN.GR)	25

Y.3	The TTCN Machine Processable form (TTCN.MP)	26

Annex <A> (normative): <Normative annex for a Technical Specification>	27

Annex <B> (informative): <Informative annex for a Technical Specification>	28

B.1	Heading levels in an annex	28

Annex <C> (informative): Bibliography	29

Annex <X> (informative): Change history	30

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

[Editor’s note: What (3GPP) entities and services the document targets. Based on text from the WID.]

The present document describes:

-	Test cases needed to ensure an adequate level of RTP operation and RTP stream monitoring.

-	Test methods capable to verify that information contained in the RTP header and in RTCP is correct and consistent with the observed characteristics of the related RTP streams:

- Between RTP/RTCP within the scope of a single RTP stream (e.g. between an RTP stream and the corresponding RTCP reporting from the remote party, or between an RTP stream and the corresponding RTCP metadata, e.g for sampling clock accuracy compensation between RTP sender and RTP receiver).

- Between RTP/RTCP across RTP streams in the same RTP session (e.g. between sent and received RTP streams, or between audio RTP streams and video RTP streams).

-	Requirements on what constitutes acceptable RTP/RTCP protocol field values, including RTP payload header and RTP payload length, based on the observed characteristics of the related RTP streams.

-	A method for an RTP/RTCP implementation to announce on the network that it has passed the necessary tests and conforms to the new specification.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications".

[3]	IETF RFC 3158 (2001): "RTP Testing Strategies".

[4]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control".

[5]	IETF RFC 3711 (2004): "The Secure Real-time Transport Protocol (SRTP)".

[6]	IETF RFC 3556 (2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth".

[7]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP) – Based Feedback (RTP/AVPF)".

[8]	IETF RFC 5506 (2009): "Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences".

[9]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	Background

[Editor’s note: Based on text from the WID.]

Today’s 3GPP conversational and real-time services (e.g. MTSI [9] and mission critical services) all use the RTP protocol [2] or the Secure RTP protocol [5] on top of UDP/IP as media transport. (S)RTP has a companion control protocol, (S)RTCP (also in [2] and [5]), which is optional but is typically also used. In the rest of this specification, only the terms RTP/RTCP are used for brevity but should be understood to be equally applicable to SRTP/SRTCP if nothing else is said.

RTCP provides means to feedback statistical characteristics of a received RTP stream from RTP receiver to RTP sender, and to carry RTP stream metadata from RTP sender to RTP receiver, both during an active RTP session and in hold conditions where no RTP is sent. While RTP/RTCP information is designed to be useful to an ongoing real-time media session, the increased focus on automation and the consequential need for service observability and automatic performance measurements also makes it convenient and common to use RTP header and RTCP as one of the information sources to monitor the RTP streams for automation purposes. This is a very straightforward approach since it was one of the very design targets for RTP/RTCP, and both RTCP and RTP are extensible and can optionally carry various types of information. Service observability is paramount to enable any tuning or optimization to achieve a well-functioning system.

However, RTCP information has little or no end-user impact on basic, single-media communication services such as a voice-only call. The reasons for this are mainly twofold;

1)	One of the intended usages of RTCP feedback reporting functionality is to allow the RTP sender to adapt its sending rate to available transport capacity since a non-acknowledged transport such as UDP/IP has no built-in congestion control, but most voice-only calls today are both low-rate and fixed-rate that has no use for adaptation, and

2)	One of the other intended usages of RTCP is to provide enough metadata information to allow close time synchronization of different RTP streams, such as e.g. voice and video for a video call, but a voice-only call is single-media and has no use for such time synchronization.

Therefore, there’s often no direct impact on the voice service performance if a UE or network-node implementation of the RTP stack includes incorrect (e.g. all-zero or random) data in RTCP for a voice-only call. RTCP information content only matters on service level when really making use of RTCP functionality such as for quality monitoring, somehow acting on varying transport characteristics (loss, delay, jitter), or when performing inter-media synchronization.

Also, while the call setup and modification protocol, SIP/SDP, is conformance tested in 3GPP scope (by TSG RAN WG5), no media-level tests are defined or performed there when this specification is written. RTP and RTCP are considered as media-level protocols in that conformance testing. The current overall level of RTCP implementation conformance and accuracy in 3GPP devices and in RTP/RTCP-terminating network nodes is therefore mostly unknown.

Since the information content in RTP/RTCP is often neither fundamental for the user-level experience nor explicitly tested today, there’s a risk that automation, performance tuning efforts, and the application using the RTP/RTCP stack will work with incorrect data, potentially making wrong decisions that could result in worse rather than better performance.

# 5	Test Architectures

[Editor’s note: Examples how to arrange one or two systems under test, RTP/RTCP data injection, and test probes, noting that other test architectures may be used. Describe as an annotated architecture figure. Highlight the general preconditions, e.g. that for all test descriptions it is implicitly assumed that systems under test are active and connected, point-to-point if not stated otherwise, and that data injection and data probes are attached and appropriately configured to run the test.]

## 5.1	General

TBD.

# 6	Verification Tests

[Editor’s note: Below sub-clause layout idea taken from IETF RFC 3158 [3]. Describe per test if, when, and what is mandatory regarding the test result; mandatory, conditionally mandatory, and optional. Consider if there should be another first-level clause defining how to measure different things, which can be referenced from this clause, as 26.131/132 are structured.]

## 6.1	General

All test descriptions in this clause use the following layout:

Status:	Mandatory / Conditionally Mandatory / Optional

[Editor’s note:	Proposals for a better name than “Status” is welcome. Does it make sense to have an optional test, and what does it then mean that it is optional? Should there just be "mandatory" and "conditional"?]

Preconditions:	Describes any preconditions that must be fulfilled to run the test.

Test procedure:	Describes the steps taken to run the test.

Stop condition:	Describes the conditions when the test is concluded, in time order if more than a single condition.

Pass criteria:	Describes the criteria that must all be met to pass the test.

Comments:	Any additional comments applicable to the test.

Mandatory tests are required to pass for a system under test.

Conditionally Mandatory tests are mandatory to pass if the functionality is supported/implemented by the system under test, can be negotiated, and is correctly negotiated to be used during the test, the method of negotiation being described as part of the Preconditions section (see above).

Optional tests are optional to execute and optional to pass for a system under test.

## 6.2	RTCP Tests

### 6.2.1	General

This clause describes tests that are applicable to all types of RTCP and SRTCP packets. If not explicitly stated otherwise, any reference to RTCP is equally applicable to SRTCP. Tests that are only applicable to SRTCP and not to RTCP are specified in clause 6.4.

If not explicitly stated otherwise, all tests in this clause assume that the system under test is set to use:

1)	AVP profile [4] for RTCP tests.

2)	SAVP profile [5] for SRTCP tests.

3)	Non-zero RTP session bandwidth.

4)	Non-zero RTCP bandwidth.

5)	A minimum RTCP report interval no less than 2 seconds.

6)	A maximum RTCP report interval no longer than 7 seconds.

7)	An average RTCP report interval in the range 4.5 – 5.5 seconds.

NOTE 1:	The minimum, maximum, and average RTCP report intervals are not static parameters for an RTP/RTCP implementation but, as described in RFC 3550 [2], depend on conditions that vary with e.g. RTP session bandwidth, RTCP bandwidth, average size of RTCP packets, number of RTP session participants, and use of optional RTP/RTCP features such as RTCP feedback [7] and/or non-compound RTCP packets [8].

NOTE 2:	The method to set RTCP report interval in the system under test can vary. The RTCP report interval can in some cases be set directly, if the system under test has appropriate means to do so. The RTCP report interval can alternatively be set indirectly by e.g. setting an appropriate RTCP bandwidth in combination with pre-knowledge about the expected average RTCP packet size. If it is not possible to set RTCP bandwidth directly, it could be possible to set an appropriate RTP session bandwidth and rely on RTCP bandwidth becoming the default 5% of that RTP session bandwidth.

### 6.2.2	Basic RTCP Tests

#### 6.2.2.1	Initial Sending No Data RTCP Test

Status:	Conditionally Mandatory

Preconditions:	1) The system under test can set explicit RTCP bandwidth
2) The system under test is pre-configured to not send any RTP packets from the start of the test session, e.g. set as receive-only.

Test procedure:	Observe system under test output.

Stop condition:	RTCP packets are sent from system under test, or time has passed corresponding to three maximum RTCP intervals (see clause 6.2.1, ca 21 seconds).

Pass criteria:	1) No RTP packets are sent.
2) RTCP packets are sent containing at least a Sender Report or Receiver Report.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.2.2	Initial Receiving No Data RTCP Test

Status:	Mandatory

Preconditions:	The system under test is pre-configured to not receive any RTP packets from the start of the test session, e.g. set as send-only.

Test procedure:	Observe system under test output.

Stop condition:	RTCP packets are sent from system under test, or time has passed corresponding to three maximum RTCP intervals (see clause 6.2.1, ca 21 seconds).

Pass criteria:	1) No RTP packets are received.
2) RTCP packets are sent containing at least a Sender Report or Receiver Report, with an empty Receiver Report block, indicated by RC=0.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.2.3	Sending Data RTCP Test

Status:	Mandatory

Preconditions:	The system under test is pre-configured to send RTP packets during the test session, e.g. set as send-receive or send-only.

Test procedure:	Observe system under test output.

Stop condition:	1) One or more RTP packets are sent.
2) An RTCP packet is sent.

Pass criteria:	1) RTP packets are sent.
2) At least one RTCP packet is sent containing at least a Sender Report.
3) The Sender Report contains nonzero Sender Info fields (NTP timestamp, RTP timestamp, sender’s packet count, and sender’s octet count).

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.2.4	Mid-Session Sending No Data RTCP Test

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.1 and 6.2.2.3.
2) The system under test is possible to control dynamically during an ongoing RTP session to turn sending of RTP packets on and off, while keeping the RTP session.

Test procedure:	1) Set the system under test to send RTP packets.
2) Observe the system under test output until at least one RTCP Sender Report is sent.
3) Set the system under test to not send RTP packets.
4) Observe the system under test output.

Stop condition:	Three RTCP packets are sent after step 3 in the test procedure.

Pass criteria:	The following must be fulfilled after step 3 in the test procedure:
1) No RTP packets are sent.
2) The first two RTCP packets sent contain Sender Reports.
3) The third RTCP packet sent contains a Receiver Report.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.2.5	Mid-Session Receiving No Data RTCP Test

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.2 and 6.2.2.3.
2) The data injection to the system under test is possible to control dynamically during an ongoing RTP session to turn sending of RTP packets on and off, while keeping the RTP session.

Test procedure:	1) Set the data injection to send RTP packets to the system under test.
2) Observe the system under test output until at least one RTCP packet is sent.
3) Set the data injection to not send RTP packets to the system under test.
4) Observe the system under test output.

Stop condition:	Two RTCP packets are sent after step 3 in the test procedure.

Pass criteria:	The second RTCP packet sent contains a Sender Report or Receiver Report with an empty Receiver Report block, indicated by RC=0.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.
If the data injection doesn’t perform step 3 fast enough after step 2, the system under test may receive some RTP packets after step 2. Those RTP packets will then be reported on in the next RTCP packet sent, which is the reason to use the second RTCP packet sent after step 3 as input to the pass criteria.

#### 6.2.2.6	Compound RTCP Packet Format Test

[Editor’s note: Should UDP header Length field and IP header Total Length field also be tested for consistency and included in the pass criteria?]

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.1 – 6.2.2.5.
2) Reduced-size RTCP [8] is not negotiated to be used in the session (see 5.3.11 for reduced-size tests), e.g. indicated by "a=rtcp-rsize" not being present in the SDP answer for the tested RTP media.

Test procedure:	Observe the system under test output from tests 6.2.2.1 – 6.2.2.5.

Stop condition:	Not applicable (implicit through tests in 6.2.2.1 – 6.2.2.5).

Pass criteria:	All RTCP packets sent must fulfil:
1) A Sender Report or Receiver Report (equally acceptable for the outcome of this test) is included as the first part of the compound RTCP packet.
2) An SDES packet with a CNAME SDES item is included in the compound RTCP packet (except when a compound RTCP packet is split for partial encryption, if used; see SRTCP tests in clause 5.5).
3) Each RTCP packet in the compound RTCP packet starts on a 32-bit boundary.
4) Each RTCP packet in the compound RTCP packet has a length field with a value that is the number of 32-bit words minus one occupied by that RTCP packet, including the RTCP packet header and any padding.

#### 6.2.2.7	RTCP Report Count Test

Status:	Mandatory

Preconditions:	The system under test has passed tests 6.2.2.1 – 6.2.2.5.

Test procedure:	Observe the system under test output from tests 6.2.2.1 – 6.2.2.5.

Stop condition:	Not applicable (implicit through tests in 6.2.2.1 – 6.2.2.5).

Pass criteria:	All RTCP packets sent must fulfil:
1) Regardless if the RTCP packet is a Sender Report or Receiver Report, the report count (RC) field is consistent with the number of report blocks observed to be included in the packet.
2) The RTCP packet length field has a value that is the number of 32-bit words minus one occupied by that RTCP packet, including the RTCP packet header and any padding.

Comments:	This test does not cover a higher report count than 1, which can be required when more than a single RTP stream is sent in an RTP session, e.g. in multi-stream scenarios as described by TS 26.114 [9] Annex S. Tests covering a higher report count than 1 may be added in future revisions of this specification.

### 6.2.3	RTCP Bandwidth and Transmission Timing Tests

#### 6.2.3.1	RTCP Disable Test

Status:	Mandatory

Preconditions:	RTCP transmission is disabled for the RTP session, e.g. negotiated in SDP by both b=RS:0 and b=RR:0 being present in the SDP answer (zero RTCP bandwidth).

Test procedure:	Observe the system under test output.

Stop condition:	Time has passed corresponding to three maximum RTCP intervals (see clause 6.2.1, ca 21 seconds).

Pass criteria:	No RTCP packets are sent.

Comments:	The test result is agnostic to RTP packets being sent or received by the system under test.

#### 6.2.3.2	RTCP Basic Interval Test

[Editor’s note: Consider if this should be split to a mandatory "RTCP Point-to-Point Interval Test" that allows a constant RTCP interval of (ca) 5 seconds, and a "RTCP Multiparty and Multistream Interval Test" that follows RFC 3158 and is conditionally mandatory for implementations supporting multistream.]

Status:	Mandatory

Preconditions:	1) The system under test is pre-configured to not send any RTP packets, e.g. set as receive-only.
2) The system under test is pre-configured with default RTCP timing (based on 5% RTCP bandwidth), e.g. neither "b=RS" nor "b=RR" are included in the SDP answer.
3) The system under test is set to not use the reduced minimum RTCP interval for high bandwidth sessions (see test 6.2.3.4).
4) The system under test is set to use an RTP session bandwidth of at least 1000 kbit/s, e.g. pre-configured or negotiated in SDP through "b=AS:1000" being included in the SDP answer. If that high RTP session bandwidth cannot be set, the maximum possible RTP session bandwidth should instead be set.
5) The data injection to the system under test is set to not send any RTP packets, such that the system under test does not receive any RTP packets.

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time (called the interval), Ii = ti – ti-1 for i=[2..n].
5) Calculate the average RTCP interval I = 1/(n-1) * ∑ i=[2..n] Ii.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 2 seconds ≤ mini Ii ≤ 2.5 seconds.
2) 5.5 seconds ≤ maxi Ii ≤ 7 seconds.
3) 4.5 seconds ≤ I ≤ 5.5 seconds.

Comments:	The RTP testing strategies in [3] recommends the test to run for at least 20 minutes and highlights that an implementation fails the test if the RTCP packets are sent with a constant interval (not randomized).

#### 6.2.3.3	RTCP Low Bandwidth Test

[Editor’s note: Will this ever be something else than the minimum 5 seconds!?]

Status:	Mandatory

Preconditions:	1) The system under test is pre-configured with default RTCP timing (based on 5% RTCP bandwidth), e.g. neither "b=RS" nor "b=RR" are included in the SDP answer.
2) The system under test is pre-configured to send and receive RTP packets during the test session, e.g. set as send-receive in SDP answer.
3) The RTP session bandwidth is set lower than 72 kbit/s, e.g. through b=AS included in SDP answer.

Test procedure:	1) Set the data injection to send a single RTP stream (one SSRC) to the system under test.
2) Observe the system under test output.
3) Note the reception time (ti) and size (Si) in bits of each RTCP packet sent from the system under test, including UDP and IP headers.
4) Count the number of RTCP packets, n, sent from the system under test.
5) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii = ti – ti-1 for i=[2..n].
6) Calculate the average RTCP interval I = 1/(n-1) * ∑ i=[2..n] Ii.
7) Calculate the average RTCP bandwidth B = (∑ i=[1..n-1] Si ) / (∑ i=[2..n] Ii).
8) Calculate the average RTCP packet size S = 1/(n-1) * ∑ i=[2..n] Si.
9) Calculate the deterministic RTCP interval, T = max( 2 * S / B; Tmin ), where Tmin = 5 seconds.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 0.4 * T ≤ mini Ii.
2) maxi Ii ≤ 1.25 * T.
3) 0.95 * T ≤ I ≤ 1.05 * T

Comments:	The values 0.4 and 1.25 above are chosen based on the [0.5;1.5] * T uniform distribution randomization of the RTCP interval, divided by the e-1.5 (=1.21828…) reconsideration factor, both taken from section 6.2.1 of IETF RFC 3550.

#### 6.2.3.4	RTCP High Bandwidth Test

Status:	Conditionally Mandatory

Preconditions:	1) The system under test can use higher RTP bandwidth than 72 kbit/s.
2) The system under test is pre-configured with default RTCP timing (based on 5% RTCP bandwidth), e.g. neither "b=RS" nor "b=RR" are included in the SDP answer.
3) The system under test is pre-configured to send and receive RTP packets during the test session, e.g. set as send-receive in SDP answer.
4) The RTP session bandwidth is set higher than 72 kbit/s, e.g. through b=AS included in SDP answer.

Test procedure:	1) Set the data injection to send a single RTP stream (one SSRC) to the system under test.
2) Observe the system under test output.
3) Note the reception time (ti) and size (Si) in bits of each RTCP packet sent from the system under test, including UDP and IP headers.
4) Count the number of RTCP packets, n, sent from the system under test.
5) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii = ti – ti-1 for i=[2..n].
6) Calculate the average RTCP interval I = 1/(n-1) * ∑ i=[2..n] Ii.
7) Calculate the reduced deterministic RTCP interval, T = 360 / BRTP, where BRTP = RTP session maximum bandwidth in kbit/s, e.g. calculated as 2 times the bandwidth from b=AS in SDP answer for point-to-point sessions (see also comment below).

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 0.4 * T ≤ mini Ii.
2) maxi Ii ≤ 1.25 * T.
3) 0.95 * T ≤ I ≤ 1.05 * T.

Comments:	The values 0.4 and 1.25 above are chosen based on the [0.5;1.5] * T uniform distribution randomization of the RTCP interval, divided by the e-1.5 (=1.21828…) reconsideration factor, both taken from section 6.2.1 of IETF RFC 3550.
The calculation of the BRTP factor in the denominator of step 7 is seemingly not in accordance with the definition of IETF RFC 4566 section 5.8 that says b=AS is the RTP session bandwidth (all RTP streams that are sent and received in the RTP session), but 3GPP specifications commonly interpret b=AS as receive bandwidth seen from the part sending the SDP, which is thus only half of the actual RTP session bandwidth in a bandwidth-symmetric point-to-point session.

#### 6.2.3.5	RTCP Explicit Bandwidth Test

[Editor’s note: Will the interval ever be something else than the minimum 5 seconds and will the RTCP bandwidth ever risk exceeding 800 bits / 5 seconds = 160 bps?]

Status:	Conditionally Mandatory

Preconditions:	1) The system under test can set explicit RTCP bandwidth
2) The system under test has passed test 6.2.3.4.
3) The system under test is pre-configured to send and receive RTP packets during the test session, e.g. set as send-receive in SDP answer.
4) The RTP session bandwidth is set lower than 72 kbit/s, e.g. through b=AS included in SDP answer.

Test procedure:	1) Negotiate an explicit RTCP bandwidth in SDP, setting "b=RS:625" and "b=RR:1875" in SDP answer.
2) Observe the system under test output.
3) Note the reception time (ti) and size (Si) in bits of each RTCP packet sent from the system under test, including UDP and IP headers.
4) Count the number of RTCP packets, n, sent from the system under test.
5) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii = ti – ti-1 for i=[2..n].
6) Calculate the average RTCP bandwidth B = (∑ i=[1..n-1] Si ) / (∑ i=[2..n] Ii).
7) Note the test result from step 6.
8) Negotiate an explicit RTCP bandwidth in SDP, setting "b=RS:0" and "b=RR:2500" in SDP answer.
9) Re-run steps 2-6 and note the test result from step 6.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less, in each of steps 7 and 9.

Pass criteria:	1) B ≤ 1250 bit/s, from step 8 of this test.
2) B ≤ 1250 bit/s, from step 10 of this test.

Comments:	Pass criteria 1 and 2 are identical because the proportion of senders in this point-to-point RTP session (100%) is larger than RS/(RS+RR) (see section 2 of IETF RFC 3556 [6]).

#### 6.2.3.6	RTCP Timeout Test

[Editor’s note: Optional on system under test capable of multistream. Timeout remote RTP sender if no RTP has been received in the last 5 RCTP intervals, meaning that SR/RR is no longer sent for that SSRC (6.2.2.5) and RTCP bandwidth is adjusted to match the new number of senders/receivers in the RTP session.]

Status:	Conditionally Mandatory

Preconditions:	1) The system under test is capable of using multiple simultaneous RTP streams in a single RTP session.
2) The system under test has passed tests 6.2.2.1, 6.2.2.3 and 6.2.2.5.
3) The data injection to the system under test is possible to control dynamically during an ongoing RTP session to turn sending of RTP packets on and off, while keeping the RTP session.

Test procedure:	1) Set the data injection to send RTP packets to the system under test.
2) Observe the system under test output until at least one RTCP packet is sent.
3) Set the data injection to not send RTP packets to the system under test.
4) Observe the system under test output.

Stop condition:	Six (6) RTCP packets are sent after step 3 in the test procedure.

Pass criteria:	The 6th RTCP packet sent contains a Sender Report or Receiver Report that does not contain any report block for the stopped SSRC from data injection.

Comments:	-

#### 6.2.3.7	RTCP Step Join Backoff Receiver Test

[Editor’s note: Optional on system under test capable of multistream. Precondition: default timing, i.e. no b=RS/RR, small RTCP bandwidth (ca 950 b/s; RTP session bandwidth ≤ 19 kbit/s ), SUT not sending RTP. Method: data injection waits for first RTCP packet from SUT, then immediately sends 100 RTCP packets with different SSRC, different SDES CNAME value and same size (ca 128 bytes each), observe time between SUT first and second RTCP packets. Check that time is T > 101 * S / ( B * Fr * (e-1.5) * 2), where S=size of RTCP packets, B=RTCP bandwidth, Fr=fraction of RTCP bandwidth allocated to receivers (normally 75%). Definitely fail if time is closer to Te > MAX( [S / (B * Fr * (e-1.5)],[2.5 / (e-1.5)]). Pass if time is no less than T and no more than 3*T]

#### 6.2.3.8	RTCP Step Join Backoff Sender Test

[Editor’s note: Optional on system under test capable of multistream. Precondition: default timing, i.e. no b=RS/RR, small RTCP bandwidth (ca 950 b/s; RTP session bandwidth ≤ 19 kbit/s ), SUT sending RTP. Method: data injection waits for first RTCP packet from SUT, then immediately sends 100 RTCP packets with different SSRC, different SDES CNAME value and same size (ca 128 bytes each), observe time between SUT first and second RTCP packets. Check that time is T > 101 * S / ( B * Fs * (e-1.5) * 2), where S=size of RTCP packets, B=RTCP bandwidth, Fr=fraction of RTCP bandwidth allocated to senders (normally 25%). Definitely fail if time is closer to Te > MAX( [S / (B * Fr * (e-1.5)],[2.5 / (e-1.5)]). Pass if time is no less than T and no more than 3*T]

### 6.2.4	Sender Report Sender Info Tests

[Editor’s note: Priority 1.]

#### 6.2.4.1	Sender SSRC Test

[Editor’s note: Unclear what is a suitable stop condition; receiving three RTCP packets per SSRC is a bit arbitrary.]

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.3.
2) The system under test is pre-configured to send RTP packets during the test session, e.g. set as send-receive or send-only.
3) The number of simultaneously used SSRC identifiers to be sent by the system under test in the RTP session is known a-priori (only one in the single-stream case).

Test procedure:	Observe system under test output.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent per used SSRC identifier (at least the first RTCP packet may be sent before first RTP packet for each SSRC).

Pass criteria:	1) The "SSRC of sender" header field in all sent RTCP packets contains a value that is identical to the "synchronization source (SSRC) identifier" header field in at least one of the sent RTP packets.
2) The "synchronization source (SSRC) identifier" header field in all sent RTP packets contains a value that is identical to the "SSRC of sender" header field in at least one of the sent RTCP packets.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.4.2	NTP Test

[Editor’s note: Having stop condition allow receiving just three RTCP packets is truly a minimum and a bit arbitrary; would it be desirable to require a larger sample size?  This test doesn’t check for correct NTP timestamp format (high 32 bits should correspond to seconds, low 32 bits to 2-32 parts of a second); should it?]

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.3.
2) The system under test is pre-configured to send RTP packets during the test session, e.g. set as send-receive or send-only.
3) The "NTP timestamp" field in the RTCP packet Sender Information is monotonously increasing (see 6.2.4.3 for test of wrapped value).

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) and NTP timestamp field value (wi) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii = ti – ti-1, and difference in NTP time, Ni = wi – wi-1, for i=[2..n].
5) Calculate the NTP timestamp clock rate estimates Ci = Ni / Ii for i=[2..n].
6) Calculate the NTP timestamp correlation quotients Qi = Ci / Ci-1 for i=[3..n].

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).

Pass criteria:	0.999 ≤ Qi ≤ 1.001 for all i=[3..n].

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criterion corresponds to a minimum clock accuracy of 0.1% but a much higher accuracy should in general be expected.

#### 6.2.4.3	Wrapped NTP Test

[Editor’s note: Same comment as for 6.2.4.2.]

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.4.2.
2) The system under test is pre-configured to send RTP packets during the test session, e.g. set as send-receive or send-only.
3) The "NTP timestamp" field in the first RTCP packet Sender Information is set to a value close to 264-1 (when interpreted as an unsigned 64-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) and NTP timestamp field value (wi) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii = ti – ti-1, and difference in NTP time, Ni = wi – wi-1, for i=[2..n].
5) For any wi and wi-1 with wi < wi-1 when interpreting wi and wi-1 as unsigned 64-bit values, calculate Ni = 264 – wi-1 + wi = (264 + wi – wi-1) mod 264.
6) Calculate the NTP timestamp clock rate estimates Ci = Ni / Ii for i=[2..n].
7) Calculate the NTP timestamp correlation quotients Qi = Ci / Ci-1 for i=[3..n].

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).
3) The "NTP timestamp" field in the last RTCP packet Sender Information is less than the "NTP timestamp" field in the first RTCP packet Sender Information, when interpreting those fields as unsigned 64-bit integers.

Pass criteria:	0.999 ≤ Qi ≤ 1.001 for all i=[3..n].

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criterion corresponds to a minimum timestamp clock accuracy of 0.1% but a much higher accuracy should in general be expected.

#### 6.2.4.4	Time Stamp Test

[Editor’s note: Having stop condition allow receiving just three RTCP packets is truly a minimum and a bit arbitrary; would it be desirable to require a larger sample size?]

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.3 and 6.2.4.1.
2) The system under test is pre-configured to send RTP packets for a single RTP stream (one SSRC) during the test session, e.g. set as send-receive or send-only.
3) The "RTP timestamp" field in the RTCP packet Sender Information is monotonously increasing (see 6.2.4.5 for test of wrapped value).

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti), NTP timestamp field value (wi), and RTP timestamp field value (ri) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii = ti – ti-1, difference in NTP time, Ni = wi – wi-1, and difference in RTP time, Ti = ri – ri-1, for i=[2..n].
5) Calculate the RTP timestamp rate estimates Ri = Ti / Ii for i=[2..n].
6) Calculate the NTP timestamp clock rate estimates Ci = Ni / Ii for i=[2..n].
7) Calculate the average RTP timestamp rate estimate R = 1/(n-1) * ∑ i=[2..n] Ri.
8) Calculate the RTP timestamp to NTP timestamp quotients Pi = Ri / Ci for i=[2..n].
9) Calculate the correlation quotients Qi = Pi / Pi-1 for i=[3..n].
10) Retrieve RTP Time Stamp rate information, S, e.g. from "…/S" on the SDP "a=rtpmap" line, applicable for the RTP stream that the RTCP packets are associated with.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).

Pass criteria:	1) 0.999 ≤ Ri / S ≤ 1.001 for all i=[2..n].
1) 0.999 ≤ Qi ≤ 1.001 for all i=[3..n].

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criteria correspond to a minimum timestamp clock accuracy of 0.1% but a much higher accuracy should in general be expected.

#### 6.2.4.5	Wrapped Time Stamp Test

[Editor’s note: Same comment as for 6.2.4.4.]

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.4.4.
2) The system under test is pre-configured to send RTP packets for a single RTP stream (one SSRC) during the test session, e.g. set as send-receive or send-only.
3) The "RTP timestamp" field in the first RTCP packet Sender Information is set to a value close to 232-1 (when interpreted as an unsigned 32-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti), NTP timestamp field value (wi), and RTP timestamp field value (ri) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii = ti – ti-1, difference in NTP time, Ni = wi – wi-1, and difference in RTP time, Ti = ri – ri-1, for i=[2..n].
5) For any ri and ri-1 with ri < ri-1 when interpreting ri and ri-1 as unsigned 32-bit values, calculate Ti = 232 – ri-1 + ri = (232 + ri – ri-1) mod 232.
6) Calculate the RTP timestamp rate estimates Ri = Ti / Ii for i=[2..n].
7) Calculate the NTP timestamp clock rate estimates Ci = Ni / Ii for i=[2..n].
8) Calculate the average RTP timestamp rate estimate R = 1/(n-1) * ∑ i=[2..n] Ri.
9) Calculate the RTP timestamp to NTP timestamp quotients Pi = Ri / Ci for i=[2..n].
10) Calculate the correlation quotients Qi = Pi / Pi-1 for i=[3..n].
11) Retrieve the intended RTP Time Stamp rate information, S, e.g. from "…/S" on the SDP "a=rtpmap" line, applicable for the RTP stream that the RTCP packets are associated with.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).
3) The "RTP timestamp" field in the last RTCP packet Sender Information is less than the "RTP timestamp" field in the first RTCP packet Sender Information, when interpreting those fields as unsigned 32-bit integers.

Pass criteria:	1) 0.999 ≤ Ri / S ≤ 1.001 for all i=[2..n].
1) 0.999 ≤ Qi ≤ 1.001 for all i=[3..n].

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criteria correspond to a minimum timestamp clock accuracy of 0.1% but a much higher accuracy should in general be expected.

#### 6.2.4.6	Packet Count Test

[Editor’s note: Require Packet Count to match reported RTP stream.]

#### 6.2.4.7	Wrapped Packet Count Test

[Editor’s note: Require Packet Count to consistent with reported RTP stream also after wrap.]

#### 6.2.4.8	Octet Count Test

[Editor’s note: Require Octet Count to match reported RTP stream.]

#### 6.2.4.9	Wrapped Octet Count Test

[Editor’s note: Require Octet Count to be consistent with reported RTP stream also after wrap.]

### 6.2.5	Source Description (SDES) Tests

#### 6.2.5.1	Basic SDES Test

[Editor’s note: Every SDES Item length must be correctly set and field content must not be zero terminated.]

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.6.
2) The system under test is pre-configured to send RTP packets during the test session, e.g. set as send-receive or send-only.

Test procedure:	Observe system under test output.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent per used SSRC identifier (at least the first RTCP packet may be sent before first RTP packet for each SSRC).

Pass criteria:	The following must be fulfilled for all sent RTCP SDES packets:
1) The source count (SC) header field that matches the number of SDES item chunks included in the respective RTCP SDES packet.
2) Each SDES item chunk ends with at least one octet containing a zero value.
3) Each SDES item in a chunk has a length field that corresponds to the text content length in octets of that SDES item, corresponding to the distance in octets starting from the octet after that length field until the start of the next SDES item or until the terminating zero value octet (see above).
4) Each SDES item in a chunk with non-zero content length has a text content where the last octet value is not zero (is not a zero-terminated string).

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.5.2	Canonical End-point Identifier (CNAME) Test

[Editor’s note: Shall be present in every compound RTCP packet; tested in 6.2.2.6. There are no restrictions on CNAME content beyond that it shall be identical for every RTCP packet belonging to the same RTP stream.]

### 6.2.6	Receiver Report Block Tests

[Editor’s note: Priority 1. Ensure that this document covers:
Incorrect loss when RTP sequence number of a stream does not start at 0 (6.2.6.4)
Incorrect loss when RTP sequence number rolls over (6.2.6.7 and 6.2.6.12)
Incorrect loss / report content when no packets are received in a reporting period (6.2.6.5 and 6.2.6.10)
Incorrect loss when RTP SSRC changes (6.2.6.3 and optionally 6.2.6.4)
Incorrect delay reporting (6.2.6.19)
Incorrect jitter reporting (6.2.6.17)]

#### 6.2.6.1	SSRC Consistency Test

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.2, 6.2.2.5, 6.2.2.6, and 6.2.2.7.
2) The system under test is pre-configured to receive RTP packets during the test session, e.g. set as send-receive or receive-only.
3) Data injection is set to send RTP.

Test procedure:	Observe system under test and data injection output.

Stop condition:	An RTCP packet is sent from system under test, containing at least a receiver report block.

Pass criteria:	The SSRC value in the RTCP report block can be found as RTP header SSRC field value in at least one of the RTP packets sent from data injection.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.
A report block containing an SSRC value identical to the SSRC value in the RTP header of received RTP packets is henceforth denoted as "reporting on" the RTP stream those RTP packets belong to.

#### 6.2.6.2	SSRC Completeness Test

[Editor’s note: The stop condition must allow that not all SSRC in the RTP session are reported in a single RTCP packet (due to MTU size limitations) but are split across multiple RTCP packets.]

Status:	Conditionally Mandatory

Preconditions:	1) The system under test is capable of using multiple simultaneous RTP streams in a single RTP session.
2) The system under test has passed test 6.2.6.1.
3) The system under test is pre-configured to receive RTP packets during the test session, e.g. set as send-receive or receive-only.
4) Data injection is set to send RTP.
5) The number of simultaneously used SSRC identifiers to be sent by the data injection in the RTP session is known a-priori (only one in the single-stream case).

Test procedure:	Observe system under test and data injection output, noting which SSRC values that are included in RTCP report blocks sent from the system under test and which SSRC values that are included in RTP headers of RTP packets sent from data injection.

Stop condition:	TBD.

Pass criteria:	All SSRC field values that can be found in RTP packet headers sent from data injection can also be found as SSRC field values in at least one of the RTCP report blocks sent from the system under test.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.3	SSRC Change Test

[Editor’s note: SUT must report both SSRC as separate streams; simultaneously until old SSRC expired, refer to (all?) other clauses in 6.2.6 that must be fulfilled for both SSRC separately. The SDES CNAME value must be kept the same across the SSRC change.]

#### 6.2.6.4	Initial Zero Loss Test

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.6.1.
2) The system under test is pre-configured to receive RTP packets during the test session, e.g. set as send-receive or receive-only.
3) Data injection is set to send a single RTP stream (one SSRC) without loss, with monotonically increasing RTP sequence number (SN), and with the first RTP packet sent containing a non-zero RTP sequence number (SN).

Test procedure:	Observe system under test and data injection output.

Stop condition:	An RTCP packet is sent from system under test, containing at least a receiver report block reporting on the RTP stream from data injection.

Pass criteria:	The report block reporting on the RTP stream from data injection must fulfil:
1) The fraction lost report block field is zero.
2) The cumulative number of packets lost report block field is zero.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.5	Zero Loss Test

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.6.4.
2) The system under test is pre-configured to receive RTP packets during the test session, e.g. set as send-receive or receive-only.
3) Data injection is set to send a single RTP stream (one SSRC) without loss and with monotonically increasing RTP sequence number (SN).

Test procedure:	Observe system under test and data injection output.

Stop condition:	Two RTCP packets are sent from system under test, each containing at least a receiver report block reporting on the RTP stream from data injection.

Pass criteria:	The report block in RTCP packets sent from the system under test, reporting on the RTP stream from data injection must fulfil:
1) The fraction lost report block field is zero in the second RTCP packet.
2) The cumulative number of packets lost report block field value is identical in the first and second RTCP packet.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.6	Loss Test

[Editor’s note: How many different loss values should be tested during an interval?]

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.6.5 and 6.2.6.10.
2) The system under test is pre-configured to receive RTP packets during the test session, e.g. set as send-receive or receive-only.
3) Data injection is set to send a single RTP stream (one SSRC) with loss according to test procedure (below) and with monotonically increasing RTP sequence number (SN).

Test procedure:	1) Observe system under test and data injection output.
2) Note the extended last sequence number received (si), fraction lost (fi) and cumulative number of packets lost (ci) for each RTCP report block sent from the system under test reporting on the RTP stream sent from data injection.
3) An RTCP packet, i=1, is sent from the system under test, reporting on the RTP stream from data injection.
4) Drop RTP packets from data injection to the system under test according to step 9 below.
5) An RTCP packet, i, is sent from the system under test, reporting on the RTP stream from data injection.
6) Calculate number of packets sent Ni = si – si-1.
7) Calculate fraction lost Fi = INT(ni * 256 / N), with n set according to 9a) – 9e) below.
8) Calculate packets lost Ci = ci – ci-1.
9) Repeat steps 4 – 8, dropping:
	a) A single RTP packet; i = 2, n2 = 1.
	b) Two RTP packets in succession; i = 3, n3 = 2.
	c) Two RTP packets with three RTP packets in between; i = 4, n4 = 2.
	d) Every third RTP packet; i = 5, n5 = N5 / 3.
	e) Every other RTP packet; i = 6, n6 = N6 / 2.

Stop condition:	Step 8 is fulfilled for i = 6, repetition 9e).

Pass criteria:	1) fi = Fi for i=[2..6].
2) Ci = INT(ni) for i=[2..6].

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.7	Wrapped Loss Test

[Editor’s note: Must be consistent on sequence number wrap]

#### 6.2.6.8	Duplicate Loss Test

[Editor’s note: Must be consistent on receiving duplicate RTP packets]

#### 6.2.6.9	All Loss Test

[Editor’s note: There must be no report block for an SSRC where no packets are received, but loss state must be consistent (not reset) when reported again.]

#### 6.2.6.10	Extended Highest Sequence Number Received Test

[Editor’s note: Must match received RTP.]

#### 6.2.6.11	Wrapped Extended Highest Sequence Number Received Test

[Editor’s note: Must match received RTP and be consistent even on RTP SN wraparound]

#### 6.2.6.12	Interarrival Jitter Test

[Editor’s note: Must match received RTP but it is unclear how to construct suitable data injection, or how to define a suitable tolerance. One case to test is without jitter.]

#### 6.2.6.13	Duplicate Interarrival Jitter Test

[Editor’s note: Must match received RTP even in the presence of duplicate RTP packets.]

#### 6.2.6.14	Last Sender Report Timestamp Test

[Editor’s note: Must contain middle 32 bits from the 64-bit NTP timestamp of the last received SR packet for the reported SSRC in the other direction; that RTP packets but no SR was received yet should be considered error condition tested by 6.2.2.3]

#### 6.2.6.15	Delay Since Last SR Test

[Editor’s note: Must "closely agree" (define tolerance; general and/or related to test instrument precision) with test instrument estimate of SR and corresponding RR reception timing]

### 6.2.7	BYE Tests

[Editor’s note: Maybe optional; RTCP BYE is hardly ever used in practice. Priority 3? No SSRC that a BYE is received for are included in any RR. No SSRC that a BYE is sent for is included in any SR. An SSRC that a BYE is received for must not be included in RTCP bandwidth and interval calculations. Must be capable to receive BYE, with and without optional text, with a note that it is optional to send BYE before RTP session transport is closed.]

### 6.2.8	Feedback Report Block Tests

[Editor’s note: May be left out of scope for the first version but could be included as a placeholder. Priority 2.]

#### 6.2.8.1	General

[Editor’s note: Expect one subclause per described feedback block type. An unknown feedback packet must be correctly ignored on reception.]

#### 6.2.8.2	AVPF Transmission Timing Tests

[Editor’s note: Priority 2.]

### 6.2.9	Extended Report Block Tests

[Editor’s note: Should likely be left out of scope for the first version but could be included as a placeholder. Priority 3.]

#### 6.2.9.1	General

[Editor’s note: Expect one subclause per described XR block type. An unknown XR packet must be correctly ignored on reception.]

### 6.2.10	APP Tests

[Editor’s note: Should likely be left out of scope for the first version but could be included as a placeholder. Priority 3. An APP packet with unknown name must be correctly ignored on reception.]

### 6.2.11	Reduced-Size Packet Tests

[Editor’s note: May be left out of scope for the first version but could be included as a placeholder. Priority 3. An RTCP packet not starting with SR or RR must be correctly ignored on reception by a receiver not supporting reduced-size packets.]

## 6.3	RTP Tests

[Editor’s note: Aspects that are applicable both to RTP and SRTP. Priority 2]

### 6.3.1	General

This clause describes tests that are applicable to all RTP and SRTP packets. If not explicitly stated otherwise, any reference to RTP is equally applicable to SRTP. Tests that are only applicable to SRTP and not to RTP are specified in clause 6.5.

[Editor’s note: Handling of changing SSRC?]

[Editor’s note: From RFC 3158: (what of this applies also to RTCP, e.g. padding?)
Receive packets with padding
Receive packets with marker bit (media specific)
Receive packets with sequence number wrap-around
Receive packets with timestamp wrap-around
Receive packet separation based on payload type
Receive packet ignoring undefined or unsupported payload type (two tests)
Receive packets with non-zero CC field and CSRC list
Send packets with correct padding (conditional to padding used)
Send packets with correct marker bit (media specific)
Send packets with SSRC chosen randomly (mandatory at least for SRTP)
Send packets with initial sequence number chosen randomly (mandatory at least for SRTP)
Send packets with sequence number incrementing by one for each RTP packet
Send packets with sequence number wrap-around
Send packets with initial timestamp chosen randomly (mandatory at least for SRTP)
Send packets with correct timestamp increment (media specific)
Send packets with timestamp wrap-around
Send packets with chosen payload type from signalling]

### 6.3.2	RTP Bandwidth and Transmission Timing Tests

[Editor’s note: Think we want to highlight this in a separate heading]

### 6.3.3	RTP Header Extension Tests

[Editor’s note: At a minimum, header extensions should be correctly skipped and ignored. If used, the profile specific field and length field(s) must be set correctly, and the total packet length must be consistent]

### 6.3.4	RTP Contributing Source Tests

[Editor’s note: At a minimum, CSRC list should be correctly skipped and ignored]

### 6.3.5	RTP Voice Tests

[Editor’s note: Specific aspects for voice streams]

### 6.3.6	RTP Video Tests

[Editor’s note: Specific aspects for video streams]

### 6.3.7	RTP Text Tests

[Editor’s note: Specific aspects for text streams]

## 6.4	SRTCP Tests

### 6.4.1	General

This clause describes tests that are only applicable to SRTCP but not to RTCP. Tests that are applicable to both SRTCP and RTCP are described in clause 6.2.

[Editor’s note: Priority 3. Mandatory to use for mission-critical services.]

## 6.5	SRTP Tests

### 6.5.1	General

This clause describes tests that are only applicable to SRTP but not to RTP. Tests that are applicable to both SRTP and RTP are described in clause 6.3.

[Editor’s note: Priority 3. Mandatory to use for mission-critical services.]

## 6.6	RTP Translator Tests

[Editor’s note: Also describe SRTP Translator, if feasible? Priority 3.]

## 6.7	RTP Mixer Tests

[Editor’s note: Expect this to be applicable also to SRTP Mixer. Priority 3.]

# Abstract Test Suite (ATS) text block

This text should be used for ATS using TTCN. The subdivision is recommended.

# Y	Abstract Test Suite (ATS)

[Editor’s note: Consider if this is appropriate for use in this specification.]

## Y.1	Introduction

This ATS has been produced using the Tree and Tabular Combined Notation (TTCN) according to ISO/IEC 9646-3 [x].

The ATS was developed on a separate TTCN software tool and therefore the TTCN tables are not completely referenced in the table of contents. The ATS itself contains a test suite overview part which provides additional information and references.

# Y.2	The TTCN Graphical form (TTCN.GR)

The TTCN.GR representation of this ATS is contained in an Adobe Portable Document Format™ file (<pdf_file_name>.PDF contained in archive <zip_file_name>.ZIP) which accompanies the present document.

# Y.3	The TTCN Machine Processable form (TTCN.MP)

The TTCN.MP representation corresponding to this ATS is contained in an ASCII file (<mp_file_name>.MP contained in archive <zip_file_name>.ZIP) which accompanies the present document.

Annex <A> (normative):
<Normative annex for a Technical Specification>

Start each annex on a new page.

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content.

Normative annexes only to appear in Technical Specifications. Use style "Heading 8".

Annex <B> (informative):
<Informative annex for a Technical Specification>

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

# B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex <C> (informative):
Bibliography

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | S4#106 | S4-191xxx |  |  |  | Initial version | 0.0.1 |