3GPP TSG-SA WG4 Meeting #106 	S4-191206

# Busan, Korea, October 21-25

Source:				Fraunhofer IIS

Title:					Evaluation of reference code for MASA format

Document for:		Discussion

Agenda Item:		7.5

## 1.	Introduction

The present contribution aims to present results of a listening test comparing the MASA format and the reference code provided by Nokia with Ambisonics at different orders.

## 2.	Listening Test

### Test Setup

To evaluate the performance of the MASA format, a preference test was designed using a modified version of the MUSHRA methodology. Neither the reference nor the hidden reference and anchors were presented to the auditor, who was asked instead to score with 100 points the condition under test he preferred most. Listeners were instructed to assess the naturalness and the pleasantness of the reproduced audio scene by taking into account in particular the localization of the sound events, the timbre and the signal quality. Four conditions were under test; two of them were in Ambisonics format while the two others in MASA format, achieving in that way a balanced representation of the two types of spatial audio format. A detailed description of the conditions and associated labels are given in the following table.

| Label | Description |
| --- | --- |
| FOA | 1st order of Ambisonics |
| HOA4 | 4th order of Ambisonics |
| MASA_mono_1dir | MASA format using 1 direction per T/F and a mono downmix |
| MASA_stereo_2dir | MASA format using 2 directions per T/F file and a stereo downmix |

### Test items

All test items consist of speech recorded on EigenMike soundfield microphone. Recorded audio scenes were either real conversations involving two to four participants or overlapping and non-overlapping sentences independently uttered by different speakers. Records were made in different environments with different background noises.

### Ambisonics conditions

Ambisonics conditions were derived from the 32 EigenMike raw signal using a spherical harmonic transformation of microphone signals followed by a frequency-dependent equalization as proposed in [1]. The equalization was performed using the same filter-bank as used in the reference code provided by Nokia for the MASA format. The first order and the fourth order of Ambisonics were rendered on loudspeaker layout CICP19 (7.1 + 4) using the HOA renderer defined in [2].

### MASA conditions

MASA conditions were generated by feeding the EigenMike raw signals to the reference C-code provided by Nokia. The conditions were directly rendered by the provided code to loudspeaker layout CICP19.

### Results

A total of 6 expert listeners participated in the test. Test results are presented using 95% confidence intervals and Student-t distribution.

## 3. Conclusion

The source presented results of a listening test evaluating the MASA format for the reproduction of a 3D audio scene on loudspeakers. It has been found that the MASA format using a mono downmix and one direction by time-frequency tile performs in a comparable way to FOA while the quality of the reproduced spatial audio scene scales up when using a stereo downmix and 2 directions per time-frequency tile, reaching a quality close to what a 4th order of Ambisonics can deliver.

## 4. References

[1] Bernschütz, B. et al. „Soft-limiting der modalen amplitudenverstärkung bei sphärischen mikrofonarrays im plane wave decomposition verfahren.“, Proceedings of the 37. Deutsche Jahrestagung für Akustik (DAGA 2011)
[2] ISO/IEC 23008-3, Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 3: 3D audio