3GPP TSG-SA4 Meeting #106	S4-191241

# Busan, Korea, 21-25 October, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR |  | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DRM client role in 5GMS client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TELUS, SONY Europe BV, Qualcomm, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS |  |  |  |  |  | Date: |  |  | 2019-10-21 |
| Category: | F, C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The current color coding of the DRM client in Figure 4.2.2-1 suggests DRM client should be in the scope of media player. DRM client is not necessary to be a part of media player. Media player is supposed to be integrated with any DRM client via its programmnig interfaces. - There is a wide ragne of DRM flavors. Some of them are proprietary. In addition, core functionnalities of hardware-based DRM is usually implemented in a Trusted Execution Environment (TEE) of the processor. So it is difficult to bind DRM client with media player as shown in the current figure from standard adoption and implementation perspetives. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the DRM client color coding to external application - Update DRM client definition - Add media decryption function definition - Media decryption and media decapsulation have their own boxes - Change multi-line connections to M4d to single line - Align on the consistent terminology proposed in S4-191123 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The implementers find difficulty in building 5GMS client |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Detailed description of the change

The change request suggests to change DRM client color to external application in Figure 4.2.2-1.

It should also be noted that encrypted media samples are not handed over to the DRM client. The Media player get licenses and keys from DRM client. DRM client might or might not be a part of Media Player. The media samples will be passed to media decryption module. The media decryption fucntion decryptes the media using the keys acquired from the license delivered by the DRM client fucntion.

DRM Client (optional): when present, DRM is a content protection mechanism with its unique key management systems and key delivery, authentication/authorization, policy enforcement and entitlement check. DRM client might or might not be a part of Media Player.

In the original document, there is no definition of media decryption. The change suggests to put media decryption and media decapsultion into their own boxes, and add media decryption definition as follows.

Media Decryption (optional): when present, media decrytion fuction is responsible to decryte the media samples using the keys provided in the DRM license, and further passing them to the media decoder to enable playback of encrypted media. The media decryption and media decoding could be implemnted on the software on general-purpose processors, or for more secured and robust architecture, the decryption, decoding and rednerfing could be implemented on the hardware on secure processors.

In addition, the changes suggests to remove the redundant connections over M4d between 5GMSd AS and Media Player components. There are no interactions between Media Presentation and Rendering between 5GMSd AS, and between Media Decoders and 5GMSd AS. A single line is used to indicate all the interactions between 5GMSd AS and Media Player. This should be enough for the high level architecture diagram like Figure 4.2.2-1.

This change enables the same Media player and encrypted media files could be compatible wth any DRM protection mechnisms as common encryption (CENC) has been used widely today, thus allowing the development of robust media player with the support of a range of content decryptionand and protection tehcnologies, for example, MSFT Playready, Apple Fairplay, Google Widevien, and a variety of proprietary DRM systems, potentially used in any 5GMSd client.

*** Start of Chagnes ***

### 4.2.2	UE Media Functions

The UE may include many detailed subfunctions that can be used individually or controlled individually by the 5GMS-Aware application. This clause breaks down several relevant identified sub-functions for which stage-3 specification is available.

The 5GMS Aware Application itself may include many functions that are not provided by the 5GMS client or the 5G UE. Examples include content discovery, notifications, social network integration, etc. The 5GMS Aware application may also include functions that are equivalent to ones provided by the 5GMS client and may only use a subset of the 5GMS client functions.

With respect to Media Player functions, Figure 4.2.2-1 shows more detailed functional components of a UE for media player functions to access the 5GMS Media AS.

Figure 4.2.2-1: UE 5G Media Downlink Streaming Functions (Media Player centric)

The following subfunctions are identified as part of a more detailed breakdown of the Media Player Function:

-	Media Access Client: Accesses Media Content such as DASH-formatted Media Segments.

- Media Decapsulation: Extracts the elementary media streams for decoding and provides media system related functions such as time synchronization, capability signalling, accessibility signalling, etc

- DRM Client (optional): when present, DRM client might or might not be a part of Media player. It provides a content protection mechanism with its unique key management and key delivery system, authentication/authorization, policy enforcement and entitlement check. DRM client is not defined within 5G Media Streaming specifications.

- Media Decryption (optional): when present, media decryiton is responsible to decrype the media samples using the keys provided in the DRM license, and further passing to the media decoder to enable playback of encrypted media. The media decryption and media decoding could be implemnted on general-purpose processor in software or hardware, or for more secured and robust architecture, the decryption, decoding and rednerfing could be impleted on the hardware on secure processors.

- Media Decoders: Decodes the media like audio or video.

-	Media Rendering / Presentation: Renders the media using an appropriate output device.

..

With respect to the Media Session Handler, Figure 4.2.2-2 shows more detailed functional components of a UE to access the 5GMS Media AF.

*** End of Changes ***

*** Start of the original document ***

### 4.2.2	UE Media Functions

The UE may include many detailed subfunctions that can be used individually or controlled individually by the 5GMS-Aware application. This clause breaks down several relevant identified sub-functions for which stage-3 specification is available.

The 5GMS Aware Application itself may include many functions that are not provided by the 5GMS client or the 5G UE. Examples include content discovery, notifications, social network integration, etc. The 5GMS Aware application may also include functions that are equivalent to ones provided by the 5GMS client and may only use a subset of the 5GMS client functions.

With respect to Media Player functions, Figure 4.2.2-1 shows more detailed functional components of a UE for media player functions to access the 5GMS Media AS.

Figure 4.2.2-1: UE 5G Media Streaming Functions (Media Player centric)

The following subfunctions are identified as part of a more detailed breakdown of the Media Player Function:

-	Media Access Client: Accesses Media Content such as DASH-formatted Media Segments.

-	DRM Client (Optional): When present, the DRM client realizes the content protection related policies of its associated DRM system.

-	Media Decoders: Decodes the media like audio or video.

-	Media Rendering / Presentation: Renders the media using an appropriate output device.

-	Media Decapsulation and decryption: extracts the elementary media streams for decoding and provides media system related functions such as time synchronization, capability signalling, accessibility signalling, etc..

With respect to the Media Session Handler, Figure 4.2.2-2 shows more detailed functional components of a UE to access the 5GMS Media AF.

*** End of the original document***