3GPP TSG-SA4 Meeting #106	S4-191242

# Busan, South Korea,  21st - 25th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2: M1d Media streaming provisioning interface. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 11.10.19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This draft CR starts correcting the M1d Provisioning Procedure for downlink streaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 5.3	Provisioning Session for Media Streaming

The present clause describes the procedure to provision the features using the 5GMS System. The 5GMS Application provider

Figure 5.3-1: High Level Procedure for provisioning the 5GMS System for downlink streaming sessions

Steps

1.	The 5GMSd Application Provider discovers the address (URL) of the 5GMSd AF for Session Provisioning. The address maybe be the result of a manual SLA negotiation procedure or a self-on-boarding procedure.

2.	The 5GMSd Application Provider authenticates itself with the system.

3.	The 5GMSd Application Provider creates a provisioning session for a Downlink Streaming Session, providing its 5GMSd Application Provider ID as input. 5GMSd Application Provider queries the capabilities and authorized features.

4. 	The 5GMSd Application Provider provisions the unicast downlink streaming session. A set of authorized features are activated, such as QoE Reporting, Consumption Reporting, Dynamic Policy, Network Assistance and Content Hosting. When Content Hosting is provided by the 5GMS System, the 5GMS Application provider selects the ingest format.

5.	When content hosting is desired, the 5GMSd AF interacts with the 5GMSd AS to allocate resources and receive the address for 5GMSd AS (M2d address). The 5GMSd AF selects the desired ingest format.

6.	The 5GMSd AF compiles the service access information. The Service Access Information contain access details and options such as M5d (Media Session Handling) Addresses for Consumption Reporting, QoE Reporting, Dynamic policy, Network Assistance, etc. When content hosting is cprovisioned, then also M4d (Media Streaming) information such as the DASH MPD is included.

7.	The 5GMSd AF provides the results to the 5GMSd Application Provider.

- When the 5GMSd Application provider selected the full service access information, then the results are provided in form or addresses and configurations for M2d (Ingest), M5d (Media Session Handling) and M4d (Media Streaming)

- When the 5GMSd Application Provider delegated the service access information handling to the 5GMS System, then a reference to the service access information (e.g. an URL) is provided. The Media Session Handler fetches the full service access information later from the 5GMSd AF.

8.	The 5GMSd Application provider can start ingesting content, using the M2d API address.

9.	The 5GMSd Application Provider executes Service Announcement and updates the UEs.

Optional

10.	The 5GMSd Application Provider may update the provisioning session.

According Provisioning

11.	The 5GMSd AF may send event related or periodic notifications to the 5GMS Application Provider.

According to Schedule or upon request

12. 	The 5GMSd Application Provider may manually terminate the Provisioning Session (at any time). All associated resources are released. Content may be removed from the 5GMSd AS. The 5GMSd Provider may configure a schedule for Provisioning Session termination.

13.	The 5GMSd AF sends a notification upon Provisioning Session terminations.