3GPP TSG-SA4 Meeting #110e	S4-201019

19-28 August 2020

Agenda item	11.5

Source: 	Intel

Title: 	ITT4RT: Updated Semantics for Signaling of Camera Calibration Parameters

Document for	Agreement

# Introduction

In this contribution, we provide further updates to the potential solution on signaling of camera calibration parameters, building on the agreed PD text from the ITT4RT telco on July 8th, in order to address the editor’s note around updating the semantics for the signaling of camera calibration parameters.

# Proposed Updates to ITT4RT PD

## 6.2	Potential Solutions for Signalling of Camera Calibration Parameters

Alignment of the ITT4RT solution to be defined and what is in the FLUS spec is desirable.

Clause 4.5 of the FLUS specification in TS 26.238 describes the FLUS source system. Accordingly the MTSI instantiation of FLUS supports SDP-based description of the relationships among multiple streams in a FLUS source system identified using the SDP attribute a=3gpp-flus-system:<urn>. A vendor-specific source system may be identified by a unique vendor-specific urn identifier such as urn:[com]:[vendor_x]:[system_y]. Alternatively, a 3GPP-specific source system description may be specified using a 3GPP-specific urn identifier such as urn:org:3gpp:itt4rt:default:2020 which would provide interoperable means to signal source system description, including parameters for camera calibration. Further details can be found in Table 4.5-1 of TS 26.238.

One potential solution can be to adopt the FLUS signaling framework to provide the relationships among the 2D video captures to be used for network-based stitching in ITT4RT and amend the SDP-based signaling in Table 4.5-1 to also include the camera calibration parameters. The existing SDP syntax in Table 4.5-1 may be reused (e.g., a=3gpp-flus-configuration: which provides source system-specific configuration parameters for the source system at the session level or a=3gpp-flus-media-configuration: which provides media stream configuration parameters for the source system at the media level) or a dedicated new SDP attribute (e.g., a= a=3gpp-flus-camera-configuration:) may be defined to explicitly signal the camera calibration parameters.

The set of camera calibration parameters to be signaled can include lens numbers, layouts, positions, distortions, angles, radius and resolutions.

As a possible solution, the SDP syntax for a=3gpp-flus-media-configuration may be defined with the following ABNF:

3gpp-flus-media-configuration = "a=3gpp-flus-media-configuration:" [SP "Param 1" SP "Param 2" SP ……. SP "Param K"]

where “Param 1”, …. , “Param K” may be the set of intrinsic, distortions and extrinsic camera parameters.

3GPP-specific source system description conveyed using the urn identifier such as urn:org:3gpp:itt4rt:default:2020 may imply such an SDP syntax to communicate camera calibration parameters. If the set of parameters to be conveyed is found to considerably increase the SDP size, another alternative may be considered where the camera calibration parameters may be described by a JSON document embedded in the SDP.

For each 2D video capture to be used for network-based stitching, the SDP attribute 3gpp-flus-media-configuration may be included under the m= line to signal the camera calibration parameters associated with that particular media stream. Session-level camera configuration parameters may be signalled using 3gpp-flus-configuration.

The set of camera calibration parameters to be signaled can include lens numbers, layouts, positions, angles, radius and resolutions.  More specifically the following parameters may be signaled:

- Number of cameras

- Layout of the cameras

- General intrinsic parameters: resolution, Focal length (focal_x, focal_y) in pixel unit (int), Principal point (center of projection), Lens distortion (deviation from ideal perspective or fisheye) of each camera. These are expressed in Figure 6b below by intrinsic parameters represented by image_height, image_width, center_x, center_y, radius in pixel unit (int) and camera field of view (fov_h, fov_v) in angle or radian degree unit (float)

- Extrinsic camera parameters for each camera represented in the above figure by translation (x, y, z coordinates) and orientation (yaw, pitch, roll) values of each camera to accommodate various rig geometries. These are depicted in Figure 6a below.

- Entrance pupil (EP), with 4 floating-point EP coefficients. EP coefficients provide a more precise model of various lens characteristics and improve the conversion from a sample location of an active area in a lens decoded picture to sphere coordinates relative to the global coordinate axes. Please note that the formulation of the distortion correction and the EP coefficients for any lens can be derived using the calibration provided in [10]. The EP coefficients are formulated using a polynomial, and added seamlessly on whatever lens projection type specified. The use of EP coefficients is especially encouraged for wide lens cameras, e.g., for fisheye omnidirectional video.

Figure 6a – Extrinsic camera calibration parameters.

Figure 6b – Intrinsic camera calibration parameters.

Below we provide further detail on potential camera calibration parameters based on ISO/IEC 23008-2 [3], considering the multi-view acquisition information SEI message for HEVC. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(eqn. 1)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector, and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1. The elements of A[ i ], R[ i ], T[ i ] are determined according to the syntax elements signalled in this SEI message and as specified below.

Earlier equation can be extended to incorporate the entrance pupil variation proposed in [10]. As a result, we propose to correct the incidence ray of  cP[ i ] = [ u v 1 ] to always pass through the camera optical center. Due to this process the distortion is removed. By adapting the proposal in [10], we propose E[i] as a 1x1 vector that is individually incorporated as follows:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( (wP + E[i]) − T[ i ] )												(eqn. 2)

where wP + E[i]) = [ x y z+E[i] ], E[i] = e1* 𝞡3 + e2* 𝞡5 + e3* 𝞡7 + e4* 𝞡9, 𝞡 is the incidence angle pertaining to each ray formed by the pixel cP[ i ] = [ u v 1 ], and [e1, e2, e3, e4] are entrance pupil coefficients. In addition, the accuracy of these entrance pupil parameters have an influence of the accuracy of estimated extrinsic parameters and thus improve the future imaging tasks. If not available, vector E is considered as 0 and a fallback to eqn. 1 is expected.

More specifically, the following intrinsic camera parameters can be signalled in the SDP for each camera as per ISO/IEC 23008-2 [3]:

focalLengthX[ i ] specifies the focal length of the i-th camera in the horizontal direction as a signed floating-point number.

focalLengthY[ i ] specifies the focal length of the i-th camera in the vertical direction as a signed floating-point number.

principalPointX[ i ] specifies the principal point of the i-th camera in the horizontal direction as a signed floating-point number.

principalPointY[ i ] specifies the principal point of the i-th camera in the vertical direction as a signed floating-point number.

skewFactor[ i ] specifies the skew factor of the i-th camera as a signed floating-point number. The value of exponent_skew_factor[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use.

The intrinsic matrix A[ i ] for i-th camera is represented by:

It is possible that the intrinsic camera parameters are equal for all of the cameras. In that case, only one set of values based on the above parameters would need to be signalled, e.g., via SDP signalling at the session level.

Furthermore, the following extrinsic camera parameters can be signalled in the SDP for each camera as per ISO/IEC 23008-2 [3]:

rE[ i ][ j ][ k ] specifies the ( j, k ) component of the rotation matrix for the i-th camera as a signed floating-point number.

The rotation matrix R[ i ] for i-th camera is represented as follows:

tE[ i ][ j ] specifies the j-th component of the translation vector for the i-th camera as a signed floating-point number.

The translation vector T[ i ] for the i-th camera is represented by:

E[ i ][ j ] specifies the j-th component of the entrance pupil vector E[i] for the i-th camera as a signed floating-point number, as per eqn (2) above.

# Proposal

It is proposed to adopt the proposed updates to the text in Section 2 in the ITT4RT permanent document.