3GPP TSG SA WG4 #110	S4-201070

# 19th – 28th August 2020	revision of S4- AHIA07

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | 2 | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | M7d APIs for 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Client APIs for M7d unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add M7d framework and instantiation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No-one never ever can use 5GMS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 13, 14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update against S4-AHIA01 |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]		DASH Industry Forum, "Specification of Live Media Ingest", 
https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[4]	3GPP TS 26.247: "Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	Standard ECMA-262, 5.1 Edition, "ECMAScript Language Specification", June 2011.

[6]	IETF RFC 6234: "US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF)".

[7]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[8]	ITU-T Recommendation X.509 (2005) | ISO/IEC 9594-8:2005: "Information Technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks".

[9]	IETF RFC 7230: "Hypertext-Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[10]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[11]	IEEE Standard 1003.1, Issue 7: "The Open Group Base Specifications", 2018.
https://pubs.opengroup.org/onlinepubs/9699919799/

[12]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[13]	ISO/IEC 23009-1: "Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

=====  CHANGE  =====

## 13.1 	DASH Media Player – APIs and Functions

### 13.1.1 	Overview

In the following, it is assumed that the Media Player (in this case a DASH client) adheres to a basic set of functionalities as shown in Figure 13.1-1. The DASH client downloads, processes and presents a DASH Media Presentation by instruction of a 5GMSd-Aware Application.

The 5GMSd-Aware Application can, in addition, configure the presentation of the media, can receive notifications on events, or can query the internal status of the DASH Player and Access Client. Different functions are provided in the DASH Access client that are typically necessary to process a DASH Media Presentation. Other functions may be available as well.

Different information in DASH Media Presentations are of use for different components of the client. The key functionalities of each of the functions are provided as follows:

- 5GMSd-Aware Application: Application that makes use of the DASH/Media Player to playback a DASH Media Presentation using the APIs defined in this clause.

- Media Player: A complete player for the playback of a Media Presentation, including media decoders as defined in TS26.511.

- Access Client: A part of the DASH Player accesses and downloads of the resources and provides the downloaded resources to the Media Playback Platform and Content Decryption for the playback of DASH content.

- Management: Controls all internal processes and the communication with the 5GMSd-aware application.

- MPD Processing:  parses and processes the MPD and extracts the relevant information

- Adaptation Set Selection: selects the Adaptation Set based on user, application and/or device capability information

- ABR Controller and Dynamic Switching: runs adaptive bitrate logic and triggers adaptive switching of Representations

- Throughput Estimation: estimates the throughput from the 5GMSd Application Server

- Metrics Collection: collects the relevant metrics and provides those to the metrics reporting function in the MSH

- Media Playback Management and Protection Controller: manages the media playback by moving downloaded information into media playback platform and also addresses handling of protection and DRM related information.

- Media Playback Platform and Content Decryption: Plays back CMAF-based media content according to the playback requirements in TS26.511.

- Event Processing: Processes DASH events and provides information to application.

Figure 13.1.1-1 DASH Client Architecture

This clause focuses on Media Player related communication through M7d. In particular, the following aspects of M7d are defined:

- Methods to interact with the Media Player are defined in clause 13.1.3

- Notification and Error Events are defined in clause 13.1.4

- Configuration and Settings APIs are defined in clause 13.1.5

- Status Information API is defined in clause 13.1.6

The communication to the media playback platform is defined through the details in TS 26.511 [12].

A 5GMSd client shall support the APIs defined in this clause 13.

### 13.1.2 Media Player model

#### 13.1.2.1 State Model

Figure 13.1.2.1-1 provides an informative client state model in order to appropriately describe the messages on the Media streaming service API. Six different states are defined.

State changes may happen based on:

-	Calls from application

-	Information provided in the Media Presentation Description (MPD)

Figure 13.1.2.1-1: State Diagram for Media Player

Table 13.1.2-1-1 defines states for the Media Player. Detailed descriptions are provided in the following subclauses.

Table 13.1.2-1: States of Media Player

| States | Definition |
| --- | --- |
| IDLE | The Media Player is not associated with any application. |
| INITIALIZED | The Media Player is associated with the application and the API communication is established. |
| READY | The Media Player has loaded an MPD and is able to playback the media in this Media Presentation. It also updates the MPD according to the MPD update mechanism. |
| PRELOADED | The Media Player has pre-loaded all media information in order to start playback instantaneously. It also updates the MPD according to the MPD update mechanism. |
| PLAYING | The Media Player playing the Media Presentation. It also updates the MPD according to the MPD update mechanism. |
| PAUSED | The playback of the Media Presentation is paused. It also updates the MPD according to the MPD update mechanism. |

It is assumed that the player is associated with the playback of a single track for each media type, namely video, audio and subtitles as defined in TS 26.511 [12].

#### 13.1.2.2 Player internal properties

The Media Player client maintains internal properties as defined Table 13.1.2.2-1. Note that the parameters are conceptual and internal and only serve for the purpose to describe message generation on the API calls. However, at the same time it is expected that the parameters can be configured and accessed from outside through the APIs defined in clause 13.1.4 and 13.1.6.

Table 13.1.2.2-1: Media Player internal properties

| States and Parameters |  |  | Definition |
| --- | --- | --- | --- |
| _configuration |  |  | See clause 13.1.4, Table 13.1.4-1. |
| _status |  |  | See clause 13.1.6, Table 13.1.6-1. |

#### 13.1.2.3 Media Player Internal Operations

Media Player internal operations are not defined.

### 13.1.3 Methods

#### 13.1.3.1	General

Based on the procedures in clause 13.1.2 this clause introduces relevant APIs.

Table 13.1.3.1-1 provides an overview over the methods defined for the DASH-based streaming API.

Table 13.1.3.1-1: Methods defined for DASH Streaming API

| Method | State after success | Brief description | Clause |
| --- | --- | --- | --- |
| initialize() | INITIALIZED | The Media Player is created | 13.1.3.2 |
| attach(MPD) | READY | sets a source URL to an MPD file or a previously downloaded and parsed MPD | 13.1.3.3 |
| preload(MPD) | PRELOADED | Streaming the media is initiated | 13.1.3.4 |
| play(MPD) | PLAYING | Playback of the media is initiated | 13.1.3.5 |
| pause() | PAUSED | Playback of the media is paused. | 13.1.3.6 |
| seek(MPD,time) | PLAYING | The playback time of the media is altered | 13.1.3.7 |
| reset() | INITIALIZED | All media related information is reset. | 13.1.3.8 |
| destroy() | IDLE | All media player related information is reset and API communication is stopped | 13.1.3.9 |

#### 13.1.3.2	Initialize

This clause defines the initialize() method.

The Media Player is created by initializing using the initialize() method. The following functions are initialized:

- Media Playback Management in order to enable API-based communication through M7d.

- In particular, the M7d Notifications and Errors API (see clause 13.1.4) and the Status Query (see clause 13.1.5) are established.

- The Request Scheduling and Download functions are established.

#### 13.1.3.3	Attach

This clause defines the attach() method.

The following pre-conditions apply:

- The MediaPlayer must be in INITIALIZED state

An 5GMSd-Aware Application calls attachMPD() to set a source URL to an MPD file or a previously downloaded and parsed MPD.

The parameters of the method are defined in Table 13.1.3.3-1.

Table 13.1.3.3-1 Parameters for attachMPD()

| Name | Type | Description |
| --- | --- | --- |
| urlOrMPD | string ¦ Object | A URL to a valid MPD or a valid MPD as defined in ISO/IEC 23009-1 [13] or TS 26.247 [4]. The URL may be augmented by MPD Anchors as defined in ISO/IEC 23009-1 [13], Annex C.4. |

The following Media Player Actions are expected:

- The Request Scheduling and Download functions are established.

- If the input is a URL, the Media Player requests the MPD at the corresponding URL through M4d.

- The MPD Processing function is established and the MPD parsed.

- If the MPD is not valid, an error ERROR_MPD_NOT_VALID is returned and the process is terminated.

- If the DASH Player does not support the profiles as indicated in the MPD, an error ERROR_PROFILE_NOT_SUPPORTED is returned and the process is terminated.

- Depending on the type of the MPD, possibly present anchors as well as the wall-clock time, the Media Player selects the Period in the content that is expected to be played next.

- The Media Playback Management and Protection Controller is established.

- The Adaptation Set Selection is established. By using capability mechanisms defined in TS26.511 as well as using other information (language settings, output capabilities, accessibility settings), the Media Player selects Adaptation Sets that are played. If no Adaptation Sets are capable to be played, an error ERROR_MEDIA_NOT_SUPPORTED is returned and the process is terminated.

- Media Playback Platform and Content Decryption is established using the methods defined in TS 26.511.

- The selected Adaptation Sets are initialized by downloading the relevant CMAF Headers through M4d in the Media Playback Platform as in TS26.511.

- Depending on the MPD information and/or M7d configuration, one or more of the following functions may be established

  - Metrics Collection

  - Event Processing

  - Client Metadata handling

- The Media Player is left in the READY state

An application may use this method to load an MPD and in order to prepare playback. In case of errors notifications, it is up to the application to initiate appropriate actions.

#### 13.1.3.4	Pre-load

This clause defines the preload() method.

The following pre-conditions apply:

- The MediaPlayer must be in INITIALIZED or READY state

An 5GMSd-Aware Application calls preload() to cause the player to begin streaming the media as set by the  attach() method in preparation for playing.

The parameters of the method are defined in Table 13.1.3.3-1.

Table 13.1.3.2-1 Parameters for attachSource()

| Name | Type | Description |
| --- | --- | --- |
| urlOrMPD | string ¦ Object | A URL to a valid MPD or a valid MPD as defined in ISO/IEC 23009-1 [13] or TS 26.247 [4]. The URL may be augmented by MPD Anchors as defined in ISO/IEC 23009-1 [13], Annex C.4. |

The following Media Player Actions are expected:

- If in INITIALIZED state, the attach() method is invoked.

- Depending on the type of the MPD, possibly present anchors as well as the wall-clock time, and other MPD information, the earliest media time span for pre-loading is identified.

- The access client schedules and generates requests for the relevant media segments based on the ABR Controller information, as well as the throughput estimation and downloads this media.

- The media is downloaded from the corresponding URL through M4d earliest at the segment availability start time of the media segment.

- The media is appropriately appended to the Media Playback Platform and Content Decryption APIs, following the description in TS26.511 for playback requirements.

- In case of a dynamic services, the content is continuously loaded to remain at the live edge. Content not at the live edge is removed. For static services, the content is loaded up to a suitable buffer duration and then downloading is stopped.

- Appropriate notifications and error messages are generated. For details refer to clause 13.1.5.

- Appropriate Status Information is generated. For details refer to clause 13.1.6.

- The Media Player is in PRELOADED state.

An application may use this method to preload media into the player in order minimize the start-up time.

#### 13.1.3.5	Play

This clause defines the play() method.

The following pre-conditions apply:

- The MediaPlayer must be in INITIALIZED or READY or PRELOAD or PAUSE state.

An 5GMSd-Aware Application calls play() to cause the player to begin streaming the media as set by the  attach() method.

The parameters of the method are defined in Table 13.1.3.5-1.

Table 13.1.3.5-1 Parameters for play()

| Name | Type | Description |
| --- | --- | --- |
| urlOrMPD | string ¦ Object | A URL to a valid MPD or a valid MPD as defined in ISO/IEC 23009-1 [13] or TS 26.247 [4]. The URL may be augmented by MPD Anchors as defined in ISO/IEC 23009-1 [13], Annex C.4. |

The following Media Player Actions are expected:

- If in INITIALIZED state, the attach() method is invoked.

- If in PAUSE state, the earliest media time is MEDIA_TIME, else, depending on the type of the MPD, possibly present anchors as well as the wall-clock time, and other MPD information, the earliest media time for start-up is identified.

- The access client checks the available buffer state of media in the Media Playback Platform. Based on this, the access client schedules and generates requests for the relevant media segments based on the ABR Controller information, as well as the throughput estimation and downloads this media.

- The media is downloaded from the corresponding URL through M4d earliest at the segment availability start time of the media.

- The media is appropriately appended to the Media Playback Platform and Content Decryption APIs, following the description in TS26.511 for playback requirements.

- Once a threshold for sufficient buffering is reached, the Media Playback platform is initiated to be started, i.e. a play is initiated, following the description in TS26.511 for playback requirements.

- The content is continuously and played back.

- Appropriate notifications and error messages are generated. For details refer to clause 13.1.4.

- Appropriate Status Information is generated. For details refer to clause 13.1.5.

- The Media Player is in PLAYING state.

An application may use this method to initiate playback of media.

#### 13.1.3.6	Pause

This clause defines pause() method.

The following pre-conditions apply:

- The Media Player must be in PLAYING state.

An 5GMSd-Aware Application calls pause() to cause the media Playback Platform to pause playback.

No parameters are attached.

The following Media Player Actions are expected:

- The playback on the playback platform is paused and the media time is maintained as MEDIA_TIME.

- The access client checks the available buffer state of media in the Media Playback Platform. Based on this, the access client schedules and generates requests for the relevant media segments based on the ABR Controller information, as well as the throughput estimation and downloads this media.

- The media is downloaded from the corresponding URL through M4d earliest at the segment availability start time of the media.

- The media is appropriately appended to the Media Playback Platform and Content Decryption APIs, following the description in TS26.511 for playback requirements.

- Once the buffers are sufficiently filled, the client stops downloading

- Appropriate notifications and error messages are generated. For details refer to clause 13.1.4.

- Appropriate Status Information is generated. For details refer to clause 13.1.5.

- The Media Player is in PAUSE state.

An application may use this method to playback media.

#### 13.1.3.7	Seek

This clause defines seek() method.

The following pre-conditions apply:

- The MediaPlayer must be in INITIALIZED, READY, PRELOADED or PAUSED state.

An 5GMSd-Aware Application calls seek() to cause the player to go a specific media time.

The parameters of the method are defined in Table 13.1.3.3-1.

Table 13.1.3.3-1 Parameters for seek()

| Name | Type | Description |
| --- | --- | --- |
| urlOrMPD | string ¦ Object | A URL to a valid MPD or a valid MPD. The URL may be augmented by MPD Anchors as defined in ISO/IEC 23009-1 [X], Annex C.4. |
| mediaTime | Unsigned integer | The media time in milliseconds for playback. |

The following Media Player Actions are expected:

- If in INITIALIZED state, the attach() method is carried out.

- If the mediaTime is not accessible return an error ERROR_MEDIA_TIME_NOT_ACCESSIBLE and terminate the process.

- The earliest media time is set to the mediaTime.

- The state is set to PAUSE.

- The play() command is issued.

An application may use this method to initiate playback of media.

#### 13.1.3.8	Reset

This clause defines the reset() method.

The following pre-conditions apply:

- The MediaPlayer may be in any state.

An 5GMSd-Aware Application calls reset()resets all information related to the media and the Media Presentation described by the MPD is destroyed.

No parameters are attached.

The following Media Player Actions are expected:

- The playback on the playback platform terminated.

- All open requests are cancelled.

- All scheduled requests are deleted.

- The current MPD is removed.

- The Media Player is left in the INITIALIZED state.

An application may use this method to terminate the playback of any media.

#### 13.1.3.9	Destroy

This clause defines destroy() method.

The following pre-conditions apply:

- The MediaPlayer may be in any state.

An 5GMSd-Aware Application calls destroy()resets all information related to the media and the network.

No parameters are attached.

The following Media Player Actions are expected:

- The playback on the playback platform terminated.

- All open requests are cancelled.

- All scheduled requests are deleted.

- The current MPD is removed.

- All network information is history is cleared.

- The Media Player is left in the IDLE state.

An application may use this method to terminate the playback of any media clear and download related information.

### 13.1.4 Configurations and settings API

Table 13.1.4-1 provides a list of configurations and setting options.

Table 13.1.4-1 Configuration API

| Status | Type |  | Parameter |  |  |  |  | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| source | object |  |  |  |  |  |  | The latest MPD. |  |  |
| maxBufferTime | Integer |  |  |  |  |  |  |  | maximum buffer time in milliseconds for the service. |  |
| clientMetadata |  | Object |  |  |  |  |  |  |  | Sets parameters for client metadata collection, see cmcd settings in dash.js |
| metricsConfiguration[] | Object |  |  |  |  |  | Defines the setting for collecting metrics. |  |  |  |

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/module-MediaPlayer.html#setAutoPlay aand http://cdn.dashjs.org/latest/jsdoc/module-Settings.html

### 13.1.5 Notifications and error events

Table 13.1.5-1 provides a list of notification events.

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/MediaPlayerEvents.html

Table 13.1.5-1 Notification events

| Status | Definition | Payload |
| --- | --- | --- |
| AST_IN_FUTURE | Triggered when playback will not start yet as the MPD's availabilityStartTime is in the future. | Time before playback will start. |
| BUFFER_EMPTY | Triggered when the media playback platform's buffer state changes to stalled | Media Type |
| BUFFER_LOADED | Triggered when the media playback platform's buffer state changes to loaded. | Media Type |
| CAN_PLAY | Sent when enough data is available that the media can be played | n/a |
| MANIFEST_LOADED | Triggered when the manifest load is complete | n/a |
| MANIFEST_VALIDITY_CHANGED | Manifest validity changed - As a result of an MPD validity expiration event. |  |
| METRIC_ADDED | Triggered every time a new metric is added. |  |
| METRIC_CHANGED | The minimum bitrate that the ABR algorithms will choose. Use NaN for no limit. |  |
| METRIC_UPDATED | Set to true if you would like dash.js to keep downloading fragments in the background when the video element is paused. |  |
| METRICS_CHANGED | Triggered anytime there is a change to the overall metrics. |  |
| PLAYBACK_ENDED | Sent when playback completes. |  |
| PLAYBACK_ERROR | Sent when an error occurs. The element's error attribute contains more information. | Error attribute |
| PLAYBACK_PAUSED | Sent when playback is paused. |  |
| PLAYBACK_PLAYING | Sent when the media begins to play (either for the first time, after having been paused, or after ending and then restarting). |  |
| PLAYBACK_SEEKED | Sent when a seek operation completes. |  |
| PLAYBACK_SEEKING | Sent when a seek operation begins. |  |
| PLAYBACK_STALLED | Sent when the media playback platform reports stalled |  |
| PLAYBACK_STARTED | Sent when playback of the media starts after having been paused; that is, when playback is resumed after a prior pause event. |  |
| PLAYBACK_WAITING | Sent when the media playback has stopped because of a temporary lack of data. |  |
| SOURCE_INITIALIZED | Triggered when the source is setup and ready. |  |

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/MediaPlayerEvents.html

Table 13.1.5-2 provides a list of error events.

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/Errors.html

Table 13.1.5-2 Error events

| Status | Definition | Payload |
| --- | --- | --- |
| ERROR_MEDIA_PLAYBACK | Triggered when there is an error from the media playback platform buffer. |  |
| ERROR_MPD_NOT_VALID | The provided MPD is not valid according to the XML schema and schematron rules | Detailed error information |
| ERROR_MEDIA_TIME_NOT_ACCESSIBLE | After seek operation, the media time is not accessible. |  |
| ERROR_PROFILE_NOT_SUPPORTED | The profile of the Media Presentation is not supported. |  |

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/Errors.html

### 13.1.5 Configurations and Settings API

Table 13.1.5-1 provides a list configurations and settings.

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/module-MediaPlayer.html#setAutoPlay aand http://cdn.dashjs.org/latest/jsdoc/module-Settings.html

Table 13.1.6-1 Configuration API

### 13.1.6 Status Information

Table 13.1.6-1 provides a list status information that can be obtained from the client.

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/module-MediaPlayer.html#getAverageThroughput

Table 13.1.6-1 Status information

| Status | Type | Parameter |  | Definition |  |
| --- | --- | --- | --- | --- | --- |
| ConfigurationInformation |  |  | Parameters defined in Table 13.1.4-1. |
| AverageThroughput |  | none |  | Average throughput computed in the ABR logic in bit/s. |  |
| BufferLength |  | MediaType "video", "audio" and "subtitle" |  | The length of the buffer for a given media type, in seconds. If no type is passed in, then the minimum of video, audio and subtitle buffer length is returned. NaN is returned if an invalid type is requested, the presentation does not contain that type, or if no arguments are passed and the presentation does not include any adaption sets of valid media type. |  |
| liveLatency | float | none |  | Current live stream latency in seconds based on the latency measurement. |  |
| MediaSetting |  | MediaType "video", "audio" and "subtitle" |  | Media settings for each media type based on the CMAF Header and the MPD information based on the selected Adaptation Set for this media type. |  |
| MediaTime |  | None |  | Current media playback time from media playback platform. The media time is in seconds and is relative to the start of the playback and provides the media that is actually rendered. |  |
| PlaybackRate |  | None |  | The current rate of playback. For a video that is playing twice as fast as the default playback, the playbackRate value should be 2.00 |  |
| selectableMedia[] | DASH Adaptation Set Parameters |  |  | Provides the selectable media (what the player is capable to playback) by providing on CMAF Header and Adaptation Set information. |  |
| Metrics[][] | Metrics |  |  | A data blob of metrics for each defined metrics collecting scheme |  |

NOTE: this is aligned with dash.js APIs http://cdn.dashjs.org/latest/jsdoc/module-MediaPlayer.html#getAverageThroughput

### 13.1.7 Usage of M7d Information by Media Session Handler

The media session handler may use the notifications, errors and status information provided through M7d to execute relevant tasks.

## 13.2 Progressive Download Player – APIs and Functions

A progressive download player is significantly simpler than a DASH client. Figure 13.2.-1 provides an overview of a progressive download client.

Figure 13.2-1 Progressive Download Client

Detailed client internal communication is for further study.

=====  CHANGE  =====

# 14	Application (M8) APIs for uplink and downlink

APIs of this reference point are not specified within this release.

For DASH-based streaming M8d must be capable to provide MPD URLs to the application with a fully-qualified domain name (FQDN) such that the media session handler is initiated.

Editor's Note: This clause should document the external requirements for M8d for certain scenarios. For example, that an MPD URL needs to be passed, that you implement the APIs., etc.