3GPP TSG-SA4 Meeting #110-e	S4-201150

# Electronic meeting, 19th-28th Aug. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR |  | rev |  | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AV production scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 19th Aug 2020 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed clause on AV Production scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sections remains uncomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 6.X (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

(….)

[REF1]		3GPP TS 22.263: " Service requirements for video, imaging and audio for professional applications (VIAPA)".

End of First Change

Start of Second Change

## Section 6.X	Audiovisual production

### 6.x.1	Motivation

In the audiovisual environment, the video formats from capture to display are not in a Direct to Consumer mode and rather rely on a workflow of different stages adapted to the local requirements and constraints. We distinguish mainly 3 stages:

-	Production format, from capture to authoring

-	Contribution format, from production to head-end encoders.

-	Distribution format, from head-end encoders to consumer devices.

Production formats from a camera to a production plant such as an OB Van (Outside Broadcasting Van) are well defined. Proprietary formats, or lossless compressions may be used in this closed environment.

Contribution formats are used for the uplink from the production area to the head-end encoders for distribtuion. Contribution signals have stronger requirements on the quality and have the following features:

-	High bitrate in order to ensure no visible artefact that may be increased at the following encoding stage.

-	Very low gop size or even intra-only coded, so as to limit the end-to-end delay in distributing the content live and so as to easy content editing when mutilple feeds are orchestrated.

Contribution signals have been historically distributed over fiber optics or satellite uplinks. With 5G, there is an opportunity to use cellular networks, particularly for uplink from remote locations.

In 3GPP TS 22.263 [REF1] (Service requirements for video, imaging and audio for professional applications (VIAPA), clause 6.2.1 describes the performance requirements for AV production applications. Uplink signals are required to fit within 10Mb/s for HD up to 500Mb/s for Tier1 UHD.

### 6.X.2	Description of the Anticipated Application

When working remotely from a broadcasting centre there is a requirement for producers, journalists and cameras crews to contribute audio, video and other data back to a broadcast centre for inclusion in a TV programme. This is of a high quality using professional video cameras and other equipment.

It may be a single live stream of content that is intended to go straight to air in a news programme. Or a longer OB such as a live sporting event.

There is also a use case for contribution and delivery of close to air or live content where uncompressed links are unavailable.

This scenario describes the ability to use 5G connectivity as a contribution feed of video, audio and ancillary data to a TV studio (e.g. news). Targeted formats are HD and UHD. Various forms of compression may be used in order to provide a balance between quality, speed and available bandwidth. It is usually observed that compression is usually on the video side and audio and data usually remain uncompressed.

The signal source is derived from a single camera that gives out an uncompressed video feed this is then likely to be compressed before transmission via a 5G link. Different forms of compression may be required for different applications with low latency lightweight compression used where quality needs to be maintained and increased compression used where bandwidth is extremely limited, and speed of contribution is required.

### 6.X.3	Source Format Properties

Table 6.X-1 provides an overview of the different source signal properties for AV production. This information is used to select proper test sequences.

Table 6.X-1 AV production source properties

| Source format properties | Possible values |
| --- | --- |
| Spatial resolution | 1920x1080, 3840x2160 |
| Chroma format | Y’CbCr |
| Chroma subsampling | 4:2:0, 4:2:2, 4:4:4 |
| Picture aspec ratio | 16:9 |
| Frame rates | 30, 50, 60, 90, 120 Hz |
| Bit depth | 8, 10 |
| Colour space formats | BT.709, BT.2020 |
| Transfer characteristics | BT.2100 (HDR) |

### 6.X.4	Encoding and Decoding Constraints

Table 6.X-2 provides an overview of encoding and decoding constraints AV production scenario using AVC and HEVC codecs as operation points. This will support the definition of detailed test conditions.

Table 6.X-2 Encoding and Decoding Configurations for AV production

| Encoding and Decoding Constraints | AVC | HEVC |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.264/AVC High Profile Level 4.0 | Main-10 Profile; Main-4:2:2 10; Main 4:2:2 10 Intra Main Tier / High Tier Level 4.1, Level 5.1, Level 6.1 |
| RAP period | All intra / 4 frames | All intra / 4 frames |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Fixed QP | Fixed QP |
| Latency requirements and specific encoding settings | Low-latency requirements, -Short GOP -Intra only | Low-latency requirements, -Short GOP -Intra only |
| Encoding complexity context | real-time encoding | real-time encoding. |
| Required decoding capabilities | H.264/AVC High Profile Level 4.0 | H.265/HEVC Main-10 Profile Level 4.1, 5.1, 6.1 |

### 6.X.5	Performance Metrics

tbd

### 6.X.6	Interoperability Considerations

tbd

### 6.X.7	Test Sequences

[Test sequences relevant from content production in 4:2:2 format or 4:2:0 by default.]

### 6.X.8	Detailed Test Conditions

tbd

### 6.X.9	External Performance Data

tbd

### 6.X.10	Additional Information

Tbd

End of Second Change