3GPP TSG SA WG4 #110	S4-201230

# 19th – 28th August 2020	revision of S4-201215

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | 2 | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | M6d APIs for 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Client APIs for M6d unspecified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add M6d framework and instantiation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No-one never ever can use 5GMS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update against S4-AHIA07 |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]		DASH Industry Forum, "Specification of Live Media Ingest", 
https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[4]	3GPP TS 26.247: "Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	Standard ECMA-262, 5.1 Edition, "ECMAScript Language Specification", June 2011.

[6]	IETF RFC 6234: "US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF)".

[7]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[8]	ITU-T Recommendation X.509 (2005) | ISO/IEC 9594-8:2005: "Information Technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks".

[9]	IETF RFC 7230: "Hypertext-Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[10]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[11]	IEEE Standard 1003.1, Issue 7: "The Open Group Base Specifications", 2018.
https://pubs.opengroup.org/onlinepubs/9699919799/

[12]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[13]	ISO/IEC 23009-1: "Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

=====  CHANGE  =====

# 12	UE Media Session Handling (M6) APIs for uplink and downlink

## 12.1 General

## This clause defines the client APIs for Media Session Handling to be used by other 5G System components such as a Media Player in a 5GMSd client or tha Media Streamer in a 5GMSu client.

## 12.2 Media Session Handling for Downlink Streaming – APIs and Functions

### 12.2.1 Overview

In the following, it is assumed that the Media Session Handler for downlink streaming adheres to a basic set of functionalities as shown in Figure 12.2-1.

The Media Session Handler is considered to run as a service in the background and gets invoked for a media session, once a media player in the 5GMSd streaming cliented is activated with an MPD URL with a media MIME type "application/dash+xml". Based on the MPD URL, the media session handler may initiate communication with the 5GMSd AF through M5d.

NOTE: the initiation of the Media Session Handler for other media types than DASH is for further study.

For an ongoing 5G Media Streaming session, the Media Session Handler is given the following authorities:

- The ability to do status query on M7d. For details see clause 13.

- The ability to process notifications and error on M7d. For details see clause 13.

- The ability to configure certain parameters on the media player based on M7d. For details again see clause 13.

In addition, the MSH can provide information on M6d to the application and possibly delegated to Media Player using M6d for each of the Media Session Handler functionalities, namely providing

- Notification and Error Events;

- Status Information.

### 12.2.2 Media Session Handler model

#### 12.2.2.1 State Model

An informative state model for the Media Session Handler is for further study.

#### 12.2.2.2 Media Session Handler internal properties

The Media Session Handler maintains internal properties as defined Table 12.2.2.2-1. Note that the parameters are conceptual and internal and only serve for the purpose to describe message generation on the API calls.

Table 13.1.2.2-1: Parameters of Media Session Handler

| States and Parameters |  |  | Definition |
| --- | --- | --- | --- |
| _Configuration |  |  |  |
|  |  | _networkAssistance | network assistance configuration |
|  |  | _policyTemplate | Policy template configuration |
|  |  | _consumptionReporting | Consumption reporting configuration |
|  |  | _metricsReporting | Metrics reporting configuration |
| _status[] |  |  | The media session handler maintains a status record |

#### 12.2.2.3 Media Session Handler Internal Operations

This aspect is for further study.

#### 12.2.2.4	Starting and Stopping a Media Session Handler

There are different ways to start a Media Session Handler. The most typical one is that the MSH start is bound to the call of a Media Player with an MPD URL. That start offers a client-server like interface realized by M6d. The service is bound such that the communicate back to the Media Player.

### 12.2.3 	General

Table 12.2.3-1 provides a list status information that can be obtained from the MSH through M6d.

Table 12.2.3-1 Status Information

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |

Table 12.2.3-2 provides a list of general notification events exposed on M6d.

Table 12.2.3-2 General Notification Events

| Event |  | Definition | Payload |  |
| --- | --- | --- | --- | --- |
| SESSION_HANDLING_ACTIVATED |  | Triggered when media session handling was activated for a specific MPD URL. |  |  |
| SESSION_HANDLING_STOPPED | Triggered when media session handling stopped for a specific MPD URL. |  |

Table 12.2.3-3 provides a list of general error events through M6d.

Table 12.2.3-3 General Error Events

| Status | Definition | Payload |
| --- | --- | --- |
| ERROR_SESSION_HANDLING | Triggered when there is an error in the media session handling. | n/a |

### 12.2.4 	Dynamic Policy Information

### Details are for furthers study.

### 12.2.5 	Network Assistance Information

### Details are for furthers study.

### 12.2.6 	Consumption Reporting Information

Table 12.2.6-1 provides a list status information that can be obtained from the MSH through M6d.

Table 12.2.6-1 Status Information related to Consumption Reporting

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |
| consumptionReport[] | Object |  | The latest sent consumption report |

Table 12.2.6-2 provides a list of general notification events exposed on M6d.

Table 12.2.6-2 Notification Events related to Consumption Reporting

| Status | Definition | Payload |
| --- | --- | --- |
| CONSUMPTION_REPORTING_ACTIVATED | Informs that consumption reporting has been activated. | n/a |
| CONSUMPTION_REPORTING_STOPPED | Informs that consumption reporting has been stopped. | n/a |
| NEW_CONSUMPTION_REPORT | Informs that a new consumption report is available and has been sent. |  |

Table 12.2.6-3 provides a list of general error events through M6d.

Table 12.2.6-3 Error Events to Consumption Reporting

| Status | Definition | Payload |
| --- | --- | --- |
| ERROR_CONSUMPTION_REPORTING | Error in consumption reporting occurred. | n/a |

### 12.2.7 	Metrics Reporting Information

Details are for furthers study.

## 12.3 Media Session Handling for Uplink Streaming – APIs and Functions

Details are for furthers study.