3GPP TSG-SA4 #110-e meeting	S4-201093

- 20th – 28th August, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | - | rev | - | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMS3: AF-based Network Assistance |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Europe B.V., Ericsson LM |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | B |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  | The Stage 3 solution for AF-based Network Assistance is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | Provide the complete proposed specification of the AF-based Network Assistance feature, based on the baseline provided in contribution S4-AHIA44 from Ericsson LM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | Incomplete specification of AF-based Network Assistance in Stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 4.2, 4.7.6, 11.2.3, 11.6 |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

CHANGE 1: Clearer abstract for the feature

## 4.2	APIs relevant to Downlink Streaming

Table 4.2-1 below summarises the APIs used to provision and use the various downlink streaming features specified in TS 26.501 [2].

Table 4.2-1: Summary of APIs relevant to downlink streaming features

| 5GMSd feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content Hosting | Content is ingested, hosted and distributed by the 5GMSd AS according to a Content Hosting Configuration associated with a Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Server Certificates Provisioning API | 7.3 |
|  |  |  | Content Processing Templates Provisioning API | 7.4 |
|  |  |  | Ingest Protocols API | 7.5 |
|  |  |  | Content Hosting Provisioning API | 7.6 |
|  |  | M2d |  |  |
|  |  | M4d | DASH (TS 26.247) or 3GP (TS 26.244) |  |
| Metrics reporting | The 5GMSd Client uploads metrics reports to the 5GMSd AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Provisioning API |  |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.3 |
| Consumption Reporting | The 5GMSd Client provides feedback reports on currently consumed content according to a provisioned Consumption Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Consumption Reporting Provisioning API |  |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Consumption Reporting API | 11.4 |
| Dynamic Policy invocation | The 5GMSd Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API |  |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSd Client requests bit rate recommendations and delivery boosts from the 5GMSd AF | M5d | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |

END OF CHANGE 1

CHANGE 2: Basic description of the procedures

### 4.7.6.	Procedures for network assistance

This procedure is used by the 5GMSd Client to request Network Assistance from the 5GMSd AF.

The 5GMSd Client first creates a Network Assistance Session resource and provides information such as a nominal time period (recommendation window) for which a bit rate recommendation is sought, an optional set of usable bit rate values and optional flow identifications. The information is used by the Network Assistance function to recommend a bit rate to the 5GMSd Client.

In the case of DASH, the 5GMSd Client determines the values for the usable bit rates from the DASH MPD by creating reasonable representation combinations with media components from different adaptation sets. The choice of usable bit rates among these may be restricted by the QoS policy operational range currently in force.

The 5GMSd Client may also request a delivery boost to be provided during the indicated nominal time period.

After the Network Assistance Session resource is provisioned, the 5GMSd Client uses the Network Assistance Session identifier when requesting a bit rate recommendation.

In order to terminate a Network Assistance Session, the 5GMSd Client deletes the Network Assistance session resource.

END OF CHANGE 2

CHANGE 3: Service Access Information resource type definition

### 11.2.3	Data model

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformation resource is specified in Table 11.2.3.1-1 below:

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | Unique identification of the M1d Provisioning Session. |
| StreamingAccess | Object | 0..1 |  |
| mediaPlayerEntry | URL String | 1..1 | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| ClientConsumptionReportingConfiguration | Object | 0..1 |  |
| reportingInterval | DurationSec | 0..1 | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| serverAddresses | Array(URL String) | 1..1 | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| locationReporting | Boolean | 1..1 | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |
| samplePercentage | Percentage | 1..1 | The percentage of streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |
| DynamicPolicyInvocationConfiguration | Object | 0..1 |  |
| serverAddresses | Array(URL String) | 1..N | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| validPolicyTemplateIds | Array(String) | 1..N | A list of Policy Template identifiers which the 5GMSd Client is authorized to use. |
| ClientMetricsReportingConfiguration | Object | 0..1 |  |
| serverAddresses | Array(URL String) | 1..N | A list of 5GMSd AF addresses to which metrics reports shall be sent. (Opaque URL, following the 5GMS URL format.) |
| dataNetworkName | String | 0..1 | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero.When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| samplePercentage | Percentage | 1..1 | The percentage of streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |
| urlFilters | Array(String) | 1..N | A list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |
| metrics | Array(String) | 1..N | A list of metrics which shall be reported. |
| NetworkAssistanceConfiguration | Object | 0..1 |  |
| serverAddress | URL String | 1..1 | 5GMSd AF address that offers the APIs for 5GMSd AF-based Network Assistance, accessed by the 5GMSd Media Session Handler. (Opaque URL, following the 5GMS URL format). |

END OF CHANGE 3

CHANGE 4: AF-based Network Assistance API specification

## 11.6	AF-based Network Assistance API

### 11.6.1	Overview

If AF-based Network Assistance is supported, then the Network Assistance API component of interface M5d, as defined in the present sub-clause, is used to obtain bit rate recommendations and to issue delivery boost requests during the ongoing media streaming session.

### 11.6.3	Data model

#### 11.6.3.1	Network Assistance Session resource

The structure of the NetworkAssistanceSession resource is specified in Table 11.6.3.1-1 below.

Table 11.6.3.1-1: Definition of NetworkAssistanceSession resource

| Property name | Type | Cardinality | D | Description |
| --- | --- | --- | --- | --- |
| windowDuration | Integer | 1..1 | I | The nominal time period, expressed in seconds, during which the Network Assistance operation applies. |
| usableBitRates | [Integer] | 0..N | I | Set of available bit rates in the downlink media session, e.g. the complete set of representation bit rates derived from a DASH MPD. |
| serviceDataFlowInformation | [ServiceDataFlowInformation] | 1..N | I | Identification of the application flows for the streaming session for which Network Assistance is wanted, e.g. 2-tuple (IP addresses) or 5-tuple (IP Addresses, protocol and ports). |

#### 11.6.3.2	Bit Rate Recommendation resource

The structure of the BitRateRecommendation resource is specified in Table 11.6.3.2-1 below.

Table 11.6.3.2-1: Definition of BitRateRecommendation resource

| Property name | Type | Cardinality | D | Description |
| --- | --- | --- | --- | --- |
| bitRateRecommendation | Integer | 1..1 | O | An estimation of throughput, or the recommendation of a bit rate, for the ensuing nominal time period (i.e. value of windowDuration). The 5GMSd client should not exceed the estimated throughput over the nominal time period. |
| windowDuration | Integer | 1..1 | O | Nominal time window. |

#### 11.6.3.3	Delivery Boost resource

The structure of the DeliveryBoost resource is specified in Table 11.6.3.3-1 below.

Table 11.6.3.3-1: Definition of DeliveryBoost resource

| Property name | Type | Cardinality | D | Description |
| --- | --- | --- | --- | --- |
| boostGranted | Boolean | 1..1 | O | Indication of whether the delivery boost request can be expected to be granted during the nominal time window. |
| windowDuration | Integer | 1..1 | I/O | Nominal time window. |

### 11.6.2	Resource structure

The Network Assistance API is accessible through the following URL base path:

{apiRoot}/3gpp-m5d-network-assistance/v1/

The supported operations and the corresponding HTTP methods are listed in table 11.6.3-1 below. In each case, the sub-resource path specified in the second column shall be appended to the URL base path

Table 11.6.2-1: Network Assistance API operations and HTTP methods

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Network Assistance Session resource |  | POST | Provision a new Network Assistance Session. |
| Fetch a Network Assistance Session resource | {naSessionId} | GET | Fetch the properties of an existing Network Asssistance Session. |
| Update a Network Assistance Session resource | {naSessionId} | PUT, PATCH | Update the properties of an existing Network Assistance Session. |
| Request a bit rate recommendation | {naSessionId}/recommendation | GET | Obtain a bit rate recommendation for the next recommendation window. |
| Request a delivery boost | {naSessionId}/boostRequest | GET | Request a delivery boost for the next recommendation window. |
| Delete a Network Assistance Session resource | {naSessionId} | DELETE | Terminate a Network Assistance session. |

### 11.6.4	Operations

#### 11.6.4.1	Network Assistance session creation

The 5GMSd Client shall use the POST method to create a Network Assistance Session with the 5GMSd AF. The AF shall return the Network Assistance Session identifier if session setup was successful. Otherwise an error code shall be returned without a session identifier.

The 5GMSd Client shall use the Network Assistance Session identifier provided by the 5GMS AF to reference all subsequent API calls to the AF that apply to that Network Assistance Session.

Table 11.6.4.1-1: Network Assistance session creation API

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response code | Remarks |
|  | {naSessionId} | 1..1 | 201 OK | A NetworkAssistanceSession resource is returned if session setup was successful. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

#### 11.6.4.2	Network Assistance session resource retrieval

The 5GMSd Client shall use the GET method with no specific API argument to retrieve a Network Assistance Session resource from the 5GMS AF. The Network Assistance Session shall be identified by the naSessionId provided by the AF on session creation.

The 5GMS AF returns the Network Assistance Session resource if retrieval was successful. Otherwise an appropriate error code is returned without the session resource in case of failure.

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | {naSessionId} | 1..1 | Session idenitifier of the session for which information is to be retrieved. |  |
| Response body | Data type | Cardinality | Response code | Remarks |
|  | NetworkAssistanceSession | 1..1 | 201 OK | The requested NetworkAssistanceSession resource is returned if a valid session identifier was provided. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

#### 11.6.4.3	Bit rate recommendation request

The 5GMSd Client shall use the GET method to request a bit rate recommendation from the 5GMS AF. The Network Assistance Session shall be identified by the naSessionId provided by the AF on session creation.

The 5GMS AF shall return the recommended bit rate if the bit rate recommendation could be carried out. Otherwise an error code is returned without a recommended bit rate.

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | {naSessionId}/recommendation | 1..1 |  |  |
| Response body | Data type | Cardinality | Response code | Remarks |
|  | BitRateRecommendation | 1..1 | 201 OK | The 5GMS AF returns a BitRateRecommendation resource if the recommendation could be carried out. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

#### 11.6.4.4	Delivery boost request

The 5GMSd Client shall use the GET method to request a delivery boost from the 5GMS AF, using the specified request URL.

Except in the case of error, the 5GMS AF shall return a DeliveryBoost resource. The value of DeliveryBoost.boostGranted shall be True if the boost request was accepted for the ongoing window, or else False if it was not accepted for the ongoing window. In case of a general error then an error code is returned without a DeliveryBoost resource.

The 5GMS AF may overwrite the supplied value of DeliveryBoost.windowDuration in cases when it is offering to provide the delivery boost for a period of time that is different from the requested window duration.

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | {naSessionId}/boostRequest | 1..1 |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DeliveryBoost | 1..1 | 201 OK | The 5GMS AF returns a DeliveryBoost resource indicating whether the boost request was granted or not, and the time window of the validity of the response. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

#### 11.6.4.5	Network Assistance Session resource modification

The 5GMSd Client shall use the PUT or PATCH methods to replace the existing Network Assistance Session parameters.

The 5GMS AF shall return the resulting NetworkAssistanceSession resource in response.

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | {naSessionId}/ NetworkAssistanceSession | 1..1 | Session idenitifier of the session for which information is to be retrieved. |  |
| Response body | Data type | Cardinality | Response Code | Remarks |
|  | NetworkAssistanceSession | 1..1 | 201 OK | The Network Assistance session resource is returned if a valid session identifier was provided. |
| NOTE: The mandatory HTTP error status codes for the PUT and PATCH methods listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

#### 11.6.4.4	 Network Assistance session termination

The 5GMSd Client shall use the DELETE method to terminate the indicated Network Assistance Session.

The 5GMS AF shall return an appropriate response code.

If the call was succesful then any subsequent calls referring to the terminated session will result in the error 404 (Not Found).

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | {naSessionId} | 1..1 | Session idenitifier of the session for which information is to be retrieved. |  |
| Response body | Data type | Cardinality | Response code | Remarks |
|  |  |  | 201 OK | The AF confirms that the Network Assistance session was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

END OF CHANGE 4