3GPP TSG SA WG4#113-e                                	S4-210495

E-meeting, 6th-14th April, 2021	A revision of S4aI201156

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.802 | CR | – | rev | – | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to TR26.802 on 5GS Broadcast-Multicast User Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TELUS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_Multicast |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Added potential standardization areas and solutions |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 and 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes against baseline document TR 26.802 v0.3.0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.2	Potential Standardization Areas:

Initially, the following areas are identified as potential standardization areas:

- Create a 5GMS-independendent 5MBS User Service Architecture

- Make 5GMS + MBS one scenario

- Define the interfaces and functions independent of 5GMS

- Expect to have a new spec TS 26.502 for 5MBS User Service Architecture

### 6.2.1	5GS User Service Architecture

Figure 6.2-1: 5G MBS network architecture with 5G Media Streaming as one scenario

In order to create a 5GMS-independent Multicast-Broadcast User Service Architecture, Figure 6.2-1 provides a view of the network architectrue with 5G multicast media streaming as one scenario. In this figure, two potential standdization areas are identified:

- How  5GMSd AF and MBSF interact to support  MBS session operations and transport (i.e. xMB-C and MB2-C reference points)

- How to provide MBSTF functionality related to MBS data handling (e.g. encoding) via xMB-U and MB2-U interfaces. Based on the definition in TS 23.247, MBSTF performs generic packet transport functionalities available to any IP multicast enabled application such as framing, multiple flows, packet FEC (encoding). It also performs multicast/broadcast delivery of input files as objects or object flows. If needed, MBSTF provides media anchor for MBS data fraffic and sourcing of IP Multicast.

NEXT CHANGE

# 7	Potential Solutions

This clause provides potential solutions for the standardization areas identified in Clause 6.

## 7.1	Broadcast-Multicast User Service Layer

A layer of “MBMS user service”-like is expected to be provided by MBSF and MBSTF. Figure 7.1-1 depicts a potential solution for 5GS Multicast-broadcast user service functional entities.

Figure 7.1-1: 5GS multicast-broadcast user service functional entities

In Figure 7.1-1, 5GMSd Application Provider is external application or content specific media functionality, e.g. media creation, encoding and formatting that use 5GMSd to stream media to 5GMSd Aware applications.

5GMS AF provides various control functions to the Media Session Handler. It may realy or initate a request for different PCF treatment.

5GS broadcast-multicast User Service enables applications. It presents a complete service offering, or a set of APIs to the end-user and allows the end-user to activate or deactivate the service. 5MBS User Service provides several delivery functions. When delivering 5MBS content to a receiving application, one or more delivery functions are used. The delivery layer provides functionality such as security and key distribution, reliability control by means of forward-error-correction techniques and associated delivery procedures.

Download delivery functions: this includes file distribution (carousle and download) which uses FLUTE protocol when delivering content over 5MBS sessions. This is equivalent to the “Download Delivery Method” in TS 26.346.

The FLUTE function described in Clause 5.3.1.1 could be hosted in 5GMS AS or MBS Session in MBSTF. Figure 5.3.1.1-1 provides a view where a simplified user plane model for FLUTE as a MTSTF function.

Media segment delivery functions: this function should support ABR and chunked segment streaming for low latency. A typical use case is to deliver source media segments encapsulated in the delivery units of a multicast media transport protocol.

Transparent delivery functions: this function spport IPTV/RTP streaming use case, as well as other application data units as part of UDP or IP flows over MBS session to the UE. This delivery method complements the download delivery function and media segment delivery function, and is particularly useful for multicast and broadcast of IP-based services for which the media codecs and application protocols are defined outside of this specification.

Group communication delivery fuction: this function delivers a UDP/IP packet flow to the UE.

Editor’s Note: transparent deliver function and group communication delivery function are FFS.

Other delivery methods may be added beyond the current release.

Delivery functions may use MBS session and may make use of point-to-point sessions trhough a set of 5MBS associated procedures to deliver 5MBS content to a receiving application.

Editor’s Note: transparent deliver function and group communication delivery function are F

MBS session refers to a multicast session or a broadcast session. In Multicast MBS session, an MBS session is to deliver the multicast communication service. A multicast MBS session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it. In Broadcast MBS session, an MBS session delivers the broadcast communication service. A broadcast MBS session is characterised by the content to send and the geographical area where to distribute it.

The FLUTE function described in Clause 5.3.1.1 could be hosted in 5GMS AS or MBS Session in MBSTF. Figure 5.3.1.1-1 provides a view where a simplified user plane model for FLUTE as a MTSTF function.

Editor’s Note: how to use the 5GS broadcast-multicast user service to address key issues 1 and 4 is FFS.

END OF CHANGES