3GPP TSG-SA4 Meeting SA4#113-e 	  S4-210533

April 6th- April 14th  2021

Agenda item	12.5

Source: 	Tencent

Title: 	Update to: Bitstream Structure for ITT4RT

Document for	Agreement

# Introduction

Tile-based delivery spatially partitions the 360-degree video frames into multiple tiles or sub-pictures. Here, the ITT4RT-Tx client may send only the required high-quality tiles or sub-pictures corresponding to the viewing orientation of the user.

The tiles or sub-pictures in the user’s viewport may be prioritized and advantageously delivered at a higher quality than tiles or sub-pictures outside the viewport. As a fallback, an extra layer with, for example, a lower resolution/quality/bitrate covering the entire panorama may be used. Assuming an appropriate player design, doing so may prevent visual artifacts, for example, black areas, when the FoV changes but the new tiles or sub-pictures are not immediately delivered due to network/streaming server delay.

With respect to Random-Access, the resolution of the tiles or sub-pictures may be changed when the user moves his/her head, but only at random-access points (RAPs). A RAP can be an access unit at which the ITT4RT-Rx client can successfully start decoding the tile or sub-picture. Therefore, the response time needed for the tile to be changed depends on the tile granularity and successive RAP distance. When the user’s orientation changes, the tiles or sub-pictures currently in the viewport may need (at least partially) to be replaced by different tiles or sub-pictures. These new tiles or sub-pictures may only be switched at the next available RAP resulting in a delayed reaction to user input.

For an immersive RTP bitstream, whenever the viewport changes, the ITT4RT-Tx client needs to deliver all the frames within the random-access point period, irrespective of the position of the viewport. Therefore, if the viewport lies in the middle of the random-access period, all frames within the access period need to be delivered and decoded as shown in Fig. 1.  The tile bitstreams cannot be decoded from the point between two random-access points. Hence, this adds latency.

5

s

e

c

5

s

e

c

5

s

e

c

H

i

g

h

R

e

s

o

l

u

t

i

o

n

N

e

w

V

i

e

w

p

o

r

t

Fig. 1 Viewport changes in the conventional bitstream. The Random Access Point (RAP) location are shown with green boxes. In this example, the nominal duration of 5 seconds between two successive RAPs is shown.

As the user’s head motion increases, a larger number of tiles or sub-pictures are delivered by the ITT4RT-Tx client, thereby increasing the motion-to-high quality (M2HQ) delay. Additionally, a lot of downloaded data is not viewed by the user, hence wasted. The ITT4RT-Tx client may encode the RTP stream with RAPs at any time. Having a RTP stream with lower RAPs may increase the required bandwidth, whereas having higher RAPs may add latency.

# Proposed text

References

[x]	 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Improved Encoder Description Update 14

Encoding Recommendations

Several encoding techniques could be used to optimize the real-time transmission and consumption of 360 video in conjunction with VDP.

Some of these techniques are briefly described in this list:

- Adaptive RAP frequency:

- For high-resolution content in the viewport, the time interval between successive random-access points (RAPs) may be useful to be reduced.  When the interval between successive random-access points is small, at the time the viewport changes, on the average the ITT4RT-Tx client needs to deliver only a smaller set of frames. Hence, the delay between the change in viewport orientation and rendering the new viewport is reduced since the random-access point period is reduced and a greater number of random access points are now available. Therefore, if bandwidth is not a limitation, the ITT4RT-Tx client may encode the high-resolution RTP bitstreams with more frequent RAPs, while the bitstream for the background with a low resolution may have a longer RAP period.

- Adaptive Coding Mode: The encoder may be instructed to favor INTRA mode decisions for CUs of newly covered areas during user’s head motion. This may be performed by adjusting the thresholds and lambda values for INTRA mode selection. This will produce INTRA slices for the newly covered picture tiles. Refer to [x] on HEVC encoder description section 6.3.

- Adaptive QP: This tool varies the quantization parameter for each CU to adjust the quality. Adaptive QP may prove useful to adjust the quality of the slices and tiles that fall within the current viewport. Higher QPs may be used for slices and tiles that cover the margins. Refer to [x] on HEVC encoder description section 6.5.4.

# Proposal

We propose to add the above text in section 9.12 “Informative Discussion on Various VDP Solutions” of the PD.

# References

- Monakhov, D., Curcio, I.D. and Mate, S., 2019, September. On Data Wastage in Viewport-Dependent Streaming. In 2019 IEEE 21st International Workshop on Multimedia Signal Processing (MMSP) (pp. 1-6). IEEE.