3GPP TSG-SA4 Meeting #114-e	S4-210792

# Online,  May 19 - 28, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on ABNF syntax for 3gpp_360video |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ITT4RT |  |  |  |  |  | Date: |  |  | 2021-5-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current version of the speicification provides two different ABNF syntaxes to signal FOV. The default value for pfov is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ABNF syntax for FOV in the 3gpp_360video attribute are aligned to that of the 3gpp_fishey attribute. RFC 8841. It is allowed for a stream ID 0 data channel application to use additional data channels. It also includes editorial improvements. The default value for pfov is change to be the same as cfov. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The speicification provides two different ABNF syntaxes to signal FOV. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex Y.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start change 1 ***

Y.6.2.3 FOV

An ITT4RT-Tx client may support sending a limited 360-degree video.

  - An ITT4RT-Tx client supporting the 3gpp_360video attribute capable of sending a limited 360-degree video shall include the parameter FOV in its SDP offer to indicate the cfov (Capture FoV) as the extent (range) of the 360-degree video with respect to the unit sphere. The range is expressed in units of 2–16 degrees with an x parameter for azimuth range and a y parameter for elevation range, sent as a comma-seperated tuple. The values for azimuth range shall be in the range of 0 to 360 * 216 (i.e., 23 592 960), inclusive, and the values for elevation range shall be in the range of 0 to 180 * 216 (i.e., 11 796 480), inclusive. In the absence of cfov, the default value of x and y are 360 and 180 degrees, respectively.

  - An ITT4RT-Rx client supporting the 3gpp_360video attribute capable that wants to receive a limited 360-degree video shall include the parameter FOV in its SDP offer/answer to indicate the pfov (Preferred FoV), where pfov <= cfov in one or both the x and y dimensions when cfov is known. The pfov range is expressed in units of 2–16 degrees with an x parameter for azimuth range and a y parameter for elevation range, sent as a comma-seperated tuple. The values for azimuth range shall be in the range of 0 to 360 * 216 (i.e., 23 592 960), inclusive, and the values for elevation range shall be in the range of 0 to 180 * 216 (i.e., 11 796 480), inclusive. In the absence of pfov, the default value of x and y are 360 and 180 degrees, respectively.

  - An ITT4RT-Tx client that has received an SDP offer from an ITT4RT-Rx client with the parameter FOV shall include in its SDP answer the parameter FOV to indicate the range of the 360-degree video it will provide. The value is the same as the FOV in the SDP offer or different based on the ITT4RT-Tx client capabilities.

An ITT4RT client supporting the 3gpp_360video attribute with the FOV parameter may include the paramater FOV_CENTER in the SDP. FOV_CENTER is expressed as a comma-separated tuple (x,y), where x is the azimuth (in units of 2–16 degrees) and y is the elevation (in units of 2–16 degrees) with respect to the global coordinates such that the range defined FOV bypasses through the coordinates defined by FOV_CENTER. The values for azimuth shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive, and the values for elevation shall be in the range of −90 * 216 (i.e., −5 898 240) to 90 * 216 (i.e., 5 898 240), inclusive. The imageattr attribute indicates the resolution of the delivered content based on the cfov and pfov options.

The ABNF syntax is defined as follows

FOV						= "fov=" 1*(fovset)

fovset					=	"[x=" azimuthrange ",y=" elevationrange "]"

azimuthrange		= "0"

/ POS-DIGIT *6DIGIT

/ "1" 7DIGIT

/ "2" ("0"/"1"/"2") 6DIGIT

/ "23" ("0"/"1"/"2"/"3"/"4") 5DIGIT

/ "235" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ "2359" ("0"/"1") 3DIGIT

/ "23592" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 2DIGIT

/ "235929" ("0"/"1"/"2"/"3"/"4"/"5") DIGIT

/ "23592960"

; 0 to 23 592 960, inclusive

elevationrange		= "0"

/ POS-DIGIT *6DIGIT

/ "10" 6DIGIT

/ "11" ("0"/"1"/"2"/"3"/"4"/"5"/"6") 5DIGIT

/ "117" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ "1179" ("0"/"1"/"2"/"3"/"4"/"5") 3DIGIT

/ "11796" ("0"/"1"/"2"/"3") 2DIGIT

/ "117964" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 1DIGIT

/ "11796480"

; 0 to 11 796 480, inclusive

FOV_CENTER			= "fov_center=[x=" centerazimuth ",y=" centerelevation "]"

centerazimuth		= "0"

/ ["-"] POS-DIGIT *6DIGIT

/ ["-"] "10" 6DIGIT

/ ["-"] "11" ("0"/"1"/"2"/"3"/"4"/"5"/"6") 5DIGIT

/ ["-"] "117" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ ["-"] "1179" ("0"/"1"/"2"/"3"/"4"/"5") 3DIGIT

/ ["-"] "11796" ("0"/"1"/"2"/"3") 2DIGIT

/ ["-"] "117964" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 1DIGIT

/ "-11796480"

; -11 796 480 to 11 796 479, inclusive

centerelevation	= "0"

/ ["-"] POS-DIGIT *5DIGIT

/ ["-"] ("1"/"2"/"3"/"4") 6DIGIT

/ ["-"] "5" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 5DIGIT

/ ["-"] "58" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7"/"8") 4DIGIT

/ ["-"] "589" ("0"/"1"/"2"/"3"/"4"/"5"/"6"/"7") 3DIGIT

/ ["-"] "5898" ("0"/"1") 2DIGIT

/ ["-"] "58982" ("0"/"1"/"2"/"3") DIGIT

/ ["-"] "5898240"

; -5 898 240 to 5 898 240, inclusive

*** End change 1 ***

*** End of changes ***