3GPP TSG-SA4 Meeting #114-e	S4-210880

# 19th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR |  | rev |  | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GSTAR] Background on MPEG Scene Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSTAR |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

[4.3.j]	ISO/IEC 23090-14:2021 DIS: “Information technology — Coded representation of immersive media — Part 14: Scene Description for MPEG-I Media”

[4.3.k]	Khronos Group, The GL Transmission Format (glTF) 2.0 Specification, Available at https://github.com/KhronosGroup/glTF/tree/master/specification/2.0/

=====  CHANGE  =====

### 4.3.5	MPEG Scene Description

A key technology in enabling immersive 3D user experiences is scene description. Scene description is used to describe the composition of a 3D scene, referencing and positioning the different 2D and 3D assets in the scene. The information provided in the scene description is then used by an application to render the 3D scene properly, using techniques such as Physically-Based Rendering (PBR) that produce realistic views.

A scene description is typically organized as a directed acyclic graph, typically a plain tree-structure, that represents an object-based hierarchy of the geometry of a scene and its attributes/properties.  Nodes are organized in a parent-child hierarchy known informally as the node hierarchy. A node is called a root node when it doesn't have a parent. Any node can define a local space transformation.

Spatial transformations are represented by transformation matrices or separate transform operations such as translation, rotation, and scaling. The transformations are applied hierarchically and iteratively from the root node down to the child nodes. Scene description also support animation nodes that allow to animate properties of the corresponding objects over time.

This structure of scene description has the advantage of reduced processing complexity, e.g. while traversing the graph for rendering. An example operation that is simplified by the graph representation is the culling operation, where branches of the graph are omitted, if deemed that the parent node’s space is not visible or relevant (level of detail culling) to the rendering of the current view frustum.

To address the needs of immersive applications, MPEG is finalizing the development of a scene description solution that adds extensions to glTF to support scene description. glTF 2.0 [4.3.k] provides a solid and efficient baseline for exchangeable and interoperable scene descriptions. However, glTF 2.0 has traditionally been focused on static scenes and assets, which makes it unfit to address the requirements and needs of dynamic and rich 3D scenes in immersive environments.

As part of its effort to define solutions for immersive multimedia, MPEG has identified the following gaps in glTF 2.0:

- No support for timed media

- No support for audio

- Limited support for interactions with the scene and the assets in the scene

- No support for local and real-time media, which are crucial for example for AR experiences

Based on this analysis, MPEG has an ongoing project to extend glTF2.0 with the ability to add timed media to gltf2.0-based scenes standardized in ISO/IEC 23090-14 [4.3.j].

Additional extensions for the support of interactivity and AR are currently being developed and will be part of the MPEG Scene Description in the next phase.

MPEG also developed an architecture to guide the work on immersive media and scene description. Figure 4.3.5-1 depicts the MPEG-I architecture and defines the key interfaces.

Figure 4.3.5-1: MPEG-I architecture and defines the key interfaces

The design focuses mainly on buffers as means for data exchange throughout the media access and rendering pipeline. It also defines a Media Access Function API to request media that is referenced by the scene description, which will be made accessible through buffers.