Source:	Ericsson LM

Title:	Architecture mapping for AR conversational services and call flow

Agenda Item:	10.9 (FS_5GSTAR)

Document for:	Agreement

# Introduction

TR 26.998 describes several AR conversational services which are listed in Table 6.1. The mapping to 5G system architecture for AR conversational services needs to be defined. In the telco on 11th of May we discussed alternative options for supporting AR conversational use-cases.

In this contribution:

- we update the mapping of AR conversational services in permanent document to STAR UE

- we propose a call flow for AR two-party calls using immersive 3D media

Table 6.1 TR 26.998

| AR conferencing | 6.6 | 8. 360-degree conference meeting 9. XR Meeting 10. Convention / Poster Session 12. AR avatar multi-party calls 13. Front-facing camera video multi-party calls 19. AR Conferencing |
| --- | --- | --- |

# Proposed change

6.5	AR two-party calls

### 6.5.1	Introduction

AR Conversational services are end-to-end use-cases that include communication between two or more parties. There are different options for mapping to 5G system:

- The MTSI architecture (TS 26.114) supports audio and 2D video conversational services. Extending the MTSI architecture to support AR signaling and immersive media. This includes both MTSI/RTP and MTSI/Data channel (DC) stack options.

- Extending the 5GMS architecture (TS 26.501) to support AR conversational services by combining live uplink and live downlink. 5GMS offers basic functionality such as QoS support, reporting, and in the future also edge, which will be beneficial for all types of applications. The typical/expected QoS parameters (especially delay) need to be clarified.

- An architecture based on something different than MTSI / IMS or 5GMS, for example browser implementations such as WebRTC. WebRTC is widely deployed today for conversational services and is built on flexible ecosystem on the device side, which is important in this case since conversational AR will require significant device-side changes.

Table 1 Comparison of different architecture options for supporting AR conversational services

| Component | MTSI: RTP and DC (TS 26.114) | 5GMS/HTTP (26.501) | Other architecture (e.g. WebRTC-based) |
| --- | --- | --- | --- |
| Protocol | SIP- and RTP-based or DC-based. SDP signaling and formats for AR are missing and need to be defined. Encoding and decoding at MTSI client needs to be extended beyond ITT4RT with improved support of AR immersive media formats (e.g. meshes, point clouds). | TCP- and HTTP-based streaming, using DASH/HLS and MPEG OMAF/CMAF technology. AR & immersive media content and signaling are assumed to work with HTTP-based streaming in the other use-case mappings. | For example, using non-IMS WebRTC data channel and/or extending WebRTC audio/video for AR media such as immersive media communications. AR signaling aspects to be studied. |
| Connection establishment | Find and connect is solved through SIP and E.164 addressing in IMS. | Find-and-connect for the conversational, UE-to-UE, case is undefined. | WebRTC implementations offer dedicated APIs for connection establishment in various contexts such as social media platforms. Browser applications are widely available. |
| Performance | Technically possible, latency should in principle not be a problem to achieve, building on the existing QoS and policy framework in 5GC. | Low latency and QoS DASH support in 5GMS to be studied. | WebRTC is designed with low latency in mind but has no defined relation to QoS and policy framework in 5GC and use of that need to be studied. |
| Deployments | Cross-operator interconnect aspects are included. Edge processing functions to be studied. | Cross-operator interconnect aspects are currently ignored. Edge processing functions to be studied (e.g. EMSA). | Cross-operator interconnect aspects are currently not applicable since WebRTC is used OTT today, but will become relevant and need study, especially if used with QoS. Edge processing functions to be studied. |
| Legal Intercept (only in scope of SA3, not SA4) | LI framework exist. Possible extensions to cover new AR media formats to be studied in SA3. | Not in scope since it is not a telephony service. | Not in scope if only OTT. |

NOTE:	There is no support of WebRTC media stack in 3GPP today, except for the WebRTC data channel stack in MTSI. WebRTC access to IMS was studied in TR 23.701 and TR 23.706, and OTT WebRTC client access to 3GPP core network through a gateway is specified in TS 24.371.

Editor’s note: It is FFS whether the study of the other architecture is limited to OTT, where the subsequent scope would be to deal with interworking through a gateway, or a completely new thing with more native WebRTC support.

### 6.5.2	Architecture mapping to AR conferencing use-case (STAR UE)

To describe the functional architecture for AR conversational use-cases (e.g. clause A5 in TR 26.998) and identify the content delivery protocols and performance indicators an end-to-end architecture is proposed. The end-to-end workflow for AR conferencing (one direction) is shown in Figure 1A. Camera is capturing the participant in an AR conferencing scenario. The camera is connected to a UE (e.g. laptop) via a data network (wired/wireless). Live camera feed, sensors and audio signals are provided to a UE/Edge node (or split) which processes, encodes, and transmits immersive media content to the 5G system for distribution. The immersive media processing function can include pre-processing of the captured 3D video, format conversion, and any other processing needed before compression. Immersive media content includes 3D representation, such as in form of meshes or point clouds, of participants in an AR conferencing scenario.  After processing and encoding, the compressed 3D video and audio streams are transmitted over the 5G system. A 5G STAR UE decodes, processes and renders the 3D video and audio stream.

The use-case be extended to bi-directional by adding a 3D camera on the receiver side and AR glasses on the sender side and applying a similar workflow.

Figure 1: Extensions to device architecture for conversational services (STAR UE)

### 6.5.3	Call flow

We consider an immersive AR two party call between Alice and Bob. The end-to-end call flow is described:

[STAR UE Alice - STAR UE Bob]:  Either one of the UEs can initiate an AR immersive call by starting an application on the phone or AR glasses.

[STAR UE Alice - STAR UE Bob]: Both UEs communicate with a signaling server to establish the AR call. During the session establishment, both parties agree on the format (e.g. point clouds, triangular/polygon meshes). The exact session type and configuration depends on the capabilities of STAR UE.

[STAR UE Alice]:  Alice is captured by a depth camera embedded within the STAR UE or an external camera which generates an immersive 3D media stream (audio and video).

[STAR UE Alice]: The immersive 3D media is encoded and transmitted in real-time to Bob over the 5G system. Additional pre-processing can be applied before encoding such as format conversion.

[STAR UE Bob]: The immersive 3D media is received on on Bob’s STAR UE. The immersive 3D media stream is decoded and rendered on AR glasses. Additional postprocessing can be applied before rendering such as format conversion, customization to match the stream to rendered environment e.g. filling holes.

[STAR UE Bob]: Bob is captured by a depth camera generating an immersive 3D media which is encoded and transmitted in real-time to Alice’s AR glasses.

[STAR UE Alice]: The immersive 3D media which is received, decoded and rendered on Alice’s AR glasses.

[STAR UE Alice - STAR UE Bob]: Both UEs terminate the service at the end of the call.

NOTE: Additional call-flows that cover other AR conferencing use-cases listed in Table 6.1 can be added.

# Conclusion

It is proposed to include Section 2 in the TR.