3GPP TSG SA WG4 #115e	S4-211035

# E-meeting, 19th – 28th August 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.925 | CR | 0001 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [8K_VR_5G] Typical Traffic Characteristics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 8K_VR_5G |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The work item asks for providing typical traffic characteristics for 8K VR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of references - Addition of 5G Media Streaming - Addition of VR Bitrates The information was provided by VR-IF in incoming LS S4-210408. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No information on 8K VR typical traffic characteristics are available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.1, 4.2, 4.4, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-210611 was agreed as revision 1 during SA4#113-e, but it said draft in the cover page and was hence not progressed for SA approval. This document removes draft from cover page. |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

[44]	M. Aracena, T. Fautier, O. Oyman, "Live VR end-to-end workflows: real-life deployments and advances in VR and network technology", SMPTE 2020 Annual Technical Conference & Exhibition, November 2020, https://www.vr-if.org/wp-content/uploads/vrif2020.115.01.pdf.

[45]	3GPP TS 26.511, "5G Media Streaming (5GMS); Profiles, codecs and formats".

=====  CHANGE  =====

## 4.1	A/V Streaming - Enhanced TV

Live and on-demand audio-visual streaming and enhanced TV services are considered according to 3GPP PSS [2], 3GP-DASH [3], MBMS [4], 5G Media Streaming for downlink [45] and the media profiles in TS 26.116 [5].

=====  CHANGE  =====

## 4.2	VR 360° Streaming

Live and on-demand VR 360° streaming are considered according to 3GPP PSS [2], 3GP-DASH [3], MBMS [4] , 5G Media Streaming [45]  and the media profiles in TS 26.118 [6].

=====  CHANGE  =====

## 4.4	Live uplink A/V streaming

Professional and consumer live uplink streaming contribution of A/V content are considered according to FLUS [11] and 5G Media Streaming for uplink [45].

=====  CHANGE  =====

## 5.2	Typical streaming/broadcast 360 VR bitrates

The following bitrates apply for streaming/broadcast 360 VR.

-	Basic 360 VR: 2.5 - 25 Mbps

NOTE 1: For viewport agnostic, the 4k UHD numbers as defined in clause 5.1 apply. For viewport dependent, the bitrates likely can be reduced to half.

- 	According to [44], with HEVC at 1080p@30fps between 2.5 - 5 Mbps on average with caps at 5 – 8 Mbps.

-	HD 360 VR: 10 - 80 Mbps

NOTE 2: For viewport agnostic, the 8k UHD numbers as defined in clause 5.1 apply. For viewport dependent, the bitrates likely can be reduced to half.

- 	According to [44], with HEVC at 4K@30fps between 10 - 18 Mbps on average with caps between 15 – 25 Mbps.

-	Retinal VR: 15 - 150 Mbps

NOTE 3: For viewport agnostic, the 8k UHD numbers as defined in clause 5.1 multiplied by 4 apply. For viewport dependent, the bitrates likely can be reduced to half or even on third.

- 	According to [44], with HEVC at 8K@30fps between 30 - 35 Mbps on average with caps at 42 Mbps.

Note 4 on framerates: cinema content is usually captured and distributed at 24 fps. TV content is usually captured and distributed at 50 or 60 fps depending on the region. The above video bitrates include frame rates of up to 60fps.

Note 5 on bitrate ranges: the bitrates are dependent on codec (e.g. AVC/H.264 [14] or HEVC/H.265 [15]), on content type and whether it is live or on-demand. Bitrates are expected to be reduced with expected encoder implementation enhancements and new codecs. In the past, that reduction has been observed to be in the order of 50 % every 10 years.