3GPP TSG-SA4 Meeting #115e	S4-211106

# Electronic meeting, Telco, August 18-27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0013 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TS 26.512 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some mistakes as well as lack of specificity in TS 26.512. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following corrections/changes are proposed: - Deletion of the entries serverCertificateIds, contentPreparation‌TemplateIds, metricsReporting‌ConfigurationIds and policyTemplateIds, currently defined as properties of the ProvisioningSession resource, from Table 7.2.3.1-1. These parameters correspond to sub-resources (not properties) of the ProvisioningSession resource. It also doesn’t make sense to define 5G Application Provider access to them in two different ways: a) indirectly as a property of ProvisioningSession and b) directly via the sub-resource path. - Addition of clarification text to clause 7.5.1 regarding use of the Content Protocols Discovery API for uplink media. - Inclusion of the configurationReportingConfigurationId property in Table 7.7.3.1-1, as well as clarification text added to the description of two existing properties in that table. - Clarification text regarding the descriptions of the reportingInterval samplePercentage properties of the MetricsReportingConfiguration resource in Table 7.8.3-1. - Correction of term “authorities” in the txt under clause 12.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect or inadequately described stage 3 text that could hamper correct and interoperable implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.1, 7.5.1, 7.7.3.1, 7.8.3, 12.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE: Table 7.2.3.1-1

### 7.2.3	Data model

#### 7.2.3.1	ProvisioningSession resource

The data model for the ProvisioningSession resource is specified in Table 7.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session indicated in the provisioningSessionType property, and this is specified in the Applicability column.

Table 7.2.3.1-1: Definition of ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | C: R R: RO | A unique identifier for this Provisioning Session. | All types. |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | C: RW R: RO U: – | The type of Provisioning Session. | All types. |
| aspId | AspId | 0..1 | C: W R: RO | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [34]. | All types. |

END OF 1st CHANGE

2nd CHANGE: Clause 7.5.1

## 7.5	Content Protocols Discovery API

### 7.5.1	Overview

The Content Protocols Discovery API is used by a 5GMS Application Provider to find out which content ingest or egest protocols are supported by the 5GMS AS instance(s) associated with a 5GMS AF. One of the supported ingest protocols is subsequently indicated in a Content Hosting Configuration for downlink media streaming. In the case of uplink streaming, one of the supported egest protocols is subsequently indicated in a corresponding Content Hosting Configuration.

END OF 2nd CHANGE

3rd CHANGE: Clause 7.7.3.1

### 7.7.3	Data model

#### 7.7.3.1	ConsumptionReportingConfiguration resource

The data model for the ConsumptionReportingConfiguration resource is specified in table 7.7.3.1-1.

Table 7.7.3.1-1: ConsumptionReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| configurationReportingConfigurationId | ResourceId | 1..1 | An identifier for this Configuration Reporting Configuration that is unique within the scope of the enclosing Provisioning Session. |
| reportingInterval | DurationSec | 0..1 | The interval between two consecutive consumption reports. The value shall be greater than zero. If absent, a single final report shall be sent immediately after the media streaming session has ended. |
| samplePercentage | Percentage | 0..1 | The proportion of media streaming clients that shall report media consumption, expressed as a floating point value between 0.0 and 100.0. If not specified, all clients shall send consumption reports. |
| locationReporting | boolean | 0..1 | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |

END OF 3rd CHANGE

4th CHANGE: Clause 7.8.3

### 7.8.3	Data model

#### 7.8.3.1	MetricsReportingConfiguration resource

The data model for the MetricsReportingConfiguration resource is specified in table 7.8.3-1 below:

Table 7.8.3-1: Definition of MetricsReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| metricsReportingConfigurationId | ResourceId | 1..1 | An identifier for this Metrics Reporting Configuration that is unique within the scope of the enclosing Provisioning Session. |
| scheme | Uri | 0..1 | The scheme associated with this Metrics Reporting Configuration. A scheme may be associated with 3GPP or with a non-3GPP entity. For downlink media streaming, if not specified, the 3GPP metrics scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 from TS 26.247 shall apply. For uplink media streaming, if not specified, the implication is that no associated uplink metrics reporting shall be performed. |
| dataNetworkName | Dnn | 0..1 | The Data Network Name (DNN) which shall be used when sending metrics reports. If not specified, the default DNN shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval between successive metrics reports. The value shall be greater than zero. If not specified, a single final report shall be sent after the media streaming session has ended. |
| samplePercentage | Percentage | 0..1 | The proportion of media streaming sessions for which metrics shall be reported, expressed as a floating point value between 0.0 and 100.0. If not specified, reports shall be sent for all sessions. |
| urlFilters | Array(String) | 0..1 | A non-empty list of content URL patterns for which metrics shall be reported. If not specified, reporting shall be done for all URLs. |
| metrics | Array(String) | 0..1 | A non-empty list of metrics which shall be collected and reported. In the case of downlink media streaming and for the 3GPP scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 the listed metrics shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [7], and the quality reporting scheme and quality reporting protocol as defined in clauses 10.5 and 10.6, respectively, of [7] shall be used. In the case of uplink streaming, no standardized metrics nor metrics reporting protocol are defined in the present document. It is assumed that those quality metrics and reporting protocol are defined by the metrics scheme. If not specified, the complete (or default if applicable) set of metrics associated with the specified scheme shall be collected and reported. |

END OF 4th CHANGE

5th CHANGE: Clause 12.2.1

## 12.2	Media Session Handling for Downlink media streaming – APIs and Functions

### 12.2.1	Overview

In the following, it is assumed that the Media Session Handler for downlink media streaming adheres to a basic set of functionalities as shown in Figure 12.2.1-1.

Figure 12.2.1-1: Usage of M6d in Media Downlink Streaming

The Media Session Handler is considered to run as a service in the background, and is invoked for a media session once a media player in the 5GMSd streaming client is activated with an MPD URL of media MIME type "application/dash+xml". Based on the MPD URL, the Media Session Handler may initiate communication with the 5GMSd AF through M5d.

NOTE:	The initiation of the Media Session Handler for other media types than DASH is for further study.

For an ongoing 5G Media Streaming session, the Media Session Handler is given the following authorizations:

1)	The ability to do status query on M7d. For details see clause 13.

2)	The ability to process notifications and error on M7d. For details see clause 13.

3)	The ability to configure certain parameters on the media player based on M7d. For details again see clause 13.

In addition, the MSH can provide information on M6d to the application and possibly delegated to Media Player using M6d for each of the Media Session Handler functionalities, namely providing:

1)	Notification and Error Events;

2)	Status Information.

END OF 5th CHANGE