Source:	Samsung Electronics Co., Ltd.

Title:	[FS_5GSTAR] Procedures and call flows for interactive immersive services

Agenda Item:	11.10

Document for:	Discussion and Agreement

- Introduction

This contribution introduces the procedures and call flows for the 5G interactive immersive services use case in 5GSTAR, using STAR-based and EDGAR-based UEs.  The procedures and call flows reference the consolidated case for downlink streaming.

- 5G Interactive Immersive Services

A core use case for 5G interactive immersive services is AR gaming, as described in A.6 of TR26.998.  In comparison to 5G downlink streaming, additional interaction information can be used by the STAR UE at the rendering stage (when all media are available to the UE), or by the Interactive Immersive Server (which can select or generate media based on the interaction information received from the UE).

- Typical Procedures and Call Flows for 5G Interactive Immersive Services using a STAR-based UE

# Figure 6.3.4.1-1 illustrates the procedure diagram for 5G interactive immersive services using a STAR-based UE when all essential AR/MR functions in an UE are available.

Figure 6.3.4.1-1: STAR-based high level procedure for 5G interactive immersive services

Prerequisites and assumptions:

-	The AR/MR Scene Manager includes immersive media rendering and scene graph handling functionalities.

-	The Media Player includes immersive content delivery and immersive media decoding functionalities.

-	The AR/MR Application in the STAR UE is run by the user.

-	The STAR UE initialises AR registration (starts analysing the surroundings where a user/UE is located), it namely:

a.	captures its surroundings via camera(s)

b.	analyses where the device is located

c.	registers the device into the analysed surroundings.

-	AR/MR Application and AR/MR Application Provider have exchanged some information, such as device capability or content configuration, for content rendering.

Editor’s Note: The exchange procedures for device capability and content configuration are FFS.

-	AR/MR Application Provider has established a Provisioning Session and its detailed configurations has been exchanged.

-	AR/MR Application Provider has completed to set up ingesting immersive contents.

Procedures:

1.	Service Announcement is triggered by AR/MR Application. Service Access Information including Media Client Entry or a reference to the Service Access Information is provided through M8d interface.

2.	Desired media content is selected.

3.	The AR/MR Application triggers the Media Client to start media playback.  The Media Client Entry is provided to the Media Client.

4.	When the AR/MR Application has received only a reference to the Service Access Information (see step 1), the Media Session Handler interacts with the Media AF to acquire the whole Service Access Information.

5.	In parallel, the Media Client is invoked to start media access and playback.

6.	The Media Client establishes the transport session to acquire entry point information. Entry point information may or may not correspond to a delivery manifest for streaming AR content, and may be a scene description, DASH MPD, or a document specific to AR/MR services.

7.	The Media Client requests the entry point.

8.	Media AS provides the entry point.

9.	The Media Client and/or AR/MR Scene Manager processes the entry point to acquire the necessary information for accessing media content.

10.	The Media Client and/or AR/MR Scene Manager notifies the necessary information acquired from the entry point to the Media Session Handler.

11.	The Media Session Handler shares the information with the Media AF, in some cases including desired QoS information.  Based on existing provisioning by the AR/MR Application Provider, the Media AF may request QoS modifications to the PDU sessions.

Media content delivery manifest fetching procedure for each AR object.  For static AR objects, a simple URL may be provided in the entry point information for downloading the AR object media data.:

12.	For the required media content, the Media Client establishes the transport session(s) to acquire delivery manifest(s) information.

13.	The Media Client requests the delivery manifest(s).

14.	Media AS provides the delivery manifest(s).

15.	The Media Client processes the delivery manifest(s).  It determines for example the number of needed transport sessions for media acquisition.  The Media Player should be able to use the delivery manifest(s) information to initialize the media pipelines for each media stream.

16.	The Media Client notifies the Media Session Handler about the delivery manifest(s).

17.	The Media Client configures the media playback pipelines.

18.	The Media Client establishes the transport session(s) to acquire the media content.

19.	The Media Client notifies to the Media Session Handler that the playback is ready.

20.	The Media Client requests the immersive media data according to the delivery manifest processed, possibly taking into account, and sending pose information for further processing (e.g., viewport dependent streaming, interactive media generation/selection)

21. The UE AR/MR Application may send interaction and/or pose information to the AR/MR Application in the Interactive Immersive Server directly.

22. The AR/MR Application may optionally generate, or select media based on the received interaction and/or pose information from the UE.

23. The Media AS receives the interactive immersive media.

24.	The Media Client receives the immersive media data and triggers the media rendering pipeline(s), including the registration of AR content into the real world accordingly.

25.	The Media Client decodes and processes the media data.

26.	The Media Client passes the media data to the AR/MR Scene Manager.

27.	The AR/MR Scene Manager renders the media, which includes the registration of the AR content into the real world accordingly.

- Typical Procedures and Call Flows for 5G Interactive Immersive Services using an EDGAR-based UE

Figure 6.3.4.2-1: EDGAR-based high level procedure for 5G interactive immersive services

Prerequisites and assumptions: Same/similar to those in a STAR UE case, as described in clause 6.3.4.1

Procedures:

1~4.	Same as steps 1~4 from the STAR UE case

5.	The Basic AR/MR Application requests the location of one or more suitable 5GMSd AS/EAS instances offering the edge capabilities that are required to serve the application.

6.	Media Session Handler requests the list of available 5GMSd AS/EAS instances and 5GMSd AF responds with the list requested.

7.	Media Session Handler selects a 5GMSd AS/EAS instance from the provided list.

8.	Media Session Handler provides the location of the selected 5GMSd AS/EAS to the Basic AR/MR Application.

9.	The Lightweight Scene Manager connects to the 5GMSd AS/EAS and provides initialization information

10.	The 5GMSd AS/EAS configures the server application accordingly and provides a customized entry point for the Lightweight Scene Manager.

11.	The Basic AR/MR Application initializes the Lightweight Scene Manager with the new entry point.

12~35. The rest of the steps are similar to steps 5~27 from the STAR UE case except step 31 where 5GMSd AS performs 2D view pre-rendering to be sent to the Media Client.

- Proposal

We propose to include the updated figures and text in sections 3 and 4 of this document as a pCR to section 6.3 of TR 26.998.