3GPP TSG-SA4 Meeting #115-e	S4-211126

# Online,  August 18 - 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on multiple 360 videos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ITT4RT |  |  |  |  |  | Date: |  |  | 2021-8-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current version of the specification provides two different syntaxes for the itt4rt_group attribute. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for multiple 360 videos may include one or more restricting groups defined using the itt4rt_group attribute. The syntax for the attribute in Y.6.2.6 is updated to support multiple groups, and the syntax in Y.6.8 is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification provides two different syntaxes for the same itt4rt_group attribute. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex Y.6.2.6, Y.6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Start change 1 ***

Y.6.2.6 Overlay and 360-degree video

An ITT4RT client that sends an SDP message with at least one 360-degree video/audio and at least one overlay shall include in SDP the attribute itt4rt_group before any media lines. The itt4rt_group attribute is used to group 360-degree media and overlay media using the mid attribute and the syntax for the SDP attribute is:

a=itt4rt_group: <group-1> / … / <group-N>

where <group-X> shall include at least one mid associated with 360-degree media and at least one mid associated with an overlay as defined by the mid attribute in the corresponding media description.

The ABNF syntax for this attribute is the following:

itt4rt-group-attrib	= "a=itt4rt_group:" rest-group *[" /" rest-group]

rest-group		= 2*(SP identification-tag)

; identification-tag is defined in RFC 5888

An ITT4RT-Tx client and an ITT4RT-Rx client may negotiate the overlays that can be associated with the 360-degree video offered by the ITT4RT-Tx client using the itt4rt_group attribute. An ITT4RT client shall indicate in an offer the overlays to be grouped with the 360-degree video using the itt4rt_group attribute. The overlays that are acceptable shall be retained in the answer and the ones that are not acceptable shall be removed. An ITT4RT-Tx client may offer overlay configuration options using the 3gpp_overlay attribute based on the list of media lines (i.e., potential overlay sources) provided in the itt4rt_group attribute in an SDP offer initiated by an ITT4RT-Rx client. The 3gpp_overlay attribute is offered in an SDP renegotiation.

The order of the media included in the itt4rt_group indicates the synchronization source with the first media always being the synchronization anchor when synchronization is required.

NOTE: 2D video received from an ITT4RT-Rx clients may be offered as an overlay by the ITT4RT MRF to other ITT4RT-Rx clients. The ITT4RT MRF (acting as the ITT4RT-Tx client) would be the source of overlay media in this case.

## *** End change 1 ***

## *** Start change 2 ***

## Y.6.8 Multiple 360-degree videos

An ITT4RT conference may contain multiple 360-degree videos which originate from multiple conference rooms at the conference location, or from remote participants.  When multiple 360-degree videos are present in an ITT4RT conference, an ITT4RT MRF shall negotiate an SDP session with every remote participant.

In the SDP offer from the ITT4RT MRF, 360-degree video is identified by either the a=3gpp_360video or a=3gpp_fisheye media line attributes.  When multiple 360-degree videos are present in the SDP offer, the ITT4RT MRF shall include the a=content attribute under the media lines for 2D or 360-degree video originating from the conference location.  For media streams originating from the main default conference room, the content attribute is set to a=content:main.  For media streams originating from other conference rooms, the content attribute is set to a=content:alt.  2D and 360-degree video from remote participants shall not include the a=content attribute under their corresponding media lines.

When there are multiple 360-degree videos from multiple sources available to the ITT4RT MRF, the ITT4RT MRF may include the ‘itt4rt_group’ attribute (as defined in Y.6.2.6) to define one or more restricting groups, each group containing at least one mid associated with a 360-degree video media line, and at least one mid associated with an overlay.

On receipt of an SDP offer containing multiple 360-degree videos from the ITT4RT MRF, an ITT4RT-Rx client shall select to receive only one 360-degree video media together with possible 2D video media from other sources, rejecting the other 360-degree video media.

Example SDP offers for multiple 360-degree video with and without group restrictions are shown in clause Y.8.

## *** End change 2 ***

## *** End of changes ***