3GPP TSG-SA4 Meeting #115-e	S4-211308

# Online, 18th – 27th August 2021													Revision of S4-211131

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.444 | CR | 0040 | rev | 1 | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test vectors for the EVS codec |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Fraunhofer IIS, Huawei Technologies Co., VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_codec-SA4 |  |  |  |  |  | Date: |  |  | 2021-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CRs to the fixed point and floating point EVS source code make a set of updated test vectors and floating-point conformance thresholds for testing conformance necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update of the test vectors for the fixed-point and floating-point conformance testing. - Update of the floating-point conformance testing thresholds in Annex A 3.4 - Update list of reference implementations due to availability of one of the commercially available compilers - Added clarification on related version of 26.443 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test vectors in TS 26.444 do not match the output from the EVS codec implementations in TS 26.442, TS 26.443 and TS 26.452. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.4., A4, Electronic attachment to 26.444 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.442 CR0043, TS 26.443 CR0040, TS 26.452 CR0004 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: Corrected various formal issues in CR form |  |  |  |  |  |  |  |  |

Start change 1

## A.3.4 Thresholds and Criteria

From the MOS-LQO differences of the test condition, the average, 95%, 99% and Maximum are computed for all bandwidths combined, as well as for each set of bandwidth condition. The number of test condition for each bandwidth and the total are summarized in Table A.6.

Table A.6: Number of test conditions per bandwidth excluding the EVS JBM [11]

| Bandwidth | NB | WB | WBIO | SWB | FB | All |
| --- | --- | --- | --- | --- | --- | --- |
| Number | 118 | 214 | 216 | 170 | 143 | 861 |

An implementation according to 26.443 Version 16.2.0 will be considered passing the MOS-LQO verification if all the average, 95 percentile, 99 percentile and maximum MOS-LQO differences are below the thresholds proposed in Table A.7 for all bandwidths.

Table A.7: Thresholds for MOS_LQO difference excluding the EVS JBM [11]

| All | Average | 95% | 99% | Max |
| --- | --- | --- | --- | --- |
| A-B | 0.001 | 0.04 | 0.07 | 0.11 |
| A-C | 0.001 | 0.02 | 0.04 | 0.09 |
| A-D | 0.002 | 0.04 | 0.07 | 0.12 |
| NB | Average | 95% | 99% | Max |
| A-B | 0.01 | 0.07 | 0.09 | 0.1 |
| A-C | 0.002 | 0.02 | 0.02 | 0.04 |
| A-D | 0.012 | 0.07 | 0.09 | 0.11 |
| WB | Average | 95% | 99% | Max |
| A-B | 0.003 | 0.05 | 0.06 | 0.08 |
| A-C | 0.001 | 0.02 | 0.04 | 0.09 |
| A-D | 0.004 | 0.05 | 0.07 | 0.09 |
| WBIO | Average | 95% | 99% | Max |
| A-B | 0.006 | 0.02 | 0.04 | 0.08 |
| A-C | 0.003 | 0.01 | 0.02 | 0.04 |
| A-D | 0.004 | 0.01 | 0.03 | 0.08 |
| SWB | Average | 95% | 99% | Max |
| A-B | 0.002 | 0.04 | 0.06 | 0.08 |
| A-C | 0.003 | 0.03 | 0.04 | 0.08 |
| A-D | 0.003 | 0.04 | 0.07 | 0.07 |
| FB | Average | 95% | 99% | Max |
| A-B | 0.005 | 0.05 | 0.07 | 0.11 |
| A-C | 0.003 | 0.03 | 0.04 | 0.05 |
| A-D | 0.003 | 0.05 | 0.07 | 0.12 |

NOTE:	The MOS-LQO verification does not include testing of the EVS JBM solution. Conformance for the EVS JBM solution in [11] is FFS.

End change 1

Start change 2

# A.4 Reference Implementations

To get the snr and mld corridor as well as the thresholds values for the MOS-LQO, as set of references implementations were used. Table A.8 list the implementations used for references, including compiler, target platform, compiler setting. These implementations are based on mainstream compilers and platforms and used the latest version of EVS code defined in TS 26.443 [8]. These implementations are not bit-exact between themselves or with the 3GPP reference implementation (Visual Studio 2018, Release mode).

Table A.8: List of Reference Implementations

| Name | Platform | Compiler | Optimization | OS |
| --- | --- | --- | --- | --- |
| aarch64_gnu-gcc-8_-armv8_O2 | ARMv8 | GCC v8 | O2 | Linux |
| aarch64_gnu-gcc-8_armv8_O3 | ARMv8 | GCC v8 | O3 | Linux |
| clang-6_armv8 _O2 | ARMv8 | Clang v6 | O2 with FMA | Linux |
| clang-6_armv8_O3 | ARMv8 | Clang v6 | O3 with FMA | Linux |
| clang-6.0_x86_64_O2 | x86_64 | Clang v6 | O2 | Linux |
| gcc-7_i686_-O0 | i686 | GCC v7 | O0 | Linux |
| gcc-7_i686_-O1 | i686 | GCC v7 | O1 | Linux |
| gcc-7_i686_-O2 | i686 | GCC v7 | O2 | Linux |
| gcc-7_i686_-O3 | i686 | GCC v7 | O3 | Linux |
| icc-2021.3.0_x86_64_avx2 | x86_64 | ICC v2021.3. | O3 with FMA | Linux |

End change  2

Start change 3

The test vectors found in the electronic attachment to TS 26.444 will be exchanged with new test vectors that are relevant for the latest updates of the EVS fixed-point code in TS26.442 and floating-point code in TS26.443.

End change 3